| # UNITED # NATIONS |  | ## E |
| --- | --- | --- |
|  | ### Economic and Social #### Council | Distr. GENERAL TRANS/SC.3/WP.3/2000/6/Add.3 2 January 2003 Original: ENGLISH |

##### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- Working Party on Inland Water Transport

- Working Party on the Standardization of Technical
and Safety Requirements in Inland Navigation

(Twenty-fifth session, 19-21 March 2003,

agenda item 4)

- HARMONIZATION OF THE REQUIREMENTS CONCERNING ANCHORS

FOR INLAND NAVIGATION VESSELS

- Addendum 3

- Transmitted by the Government of Belarus

Table 1:  Passenger vessels

| Displacement | Dimensions |  |  | Mean height of super-structure above waterline | Number, type and weight (calculated according to national requirements) of |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation of the vessel, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | bow anchors | stern anchors |  |  |
| D (t) | L  (m) | B */ (m) | d */ (m) | HM (m) | MB (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 37.54 | 24.0 | 3.96 | 0.70 | 3.75 | 2 x 35, Matrosov | - | 2 x 60.5 cable | Zones 2 and 3, 60 passengers |
| 39.4 | 24.3 | 4.13 | 0.68 | 3.77 | 2 x 35, Matrosov | - | 2 x 60.5 cable | Zone 2, 38 passengers |

Tables 2:  Pushers

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight of stern anchors calculated according to national requirements | Length of chain of stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- |
| P (kW) | CC (t) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 |
| 220 | 1800 | 2 x 150, Matrosov | 2 x 50 | Dnieper, (zones 1 and 2), P96 type |
| 220 | 1800 | 1 x 125, Matrosov | 2 x 50 | Dnieper, (zone 2), Pripyat and Sozh (zone 3), P96 type |

Table 3:  Self-propelled pusher vessels

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight (calculated according to national requirements) of |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- |
|  |  | bow anchors | stern anchors |  |  |
| P (kW) | CC (t) | MS (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 2 x 220 | 2525 | 2 x 500, Hall | 1x800, Hall | 2 x 125/100 | Dnieper (Zone 1), carriage of genertal and bulk cargoes and containers. |

Table 4: Pushed barges

| Dimensions |  |  | Carrying capacity | Number, type and weight of bow anchors calculated according to national requirements | Length of chain of bow anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- |
| L (m) | B (m) | d (m) | CC (t) | MB (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 73.68 | 10.18 | 1.77 | 900 | 2 x 200, Matrosov | 2 x 101/76 | Zones 1, 2 and 3 |
| 73.68 | 10.18 | 1.87 | 900 | 2 x 500, Hall | 2 x 100 | Zones 1 and 2 |
| 49.96 | 10.06 | 1.54 | 540 | 2 x 300, Hall | 2 x 51 | Zones 2 and 3 |
| 50.11 | 10.06 | 1.60 | 560 | 2 x 150, Matrosov | 2 x 51 | Zones 2 and 3 |