| # UNITED # NATIONS |  | ## E |
| --- | --- | --- |
|  | ### Economic and Social #### Council | Distr. GENERAL TRANS/SC.3/WP.3/2000/6/Add.2 2 January 2001 Original: ENGLISH |

##### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- Working Party on Inland Water Transport

- Working Party on the Standardization of Technical
and Safety Requirements in Inland Navigation

(Twenty-first session, 14-16 March 2001,

agenda item 4)

- HARMONIZATION OF THE REQUIREMENTS CONCERNING ANCHORS

FOR INLAND NAVIGATION VESSELS

- Addendum 3

- Transmitted by the Governments of Poland and Ukraine

GE.00

###### POLAND

Table 1:  Passenger vessels

| Displacement | Dimensions |  |  |  |  | Mean height of super-structure above waterline | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation of the vessel, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | bow anchors | stern anchors |  |  |
| D (t) | L  (m) | B */ (m) | d */ (m) | HM (m) | MB (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 10 | 12.6 | 3.00 | 0.50 | 2.00 | 1 x 25, special** | - | 30 | Zone 3, 30 passengers |
| 12 | 13.5 | 2.94 | 0.44 | 3.03 | 1 x 25, Hall | - | 25 | Zone 3, 40 passengers |
| 18 | 31.5 | 5.50 | 0.95 | 5.00 | 2 x 100, Hall | - | 50 | Zone 2, 140 passengers |
| 34 | 18.2 | 4.10 | 0.50 | 2.80 | 1 x 75, Hall | - | 40 | Zone 3, 101 passengers |
| 44 | 24.0 | 3.13 | 1.00 | 4.20 | 1 x 75, Hall | - | 20 | Zone 3, 65 passengers |
| 49 | 29.9 | 3.05 | 1.00 | 3.20 | 2 x 100, Hall | - | 20 | Zone 3, 65 passengers |
| 62 | 29.7 | 5.42 | 0.75 | 3.25 | 1 x 100, Hall | - | 25 | Zone 3, 203 passengers |
| 155 | 35.0 | 6.00 | 1.55 | 5.00 | 2 x 250,Hall | - | 2 x 100 | Zone 3, 250 passengers |
| 168 | 35.0 | 6.00 | 1.64 | 5.00 | 2x 250, Hall | - | 2 x 50 | Zone 2, 250 passengers |

Tables 2:  Pushers

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight of stern anchors calculated according to national requirements | Length of chain of stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- |
| P (kW) | CC (t) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 |
| 121 | 2 x 150 | 2 x 100, Hall | 2 x 30 | Zone 3, Daniel type |
| 2 x 107 | 2 x 200 | 1 x 100, Hall | 30 | Zone 3, LOS PS-200 |
| 2 x 121 | 2 x 200 | 2 x 300, four-armed | 2 x 50 | Zone 3, TUR |
| 2 x 154 | 2 x 500 | 2 x 400, special | 2 x 50 | Zone 2, Karibu |
| 2 x 155 | 2 x 500 | 2 x 400, special | 2 x 30 | Zone 2, Bizon-IIIP |
| 2 x 155 | 2 x 500 | 2 x 400, special | 2 x 30 | Zone 3, Bizon-IIIT |

Table 4: Pushed barges

| Dimensions |  |  |  |  | Carrying capacity | Number, type and weight of bow anchors calculated according to national requirements | Length of chain of bow anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L (m) | B (m) | d (m) | CC (t) | MB (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 34.00 | 6.56 | 0.90 | 131 | 1 x 150 four armed | 30 | Zone 3, BP-130 type |
| 34.00 | 6.56 | 0.93 | 145 | 2 x 150 four armed | 2 x 25 | Zone 3, B-125 type |
| 30.44 | 4.55 | 1.50 | 151 | 1 x 150 special | 30 | Zone 3, BP-150 type |
| 31.05 | 10.00 | 1.00 | 224 | 1 x 300 four-armed | 67 | Zone 3, BP-200 type |
| 34.28 | 8.60 | 1.55 | 352 | 1 x 350,four-armed | 2 x 22 | Zone 3, BPP-400 type |
| 32.50 | 8.18 | 1.97 | 393 | 1 x 350, Hall | 50 | Zone 2, 190/1 type |
| 44.88 | 8.90 | 1.60 | 461 | 1 x 450, special | 60 | Zone 3, BP-500/II-SAU |
| 44.95 | 8.92 | 1.60 | 471 | 1 x 500, special | 55 | Zone 2, OBP-500 type |
| 44.91 | 8.90 | 1.61 | 478 | 1 x 500, special | 55 | Zone 2, OBP-500/III type |
| 44.87 | 8.91 | 1.60 | 495 | 1 x 450, Hall | 55 | Zone 3, BPC-500 type |
| 46.96 | 8.60 | 1.55 | 511 | 1 x 400, Hall | 47 | Zone 3, BPP-500/K type |
| 51.20 | 8.18 | 2.00 | 641 | 2 x 450, Hall | 2 x 75 | Zone 3, WW-550 type |
| 58.95 | 8.96 | 2.00 | 762 | 2 x 400, Hall | 2 x 60 | Zone 2, BP-800 |

Table 4: Pushed barges (continued)

| Dimensions |  |  |  |  | Carrying capacity | Number, type and weight of bow anchors calculated according to national requirements | Length of chain of bow anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L (m) | B (m) | d (m) | CC (t) | MB (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 66.34 | 9.00 | 1.95 | 878 | 1 x 450, Hall 1 x 350, Hall | 2 x 50 | Zone 3, VC-900 type |
| 60.77 | 9.08 | 2.50 | 971 | 1 x 400, Special | 45 | Zone 2, BP-1000P type |
| 76.50 | 11.37 | 3.77 | 2486 | 2 x 500, Hall | 2 x 82.5 | Zone 1, BP-3000 type |
| 76.59 | 11.38 | 3.79 | 2514 | 2 x 500, Hall | 2 x 80 | Zone 1, PMP-2950 type |
| 76.42 | 11.39 | 3.95 | 2721 | 2 x 1150, special | 2 x 90 | Zone 2, BP-2700 type |

###### UKRAINE

Table 1:  Passenger vessels

| Displacement | Dimensions |  |  |  |  | Mean height of super-structure above waterline | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation of the vessel, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | bow anchors | stern anchors |  |  |
| D (t) | L (m) | B (m) | d (m) | HM (m) | MB (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 2000 | 117.0 | 17.0 | 1.69 | 8.37 | 2 x 1750, Hall | - | 2 x 75 | Danube (Zone 3) m/v “Ukraina” |
| 1514 | 105.9 | 16.1 | 1.65 | 8.25 | 2 x 1500, Hall | - | 2 x 75 | Danube (Zone 3) m/v “Volga |
| 891 | 86.12 | 14.4 | 1.82 | 10.2 | 2 x 1250, Hall | - | Starboard 100 Port 75 | Danube (Zone 3) m/v “Dunay” |
| 3570 | 125.0 | 16.7 | 2.76 | 10.8 | 2 x 2000, Hall | - | 2 x 150 | Dnieper (Zones 1 and 2), Danube, north-west of the Black Sea. Vessels of 301 type |

Note:  	In column 6 a calculated mass of anchors is indicated.

In column 8 a real length of chain is shown.

Tables 2:  Pushers

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight of stern anchors calculated according to national requirements | Length of chain of stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- |
| P (kW) | CC (t) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 |
| 1472 | 8000 | 2 x 1450, Hall | 100 | Danube (Zone 3), carriage of all sorts of cargo except bulk liquids, m/v “Riga” |
| 1766 | 4400 | 2 x 1560, Hall | 2 x 125 | Dnieper (Zones 1 and 2), north-west of the Black Sea, Danube (Zone 3), vessels of H-3290 type |
| 1472 | 4400 | 2 x 1450, Hall | 2 x 125 | Dnieper (Zones 1 and 2), north-west of the Black Sea, Danube (Zone 3), vessels of 4281 type. |

Note:  	In column 6 a calculated mass of anchors is indicated.

In column 8 a real length of chain is shown.

Table 3:  Self-propelled pusher vessels

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | bow anchors | stern anchors |  |  |
| P (kW) | CC (t) | MS (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 2 x 722 | 8500 | - | 2x1000, Hall | -/2 x 75 | Danube (Zone 3), carriage of bulk, high-volume cargo, containers, m/v “Captain Antipov” |

Note:  	In column 6 a calculated mass of anchors is indicated.

In column 8 a real length of chain is shown.

Table 4: Pushed barges

| Dimensions |  |  |  |  | Carrying capacity | Number, type and weight of bow anchors calculated according to national requirements | Length of chain of bow anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L (m) | B (m) | d (m) | CC (t) | MB (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 76.53 | 15.04 | 2.50 | 2300 | 2 x 1000, Hall | Port – 75 starboard - 100 | Danube (Zone 3), high-volume, heavy, bulk cargo, barge-section C-1 |
| 76.50 | 11.00 | 2.50 | 15008 | 2 x 900, Hall | 2 x 100 | Danube (Zone 3), high-volume, heavy, bulk cargo, barge-section C-1 |
| 38.25 | 11.00 | 3.30 | 1078 | 2 x 700, Hall | 2 x 100 | Danube (Zone 3), north-west of the Black Sea, high-volume, heavy cargo, section-platform PDM-12 |
| 76.50 | 10.00 | 2.70 | 1588 | 2 x 1000, Hall | 2 x 165 | Danube (Zone 3), high-volume, heavy and bulk cargo, barge-section S-401 |
| 90.30 | 16.50 | 2.50 | 2000 | 2 x 1500, Hall | Port – 190 starboard - 220 | Danube (Zone 3), north-west of the Black Sea, section-platform TMI |
| 76.50 | 11.00 | 2.70 | 1778 | 2 x 900, Hall | 2 x 100 | Danube (Zone 3), high-volume, heavy and bulk cargoes, barge-section S-401 |
| 84.80 | 15.20 | 2.42 | 2200 | 2 x 1000, Hall | 2 x 100 | Dnieper (Zones 1 and 2), north-west of the Black Sea,, Danube (Zone 3), open storage cargoes, barge D021type |
| 84.80 | 15.20 | 2.42 | 2100 | 2 x 800, Hall | 2 x 125 | Dnieper (Zones 1 and 2), north-west of the Black Sea,, Danube (Zone 3), open storage cargoes, barge 1021type |

Note:  	In column 6 a calculated mass of anchors is indicated.

In column 8 a real length of chain is shown.

*	L - Length, B - Beam and d - Draught of vessels

**	Special anchor = anchor with increased holding power and reduction of required mass to 75%.

