| UNITED NATIONS | E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/SC.3/WP.3/2000/6 21 December 1999 Original: ENGLISH |
| --- | --- | --- |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Group on Inland Water Transport

Working Party on the Standardization

of Technical and Safety Requirements

in Inland Navigation

(nineteenth session, 14-16 March 2000,

agenda item 4)

HARMONIZATION OF THE REQUIREMENTS CONCERNING ANCHORS FOR

INLAND NAVIGATION VESSELS

Transmitted by the Governments of Lithuania, Romania and

the Russian Federation

Note:	At its seventeenth session, the Working Party agreed that it would be useful to collect information on actual anchor requirements for vessels other than self-propelled cargo vessels (covered by resolution

No. 36, TRANS/SC.3/104/Add.3) and on the basis of this information and using also relevant CCNR and draft EC provisions to try to develop, with the help of a volunteer delegation, minimum pan-European anchor requirements for the following types of vessels: (i) passenger vessels; (ii) pushers; (iii) self-propelled pusher vessels; and (iv) pushed barges.  Governments were invited to complete the tables set out in annex 2 to TRANS/SC.3/WP.3/35, reflecting the existing national requirements in their countries as far as the equipment of the above four types of inland navigation vessels was concerned (TRANS/SC.3/WP.3/35, paras. 13 and 14).

Reproduced below is the information received from Governments.

GE.99-

LITHUANIA

Table 1:  Passenger vessels

| Displacement | Dimensions |  |  |  |  | Mean height of super-structure above waterline | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation of the vessel, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | bow anchors | stern anchors |  |  |
| D (t) | L */ (m) | B */ (m) | d */ (m) | HM (m) | MB (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 200 200 | 40 40 | 6 6 | 1.2 1.5 | 6.0 6.0 | 2 x100, Matrosov 2 x100, Matrosov | - - | 2x75 2x75 | River Nemunas, Kaunas-Jurbarkas River Nemunas, Klaipeda-Kaunas |

Tables 2:  Pushers

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight of stern anchors calculated according to national requirements | Length of chain of stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- |
| P (kW) | CC (t) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 |
| 600 | 1000 | 2x150, Matrosov | 2x100 | River Nemunas, Kurshskiy Zaliv (construction material, coal, timber, containers) |

*/ L - Length, B - Beam and d - Draught of vessels

Table 3:  Self-propelled pusher vessels

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | bow anchors | stern anchors |  |  |
| P (kW) | CC (t) | MS (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 220 | 300+600=900 | 2x125, Matrosov | 1x1,250 (four arms anchor) | 50+75/25 | River Nemunas, Kurshskiy Zaliv (construction material, coal, timber) |

Table 4:   Pushed barges

| Dimensions |  |  |  |  | Carrying capacity | Number, type and weight of bow anchors calculated according to national requirements | Length of chain of bow anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L */ (m) | B */ (m) | d */ (m) | CC (t) | MB (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 72 | 12 | 1.1 | 600 | 2x200, Matrosov | 75/100 | River Nemunas, Jurbarkas-Kaunas (construction material, coal, timber, containers) |
| 72 | 12 | 1.5 | 1050 | 2x200, Matrosov | 75/100 | Klaipeda-Kaunas |

ROMANIA

Table 1:  Passenger vessels

| Displacement | Dimensions |  |  |  |  | Mean height of superstructure above waterline | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation of the vessel, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | bow anchors | stern anchors |  |  |
| D (t) | L */ (m) | B */ (m) | d */ (m) | HM (m) | MB (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 14 60 59 61 85 226 472 | 14.2 24.0 25.1 24.0 27.7 45.2 61.4 | 4.1 4.8 5.1 4.5 6.3 7.1 11.3 | 0.4 1.2 0.8 1.5 1.0 1.5 1.8 | 3.1 3.0 3.1 3.4 3.6 4.2 6.6 | 2X75, Hall 2X150, Speck 2X150, Hall 2X150, Hall 2X175, Hall 2X200, Hall 2X350, Hall | - - - - - - - | 2x30/- 2x30/- 2x50/- 2x50/- 2x50/- 1x50+1x75/- 1x50+1x75/- | Danube, 30 passengers Danube, 25 passengers Danube, 60 passengers Danube, 120 passengers Danube, 90 passengers Danube, 150 passengers Danube, 300 passengers |

Tables 2:  Pushers

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight of stern anchors calculated according to national requirements | Length of chain of stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- |
| P (kW) | CC (t) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 |
| 2x220 2x295 2x310 2x600 2x655 2x880 2x925 2x1000 2x1325 2x1765 | 2000 3000 3000 6000 6000 9000 9000 9000 12000 18000 | 2x200, Hall 2x400, Hall 2x500, Speck 2x900, Hall 2x650, Hall 2x930, Hall 2x2100, Speck 2x1750, Speck 2x1500, Speck 2x1320, Hall | 2x75 2x100 2x75 2x75 2x75 2x75 2x100 2x75 2x82.5 2x75 | Danube, max. 6 km/h A A A A A A A A |

Table 3:  Self-propelled pusher vessels

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | bow anchors | stern anchors |  |  |
| P (kW) | CC (t) | MS (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 2x650 | 4000 | 2x700, Speck | 2x1000, Speck | 2x100/2x75 | Danube, max. 6 km/h |

Table 4:   Pushed barges

| Dimensions |  |  |  |  | Carrying capacity | Number, type and weight of bow anchors calculated according to national requirements | Length of chain of bow anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L */ (m) | B */ (m) | d */ (m) | CC (t) | MB (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 61.4 71.0 70.2 71.0 70.3 76.5 76.2 76.5 86.3 89.0 | 11.0 11.0 11.0 11.0 11.0 11.0 11.0 11.0 11.0 11.0 | 2.0 1.8 2.5 2.4 2.5 2.7 3.0 3.1 2.9 3.8 | 1000 1000 1300 1500 1500 1700 2000 2000 2000 3000 | 1x1920, Hall 2x600, Hall 1x2000, Hall 1x1000, Hall 1x1980, Hall 1x1250, Hall 1x1920, Hall 1x1920, Hall 1x1740, Hall 1x2100, Hall | 1x75 2x75 1x120 1x110 1x120 1x100 1x100 1x100 1x100 1x100 | Danube, max. 6 km/h A A A A A A A A A |

RUSSIAN FEDERATION

Table 1:  Passenger vessels

| Displacement | Dimensions |  |  |  |  | Mean height of super-structure above waterline | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation of the vessel, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | bow anchors | stern anchors |  |  |
| D (t) | L */ (m) | B */ (m) | d */ (m) | HM (m) | MB (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 3850 | 129 | 16 | 2.85 | 13.8 | 2x1575 of enhanced holding power | 1x855 of enhanced holding power | 175 and 150/125 | Navigational zone 1 (AM@ basin) |
| 1390 | 90.2 | 13.5 | 1.66 | 11 | 2x1000, Hall | 1x500, Hall | 125 and 100/75 | Navigational zone 2 (AO@ basin) |
| 35.0 | 24.3 | 3.96 | 0.68 | 5.2 | 2x35, Matrosov, of enhanced holding power | - | 60, steal anchor line | Navigational zone 3 ( AP@ basin) |

Tables 2:  Pushers

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight of stern anchors calculated according to national requirements | Length of chain of stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- |
| P (kW) | CC (t) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 |
| 810 | 9000 | 1x1750, Hall | 300, steal anchor line | Navigational zone 2 (@O@ basin) |
| 1765 | 15000 | 2x1250, Hall | 125 | Navigational zone 2 (@O@ basin) |
| 220 | 2000 | 1x125, Matrosov | 120, steal anchor line | Navigational zone 3 (@P@ basin) |

Table 3:  Self-propelled pusher vessels

| Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight (calculated according to national requirements) of |  |  | Length of chain of bow/stern anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | bow anchors | stern anchors |  |  |
| P (kW) | CC (t) | MS (kg) | MS (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 2x331=662 | 2000 | 1x1500 and 1x1250, Hall | 1x1000, Hall | 175 and 175/75 | Navigational zone 1 (AM@ basin) |
| 2x880=1760 | 11400 | 2x1750, Hall | 2x1250, Hall | 155 and 155/75 | Navigational zone 2 (@O@ basin) |
| 165.5 | 300 | 1x150, Matrosov | 1x100, Matrosov | 75/75, steal anchor line | Navigational zone 3 (AP@ basin) |

Table 4:   Pushed barges

| Dimensions |  |  |  |  | Carrying capacity | Number, type and weight of bow anchors calculated according to national requirements | Length of chain of bow anchors | Additional observations: Main region (zone) of operation, vessels for carrying light voluminous cargo, etc. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L */ (m) | B */ (m) | d */ (m) | CC (t) | MB (kg) | l (m) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 85.7 | 16.5 | 2.55 | 2500 | 2x1000, Hall | 150 and 150 | Navigational zone 1 (AM@ basin) |
| 113 | 16.5 | 3.48 | 5000 | 2x1250, Hall | 100 and 100 | Navigational zone 2 (AO@ basin) |
| 91.0 | 15.5 | 2.6 | 2000 | 2x800, Hall | 102 and 77 | Navigational zone 3 (AP@ basin) |