###### UNITED

NATIONS

E

Economic and Social

Council

- Distr.

- GENERAL

- TRANS/WP.15/2001/20

- 10 April 2001

- ENGLISH ONLY

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport

of Dangerous Goods

(Seventieth session,

Geneva, 7-11 May 2001

agenda item 2e)

EUROPEAN AGREEMENT CONCERNING THE INTERNATIONAL CARRIAGE OF DANGEROUS GOODS BY ROAD (ADR)

RESTRUCTURED ADR

CORRECTIONS

Note by the Secretariat

The replacement of Annexes A and B of ADR by the restructured annexes adopted by the Working Party at its sixty-ninth session has been proposed by the Government of Portugal to all Contracting Parties in accordance with the amendment procedure of Article 14 of ADR (Depositary Notification C.N. 1078.2000.TREATIES-3 of 1 January 2001)

By Depositary Notification C.N. 282.2001.TREATIES-1 of 3 April 2001, the Secretary-General informed all Contracting parties that the amendments were deemed accepted and will enter into force on 1 July 2001.

The UN/ECE Secretariat has prepared a consolidated version of the ADR accordingly, which is being published under symbol ECE/TRANS/140, Vol. I and II.

When preparing the publication, the Secretariat noticed that there were some obvious mistakes in the text annexed to notification C.N. 1078.2000.TREATIES-3, and these mistakes have been corrected in the publication. The Secretariat reproduces hereafter the list of corrections made.

1.6.4.12	Replace "Tank-containers and MEGCs constructed before 1 July 2003" with "Tank-containers and MEGCs constructed before 1 January 2003"

1.8.3.2. (a)	Replace "1.1.3" with "1.1.3.6"

2.1.3.4	Under class 6.1, for UN No. 1051, replace "containing not more than 3% water" with "containing less than 3% water"

2.2.1.1.7	Replace "CHARGES, EXPLOSIVE without detonator: UN Nos. 0442, 0443, 0444, 0445" with "CHARGES, EXPLOSIVE, COMMERCIAL without detonator: UN Nos. 0442, 0443, 0444, 0445"

Replace "CHARGES, SHAPED, COMMERCIAL, without detonator" with "CHARGES, SHAPED, without detonator"

Replace "FUSE, INSTANTANEOUS, NON-DETONATING: UN No. 0101" with "FUSE, NON-DETONATING: UN No. 0101"

Same amendment in the Note after the definition of "IGNITERS: UN Nos. 0121, 0314, 0315, 0325, 0454"

2.2.3.3	In the list, under classification code F1, for UN Nos. 1987 and 1989, delete "FLAMMABLE,"

2.2.41.1.18	In the list of UN Nos. delete "3270,"

2.2.41.4	Insert the following note before the table:

"NOTE: For the packing methods, see 4.1.4.1, packing instruction P520, and 4.1.7.1."

2.2.7.2	Under the definition of "small container" replace "an overall outer dimension" with "any overall outer dimension"

2.2.7.7.2.1	For Ar-39, read in the A1 column "4 x 101" and in the A2 column "2 x 101"

2.2.8.3	Under note f to the table, replace "2.2.61.1.4" with "2.2.61.1.4 to 2.2.61.1.9"

2.2.9.1.8	Amend the end to read as follows: "... which meet the descriptions of special provisions 235 or 296 of Chapter 3.3."

Table A of Chapter 3.2

In column (8), replace whenever it appears "See 2.7" with "See 2.2.7"

In column (13), delete, whenever they appear, "TE15", "TE16" and "TE17"

In Column (16), delete, whenever it appears, "V4"

In Column (16), insert for all Class 4.3 goods, "V1"

In Column (17), replace, whenever it appears, "VW08" with "VV8"

In Column (19), replace, whenever it appears, "S17 S20" with "S17"

| UN No. | Column No. | Amendments |
| --- | --- | --- |
| 0340 and 0341 | 16 | Add "V3" |
| 1078 | 2 | Replace "mixture P2" with "mixture F3" |
| 1093 | 2 | Replace "STABILISED" with "STABILIZED" |
| 1139 (for the 8 entries) | 2 | Add, after "COATING SOLUTION", "(includes surface treatments or coatings used for industrial or other purposes such as vehicle under coating, drum or barrel lining)" |
| 1155 | 2 | Amend to read "DIETHYL ETHER (ETHYL ETHER)" |
| 1170, packing group III | 2 | Delete "ETHANOL (ETHYL ALCOHOL) or" |
| 1263, packing group I | 6 | Add "640" |
| 1324 | 2 | Add at the end ", except scrap" |
| 1389, 1391, 1392, 1407, 1415, 1420, 1421, 1422, 1423, 1428, 2257 | 13 | Insert "TT3" |
| 1657 | 2 | Delete ", solid" |
| 1708, liquid entry | 11 | Delete "TU15" |
| 1708, solid entry | 13 | Delete "TT1" |
| 1790 (three entries) | 2 | Delete ",solution" |
| 1965 | 2 | Insert "A0," after "A02," |
| 1989, packing group II, first entry | 11 | Insert "TP1 TP8 TP28" |
| 2059 (5 entries) | 2 | Add, at the end, "with not more than 12.6% nitrogen, by dry mass, and not more than 55% nitrocellulose" |
| 2067 to 2070 | 6 | Delete "193" |
| 2211 and 3314 | 5 | Insert "none" |
| 2426 | 2 | Read as follows: "AMMONIUM NITRATE, LIQUID, hot concentrated solution, in a concentration of more than 80% but not more than 93%" |
| 2660 | 2 | Amend to read "NITROTOLUIDINES (MONO)" |
| 2672 | 2 | Insert, after "SOLUTION," "relative density between 0.880 and 0.957 at 15 °C" |
| 2758, 2760, 2762, 2764, 2772, 2776, 2778, 2780, 2782, 2784, 2787, 3021, 3024, 3346, 3350 | 2 | Add ", flash-point less than 23 °C" |
| 2790,entry of packing group III | 2 | Amend to read "ACETIC ACID SOLUTION, more than 10% and less than 50% acid, by mass" |
| 2862 | 6 | Add "600" |
| 2903, 2991, 2993, 2995, 2997, 3005, 3009, 3011, 3013, 3015, 3017, 3019, 3025, 3347, 3351 | 2 | Add ", flash-point not less than 23 °C" |
| 2929 | 6 | Delete "615" |
| 3014, packing group II | 11 | Insert "TP2 TP13 TP27" |
| 3065, entry in packing group II | 6 | Delete "144 145 247" |
| 3175 | 2 | Read as follows "SOLIDS or mixture of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61°C" |
| 3176, packing group II | 6 | Delete "216" |
| 3234 | 19 | Delete "S18" |
| 3240 and 3319 | 12 | Delete "ST(+)" |
| 3269 | Delete the three rows in which a tank code is shown in Column (12) |  |
| 3269, entry of packing group III | 6 | Delete "640" |
| 3270 | 2 | Add, at the end ", with not more than 12.6% nitrogen, by dry mass" |
| 3270 | 6 | Add "286" |
| 3284, packing group I | 13 | Delete "TE2" |
| 3285, packing group I | 13 | Delete "TE3" |
| 3337 | 2 | Add at the end "(Pentafluoroethane, 1,1,1-trifluoroethane, and 1,1,1,2-tetrafluoroethane zeotropic mixture with approximately 44% pentafluoroethane and 52% 1,1,1-trifluoroethane)" |

Add a new entry to read as follows:

| (1) | (2) | (3a) | (3b) | (4) | (5) | (6) | (7) | (8) | (9a) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1268 | PETROLEUM DISTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S.(vapour pressure at 50 °C more than 110 kPa but not more than 175 kPa) | 3 | F1 | I | 3 | 274 640 | LQ3 | P001 |  |

| (9b) | (10) | (11) | (12) | (13) | (14) | (15) | (16) | (17) | (18) | (19) | (20) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MP7 MP17 | T11 | TP1 TP8 TP9 | L1.5BN |  | FL | 1 |  |  |  | S2 S20 | 33 |

Replace the row of UN No. 1989, packing group I with the following two:

| (1) | (2) | (3a) | (3b) | (4) | (5) | (6) | (7) | (8) | (9a) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1989 | ALDEHYDES, N.O.S. (vapour pressure at 50 °C more than 175 kPa) | 3 | F1 | I | 3 | 274 640 | LQ3 | P001 |  |

| (9b) | (10) | (11) | (12) | (13) | (14) | (15) | (16) | (17) | (18) | (19) | (20) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MP7 MP17 | T11 | TP1 TP9 TP27 | L4BN |  | FL | 1 |  |  |  | S2 S20 | 33 |

| (1) | (2) | (3a) | (3b) | (4) | (5) | (6) | (7) | (8) | (9a) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1989 | ALDEHYDES, N.O.S. (vapour pressure at 50 °C more than 110 kPa but not more than 175 kPa) | 3 | F1 | I | 3 | 274 640 | LQ3 | P001 |  |

| (9b) | (10) | (11) | (12) | (13) | (14) | (15) | (16) | (17) | (18) | (19) | (20) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MP7 MP17 | T11 | TP1 TP9 TP27 | L1.5BN |  | FL | 1 |  |  |  | S2 S20 | 33 |

3.3.1	SP 193 Delete

SP580	Add "for carriage in bulk" after "specially equipped vehicles" and "specially equipped containers"

SP582	Under "Mixture F1" replace "dichlorodifluoromethane" with "dichlorofluoromethane"

SP583	Replace "Mixture B1 have a vapour pressure at 70 °C not exceeding 2.3 MPa (23 bar) and..." with " Mixture B1, have a vapour pressure at 70 °C not exceeding 2.6 MPa (26 bar) and..."

SP623 	In the second sentence, delete "by road"

Add a new special provision to read as follows:

"644	This substance is admitted for carriage provided that:

-	The pH is between 5 and 7 measured in an aqueous solution of 10% of the substance carried;

-	The solution does not contain more than 0.2% combustible material or chlorine compounds in quantities such that the chlorine level exceeds 0.02%."

3.4.1		Replace "4.1.1.4 " with "4.1.1.4 to 4.1.1.8"

4.1.1.4		Delete "under a particular Class"

P101		Replace "(see 5.4.1.2.1 c))" with "(see 5.4.1.2.1 (e))"

P200	B. (7) Amend the end to read as follows: "... outlined in the previous paragraphs of this section B;"

C. (12) z: In the last sentence delete "(see also 5.4.1.1.12)"

P200	Replace the existing row for UN NO. 1020 with the following two:

| 1020 | CHLOROPENTA-FLUOROETHANE (REFRIGERANT GAS R115) | 2A | (1),(2),(3),(5) |  | 2.5 | 10 | 1.08 | n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1021 | 1-CHLORO-1,2,2,2-TETRAFLUORO-ETHANE (REFRIGERANT GAS R124) | 2A | (1),(2),(3),(5) |  | 1.2 | 10 | 1.2 | n |

For UN No. 1975, add in the last column, Special requirements, "l"

Replace the existing row for UN No. 2199 with the following one:

| 2199 | PHOSPHINE c | 2TF | (1),(5) (1),(5) |  | 22.5 25.0 | 5 5 | 0.30 0.51 | e, g, l, n e, g, l, n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Replace the existing row for UN No. 2599 with the following one:

| 2599 | CHLOROTRIFLUORO-METHANE AND TRIFLUOROMETHANE, AZEOTROPIC MIXTURE with approximately 60% chlorotrifluoromethane (REFRIGERANT GAS R503) | 2A | (1),(2),(3),(5) (1),(2),(3),(5) (1),(2),(3),(5) |  | 3.1 4.2 10.0 | 10 10 10 | 0.11 0.20 0.66 | n n n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

P205(1)	Replace "The special packing provisions of 4.1.6.1" with "The special packing provisions of 4.1.6"

P406	Under additional requirement 3, replace "maximum permitted quantity per packaging" with "maximum permitted quantity per package"

P410	Under note a delete "inner"

P504	Amend the part of the table for jerricans to read as follow:

| Jerricans |  |
| --- | --- |
| steel, non-removable head (3A1) steel, removable head (3A2) | 60 l 60 l |
| aluminium, non-removable head (3B1) aluminium, removable head (3B2) | 60 l 60 l |
| plastics, non-removable head (3H1) plastics, removable head (3H2) | 60 l 60 l |

P801	Amend the first sentence to read "This instruction applies to new and used batteries..."

P904 (2) Amend the beginnning to read as follows: "Packagings, which need not conform to the packaging test requirements ..."

4.1.4.4	PR4 	Amend the first sentence to read "This substance shall be packed in steel..."

PR6	Delete the reference to note b after "a check of its tare."

4.1.9.1.5	Amend the end to read as follows: "...requirements of Chapters 4.1 , 4.2 or 4.3 for that subsidiary risk."

4.1.9.2.3 (a) and (b)	Replace "conveyance" with "vehicle"

4.1.10.4	In the second sentence replace "P621" with "P650"

4.3.2.4	Amend the end of the Note under the title to read "... special provisions TU1, TU2, TU4, TU16 and TU35 of 4.3.5 may apply."

4.3.4.1.1	In the table:

- 	in the second column, under part 3 , replace "6.8.2.2.3" with "6.8.2.2.2";

- 	in the third column, under part 4, amend the explanations for "V", "F" and "N" as follows:

"V =	tank with a venting system, according to 6.8.2.2.6, but no flame trap; or nonexplosionpressure proof tank;

F =	tank with a venting system, according to 6.8.2.2.6, fitted with a flame trap; or explosionpressure proof tank;

N =	tank with a safety valve according to 6.8.2.2.7 or 6.8.2.2.8 and not hermetically closed; these tanks may be fitted with vacuum valves;"

4.3.4.1.3 d)	Replace "UN No. 2015 hydrogen peroxide with more than 70% hydrogen peroxide" with "UN No. 2015 hydrogen peroxide, aqueous solution, stabilized with more than 70% hydrogen peroxide" and "UN No. 2015 hydrogen peroxide with 60-70% hydrogen peroxide" with "UN No. 2015 hydrogen peroxide, aqueous solution, stabilized with 60-70% hydrogen peroxide"

4.3.5	TU35	Amend the beginning to read as follows: "Empty fixed tanks (tank-vehicles), empty demountable tanks ..."

4.4 and 4.5	Amend the note under the title to read "... for fixed tanks (tank-vehicles), demountable tanks, tank-containers and tank swap bodies, ..."

4.5.2.1	Add "except those of 4.3.2.2.4 and 4.3.2.3.3" after "Chapter 4.3"

5.4.1.2.1 (c)	Add "of 4.1.4.1" after "packing instruction P101"

6.1.3.7	Replace " NL/VL/123" with " NL/VL123" and " NL/VL/124" with " NL/VL124"

6.1.3.8	Replace " NL/RB/85/RL" with " NL/RB/85 RL"

6.1.3.9	Replace "1A2T/Y/300/94" with "1A2T/Y300/S/94"

6.1.6.2	In the table, under Class 8, for the corrosive acid inorganic liquids, without subsidiary risk, replace the classification code "C2" with "C1"

6.2.1.4.3	Replace " the provisions of Class 2" with "the provisions for Class 2"

6.2.1.7.1 (c)	Delete "or batch"

6.2.2	In the table, change the order of the titles of the documents in the sixth and seventh row so that directive 84/526/EEC refers to seamless, unalloyed aluminium and aluminium alloy gas cylinders and directive 84/527/EEC refers to welded unalloyed steel gas cylinders

6.2.4.3	Replace "of this paragraph" with "of this section"

6.5.2.2.4	Insert "see" before "6.5.2.1.1 (f)"

6.6.5.4.2	Insert "made" before "available"

6.7.2.1	In the definition of "Design pressure", under (b) (iii) add "or" at the end

6.8.2.1.3	In the left hand column, replace "6.8.2.1.17 and 6.8.2.1.21" with "6.8.2.1.17 to 6.8.2.1.21"

6.8.2.3.1	Amend the first sentence to read "... type of tank-vehicle, demountable tank, tank-container, tank swap body, battery-vehicle or MEGC..."

6.8.2.4.1	Move the last paragraph before the last but one in order to group the requirements concerning the hydraulic pressure test.

8.2.1.2	Replace the reference to "8.2.2.3.3" with "8.2.2.3.2"

___________________