Distr.

- GENERAL

- TRANS/WP.29/GRE/2001/27

18 July 2001

- Original:  ENGLISH

- Economic COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonisation of Vehicle Regulations (WP.29)

- Working Party on Lighting and Light-Signalling (GRE)

(Forty-seventh session, 1-5 October 2001,

agenda item 2.4.)

PROPOSAL FOR DRAFT AMENDMENTS (SUPPLEMENT 4 TO THE 01 SERIES)

TO REGULATION No. 74

(Installation of lighting and light-signalling devices on mopeds)

- Transmitted by the Expert

from the International Motorcycle Manufacturers Association (IMMA)

Note:	The text reproduced below was prepared by the expert from IMMA with the aim to allow the use of amber colour front position lamps for motorcycles.  Although an equivalent proposal had not been accepted within draft Supplement 3 to the 01 series of amendments to Regulation No. 74 (TRANS/WP.29/786), IMMA decided to table it again, because its acceptance would improve global harmonization and avoid practical problems.  The text is based on a section of the document distributed without a symbol (informal document No. 13) during the forty-sixth session of GRE (TRANS/WP.29/GRE/46, para. 76).

__________

Note:	This document is distributed to the Experts on lighting and light-signalling only.

- GE.01-22575

## A.	PROPOSAL

Paragraph 5.13., amend to read: 1/

"5.13. 	Colours of the lights

...........

front position lamp: 	white or amber

............"

Paragraph 6.3.1., amend to read: 1/

"6.3.1. 	Number

One or two. 	if coloured white

or

Two (one per side) 	if coloured amber"

Paragraph 6.3.7., amend to read: 1/

"6.3.7. 	Other requirements

When the front position lamp is reciprocally incorporated in the front direction indicator lamp, either

a) the ratio between the luminous intensities actually measured of the two lamps when turned on simultaneously and the intensity in the front position lamp when turned on alone shall be no less than 5 to 1; 	or

b) the electrical connection shall be such that the position lamp, on the same side as the direction indicator lamp, is switched off only when the direction indicator lamp is flashing.”

Paragraph 6.8.7., amend to read: 1/

"6.8.7. 	May not be "reciprocally incorporated" with any other lamp, except amber front position lamp."

*     *     *

B.	JUSTIFICATION

1/	Front position lamps incorporating direction indicator lamps

__________