Distr.

- GENERAL

- TRANS/WP.29/GRE/2001/31/Add.1

24 January 2002

- Original:  ENGLISH

- Economic COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonisation of Vehicle Regulations (WP.29)

- Working Party on Lighting and Light-Signalling (GRE)

(Forty-eighth session, 9-12 April 2002,

agenda item 1.3.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48

(Installation of lighting and light-signalling devices)

Addendum 1

- Transmitted by the Expert from the Working Party "Brussels 1952" (GTB)

Note:	The text reproduced below was prepared by the expert from GTB, following the consideration of the original proposal by GRE, during its forty-seventh session (TRANS/WP.29/GRE/47, paras. 20-23).  GTB examined the questions and suggestions made, and proposes in this document the improvements regarding definitions only.

__________

Note:	This document is distributed to the Experts on lighting and light-signalling only.

- GE.02-20436

## A.	PROPOSAL

The original proposal (TRANS/WP.29/GRE/2001/31) was:

2.7.	"Lamp" means a device designed to illuminate the road or to emit a light signal to other road users. Lighting units of adaptive front lighting systems, rear registration plate lamps and retro-reflectors are likewise to be regarded as lamps;

New proposal is:

Delete the amendment.  This should be taken up when inserting the provisions concerning the AFS.

__________

The original proposal (TRANS/WP.29/GRE/2001/31) was:

2.7.1.1.1.	"Replaceable light source" means a light source which can be inserted in and removed from the holder of its device without tool;

2.7.1.1.2.	"Non-replaceable light source" means a light source which cannot be removed from its device without damaging the device;

in case of a light source module:  a light source which cannot be removed  from its light source module without damaging the light source module.

New proposal is:

"2.7.1.1.1.	"Replaceable light source" means a light source which is designed to be inserted in and removed from the holder of its device without tool;

2.7.1.1.2.	"Non-replaceable light source" means a light source which can only be replaced by replacement of the device to which this light source is fixed;

in case of a light source module: a light source which can only be replaced by replacement of the light source module to which this light source is fixed."

__________

The original proposal (TRANS/WP.29/GRE/2001/31) was:

2.7.1.1.5.	"Gas-discharge light source" means a light source where the element for optical radiation is a discharge arc producing electro-luminescence / fluorescence;

New proposal is:

The proposal shall not be changed.  Do not take out ‘fluorescence’;  the definition is correct.

__________

The original proposal (TRANS/WP.29/GRE/2001/31) was:

2.7.1.1.7.	"Electro-luminescence plate" means a light source where the element for optical radiation is an active layer producing electro-luminescence;

New proposal is:

Delete the proposal and re-submit it as soon as the requirements would be defined.

__________

The original proposal (TRANS/WP.29/GRE/2001/31) was:

The "Annex for clarification" was intended for the GRE document only.

New proposal is:

Insert new annex 10 and add a footnote referring to it to paragraphs 2.7.1. and 2.7.27., to read as follows:

"2.7.1.	Light source  */

.........

2.7.27.	"Distributed lighting system” or “ distributed light-signalling system” (DLS).means a lamp or lamp system comprising a light-generator, light guide(s) and outer lens(es). */

__________

*/	For clarification see annex 10"

Insert new annex 10, to read:

"Annex 10

EXAMPLES OF Light Source options

"Standard"

DLS

Light Source:

Replaceable

Approved to Regulations

LIGHT SOURCE MODULE

Light Source:

Non-replaceable

Non-approved

SEALED

Light Source:

Non-replaceable

Non-approved

*     *     *

# B.	JUSTIFICATION

The above proposal improves the definitions concerned.

It was prepared in response to the first consideration by GRE of document TRANS/WP.29/GRE/2001/31.

__________