| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/GRE/2001/31/Rev.2 19 January 2004 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

(Fifty-second session, 30 March - 2 April 2004,

agenda item 2.1.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48

(Installation of lighting and light-signalling devices)

Transmitted by the expert from the Working Party "Brussels 1952" (GTB)

Note:  The text reproduced below was prepared by the expert from GTB, in order to introduce into the Regulation the remaining provisions concerning Distributed Lighting Systems (DLS).

_______

__________________

Note:	This document is distributed to the Experts on Lighting and Light-Signalling only.

GE.04-20306

Paragraph 2.6., amend to read:

"2.6.	"Device" means an element or an assembly of elements used to perform one or more functions.  A device may also be constituted by the extreme outlet of a light-guide, as part of a distributed lighting or light-signalling system, or another light source, having a built-in outer lens."

Insert new paragraphs 2.7.28. and 2.7.29., to read:

"2.7.28.	"Light transmitting component" means any part of a lamp or lamp system which transmits luminous flux;

2.7.28.1.	"Light-guide" means a light transmitting component based on defraction / reflection;

2.7.29.	"Distributed lighting system" or "distributed light-signalling system" (DLS) means a lamp or lamp system comprising a light-generator, light-guide(s) and outer lens(es);  */

2.7.29.1.	"Light-generator" means an optical device which is a component of a distributed lighting or light-signalling system that collects the luminous flux emitted by the light source(s) and distributes it into one or more light-guides.  A light-generator is to be considered a light transmitting component."

Paragraph 3.2.2., amend to read:

"3.2.2.	a list of the devices or of the DLS prescribed by the manufacturer for the lighting and light-signalling assembly.  The list may include several types of devices for each operation or several DLS.  Each type must be duly identified (component / system, type approval mark, name of the manufacturer, etc.); in addition the list may include in respect of each function the additional annotation "or equivalent devices"."

Paragraph 3.2.3., amend to read:

"3.2.3.	a layout drawing of the lighting and light-signalling equipment as a whole, showing the position of the various devices on the vehicle; in addition, in the case where a distributed lighting and light-signalling system is fitted, the layout of the system showing the position of its various components (light generator(s), light-guide(s), etc.) in the vehicle;"

__________

*/	For clarification see annex 10

Insert a new annex 10, to read:

"Annex 10

EXAMPLES OF LIGHT SOURCE OPTIONS

Light-generator

Light-guide(s)

Outer lens(es)

‘Standard’

DLS

Light Source:

Replaceable

Approved to Regulations

LIGHT SOURCE MODULE

Light Source:

Non-replaceable

Non-approved

SEALED

Light Source:

Non-replaceable

Non-approved

"

_________