Distr.

GENERAL

TRANS/WP.29/GRPE/2001/1

18 October 2000

ENGLISH

Original: ENGLISH and

FRENCH

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Pollution and Energy (GRPE)

(Forty-first session, 16-19 January 2001,

agenda item 8.5.)

APPLICATION OF ECE REGULATIONS TO HYBRID VEHICLES

Transmitted by the Expert from France

Note:	The text reproduced below was prepared by the expert from France and distributed without a symbol (informal document No. 11) during the fortieth session of GRPE (TRANS/WP.29/GRPE/40, paras. 78 and 79).

________________

Note:   This document is distributed to the Experts on Pollution and Energy only.

GE.00-

# INTRODUCTION

- ECE REGULATIONS CONCERNED:

Regulation No. 83 – Emissions of M1 and N1 categories of vehicles

Regulation No. 85 – Measurement of the net power

Regulation No. 101 – Emission of CO2 and fuel consumption measurement

and eventually:

Regulation No. 10 – Electromagnetic compatibility

Regulation No. 51 – Noise of M and N categories of vehicles

Regulation No. 68 – Measurement of maximum speed

Regulation No. 100 – Safety of battery electric vehicles

- SCOPE:  vehicles of categories M1 and N1.

- AIM:

  - Ensure continuity and coherence between Regulations from a pure thermal vehicle to a pure electric vehicle.

  - Hybrid vehicle regulations should take into account their specific characteristics:

- very low exhaust emissions,

- very low CO2 emissions.

*      *      *

# PROPOSAL

- DEFINITIONS AND CATEGORIES OF HYBRID VEHICLES

A hybrid vehicle is a vehicle fitted with at least a thermal engine and at least an electric engine.

The hybrid vehicle classification should, in particular, take into account the following criteria:

- externally electric chargeable,

- range during pure electric mode.

| Pure electric vehicle | Hybrid vehicle |  |  |  | Pure thermal vehicle |
| --- | --- | --- | --- | --- | --- |
|  | Electric vehicle with a range extender | Electric hybrid vehicle |  | Thermal vehicle with electric assistance |  |
| Externally electric chargeable | Externally electric chargeable | Externally electric chargeable | Not externally electric chargeable | Not externally electric chargeable | _ |
| Minimum range in pure electric mode on complete cycle | Minimum range in pure electric mode on complete cycle | Minimum range in pure electric mode on urban cycle | Minimum range in pure electric mode on urban cycle | No range in pure electric mode |  |

  - Definition of a pure electric vehicle

Vehicle in which on-board electric energy is used for traction purpose and which is dependent of external electric infrastructure.

  - Definitions of different hybrid vehicle categories

    - Electric vehicle with a range extender

Pure electric vehicle principle, for which the range is extended by an on-board thermal source, occasionally used, and which is characterized by the following:

- Possibility of charging directly to electric network,

- A normal functionning in pure electric mode,

- A "sufficient" range in pure electric mode, on ECE + EUDC cycle,

- A limited fuel tank capacity,

- Eventually, a thermal power limitation.

    - Electric hybrid vehicle

Electric hybrid vehicles:

- Can run on a pure electric mode,

- Have a "sufficient" urban range in electric mode.

They are divided in two sub-categories:

      - Externally electric chargeable vehicles

      - Not externally electric chargeable vehicles

    - Thermal vehicle with electric assistance

Hybrid vehicle without external charging possibility and with no range in pure electric mode.

2.		AMENDMENTS TO EXISTING ECE REGULATIONS

  - Regulation No. 83 – Emissions of M1 and N1 categories of vehicles

All the requirements of this Regulation could be adapted and apply to hybrid vehicles just like to thermal vehicles:

- 20 °C emissions,

and eventually:

- -7 °C emissions,

- on-board diagnostics (OBD)

- evaporation emissions, etc.

State of charge of the battery should be taken into account.

Eventually, the proper balance of the different functionning modes should be taken into account.

  - Regulation No. 85 – Measurement of the net power

Electric and thermal engine power measurements are already taken into account in this Regulation.

A combination of the electric power and the thermal power is to be considered in order to reach real functionning conditions.

Measurement of the maximum absorbed power on a chassis dynamometer could be considered, if it would be easier to perform.

  - Regulation No. 101 – Emission of CO2 and fuel consumption measurement

    - Fuel consumption

Adaptation of the electric and thermal methods of fuel consumption measurements in this Regulation.

Fuel consumption labelling:

- Electric: expressed in Wh/km,

- Thermal: expressed in l/100 km.

    - CO2 emission

Adaptation of the electric and thermal methods of CO2 measurement in the Regulation.

State of charge of the battery should be taken into account.

Eventually, the proper balance of the different functionning modes should be taken into account.

    - Minimum range in electric mode

The minimum ranges for each category are to be defined (cycle and distance).

  - Other ECE Regulations

Regulation Nos. 10 (Electromagnetic compatibility), 51 (Noise),

68 (Measurement of maximum speed) and 100 (Safety of battery electric vehicles) should be taken into account and adapted to hybrid vehicles, if necessary.

____________