UN/SCEGHS/3/INF.5/Add.6

Sub-Committee of Experts on the Globally

Harmonized System of  Classification

and Labelling of Chemicals

(Third session, 10-12 July 2002)

DRAFT GHS

ANNEX 1

[Blank page]

ANNEX 1

ALLOCATION OF LABEL ELEMENTS

[Blank page]

### ANNEX 1

### Allocation of label elements

A1.1	For the GHS, the assigned pictogram, signal word and hazard statement are given it that order for each hazard category of the hazard class. Where the hazard class and or categories are covered under the UN Model Regulations, the assigned corresponding pictogram is given for each category below the GHS requirements.

| ## Explosives |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Unstable/ Division 1.1 | Division 1.2 | Division 1.3 | Division 1.4 | Division 1.5 | Division 1.6 |
| ## Danger Explosive; mass explosion hazard | Danger Explosive; severe projection hazard | Danger Explosive; fire, blast or projection hazard | 1.4 * Warning Fire or projection hazard | 1.5 * Warning May explode in fire | 1.6 * |
|  |  |  | * Apply to substances or mixtures subject to UN Model Regulations requirements only (see also par. 1.4.10.5.1 (d) (ii)) |  |  |
| Notes on colours of UN Model Regulations pictogram elements: - For Divisions 1.1, 1.2 and 1.3: Symbol exploding bomb: black; background: orange; figure “1” in bottom corner. (2) For Divisions 1.4, 1.5, 1.6: Background: orange; Figures: black; figure “1” in bottom corner. |  |  |  |  |  |

| Flammable gasES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | - | - | Note |
| Danger Extremely flammable gas | No symbol Warning Flammable gas |  |  | Under the UN Model Regulations, the symbol, number and border line may be shown in black instead of white. The background colour stays red in both cases. |
|  | Not required under the UN Model Regulations |  |  |  |

| ## FLAMMABLE AEROSOLS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | - | - | ## Note |
| Danger Extremely flammable aerosol | Warning Flammable aerosol |  |  | Under the UN Model Regulations, the symbol, number and border line may be shown in black instead of white. The background colour stays red in both cases. |

| OXIDIZING GASES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | - | - | - | Notes |
| Danger May cause or intensify fire; oxidizer |  |  |  |  |
|  |  |  |  | Pictogram colours: Symbol and number: black. Background: yellow. |

| ## Gases under pressure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Compressed gas | Liquefied gas | Refrigerated liquefied gas | Dissolved gas | Notes on UN Model Regulations pictogram |
| Warning Contains gas under pressure; may explode if heated | Warning Contains gas under pressure; may explode if heated | Warning Contains refrigerated gas; may cause cryogenic burns or injury | Warning Contains gas under pressure; may explode if heated | (1) Not required for toxic or flammable gases. (2) The symbol, number and border line may be shown in white instead of black. The background stays green in both cases. |

| ## Flammable liquidS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | Category 3 | Category 4 |  |
| Danger Extremely flammable liquid and vapour | Danger ## Highly flammable liquid and vapour | Warning Flammable liquid and vapour | No symbol Warning Combustible liquid | Note on UN Model Regulations pictogram The symbol, number and border line may be shown in black instead of white. The background colour stays red in both cases. |
|  |  |  | NOT REQUIRED UNDER THE UN Model Regulations |  |

| ## FLAMMABLE SOLIDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | - | - | Note |
| Danger Flammable solid | Warning Flammable solid |  |  | Colours for UN Model Regulations Pictogram: Symbol and figure: black. Background: white with seven vertical stripes. |

| ## SELF-REACTIVE SUBSTANCES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type A | Type B | Types C and D | Types E and F | Type G |
| Danger Heating may cause an explosion | ## Danger Heating may cause a fire or explosion | Danger Heating may cause a fire | Warning Heating may cause a fire | No label elements are allocated for this category |
| Same as for explosives (follow same symbol selection process) |  |  |  | Not required under UN Model Regulations |
| Notes: (1) For Type B, under UN Model Regulations, special provision 181 may apply (Exemption of explosive label with competent authority approval. See Chapter 3.3 of UN Model Regulations for more details). (2) For colours of UN Model Regulations pictograms see descriptions for Flammable Solids and Explosives. |  |  |  |  |

| ## PYROPHORIC LIQUIDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | - | - | - | Notes |
| Danger Catches fire spontaneously if exposed to air |  |  |  | Colours of UN Model Regulations pictogram: Symbol and Figure: black. Background: upper half: white; lower half: red. |

| ## PYROPHORIC SOLIDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | - | - | - | Note |
| Danger Catches fire spontaneously if exposed to air |  |  |  | Colours of UN Model Regulations pictogram: Symbol and Figure: black. Background: upper half: white; lower half: red. |

| ## SELF-HEATING SUBSTANCES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | - | - | Note |
| Danger Self-heating; may catch fire | Warning Self-heating in large quantities; may catch fire |  |  | Colours of UN Model Regulations pictogram: Symbol and Figure: black. Background: upper half: white; lower half: red. |

| ## Substances, which in contact with water, emit flammable gases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | Category 3 | - | ## Note |
| Danger In contact with water releases flammable gases which may ignite spontaneously | Danger In contact with water releases flammable gases | Warning In contact with water releases flammable gases |  | Under the UN Model Regulations, the symbol, number and border line may be shown in black instead of white. The background stays blue in both cases. |

| ## OXIDIZING LIQUIDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | Category 3 | - | Note |
| Danger May cause fire or explosion; strong oxidizer | Danger ## May intensify fire; oxidizer | Warning May intensify fire; oxidizer |  | UN Model Regulations Pictogram colours: Symbol and number: black. Background: yellow. |

| ## OXIDIZING SOLIDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | Category 3 | - | Note |
| ## Danger May cause fire or explosion; strong oxidizer | Danger May intensify fire; oxidizer | Warning May intensify fire; oxidizer |  | UN Model Regulations Pictogram colours: Symbol and number: black. Background: yellow. |

| ## ORganic Peroxides |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type A | Type B | Types C and D | Types E and F | Type G |
| Danger Heating may cause An explosion | Danger Heating may cause a fire or explosion | Danger Heating may cause a fire | Warning Heating may cause a fire | There are no label elements allocated to this hazard class. |
| Same as for explosives (follow same symbol selection process) |  |  |  | NOT REQUIRED UNDER UN Model Regulations |
| Notes: (1) For Type B, under UN Model Regulations, special provision 181 may apply (Exemption of explosive label with competent authority approval. See Chapter 3.3 of UN Model Regulations for more details). (2) UN Model Regulations Pictogram colours: See tables for Oxidizing liquids and Explosives. |  |  |  |  |

| ## Corrosive to metals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | - | - | - | Note |
| Warning May be corrosive to metals |  |  |  | UN Model Regulations pictogram colours: Symbol: black background: upper half: white; lower half: black with white border; Figure 8: white. |

| ## ACUTE TOXICITY: ORAL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | Category 3 | Category 4 | Category 5 |
| Danger Fatal if swallowed | Danger Fatal if swallowed | Danger Toxic if swallowed | Warning Harmful if swallowed | No symbol Warning May be harmful if swallowed |
|  |  |  | NOT REQUIRED UNDER THE UN Model Regulations Notes: For gases and aerosols under the UN Model Regulations , replace the number 6 in the bottom corner of the pictogram by 2. UN Model Regulations pictogram colours: Symbol and figure: black. Background: white. |  |

| ## Acute Toxicity: dermal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | Category 3 | Category 4 | Category 5 |
| Danger Fatal in contact with skin | Danger Fatal in contact with skin | Danger Toxic in contact with skin | Warning Harmful in contact with skin | No symbol Warning May be harmful in contact with skin |
|  |  |  | NOT REQUIRED UNDER THE UN Model Regulations Notes: For gases and aerosols under the UN Model Regulations, replace the number 6 in the bottom corner of the pictogram by 2. For UN Model Regulations pictogram colours: Symbol and figure: black. Background: white. |  |

| ## Acute Toxicity: inhalation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | Category 3 | Category 4 | Category 5 |
| Danger Fatal if inhaled | Danger Fatal if inhaled | Danger Toxic if inhaled | Warning Harmful if inhaled | No symbol Warning May be harmful if inhaled |
|  |  |  | # NOT REQUIRED UNDER THE UN Model Regulations Notes: For gases and aerosols under the UN Model Regulations, replace the number 6 in the bottom corner of the pictogram by 2. For UN Model Regulations pictogram colours: Symbol and figure: black. Background: white. |  |

| ## SKIN CORROSION/IRRITATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1A | Category 1B | Category 1C | Category 2 | Category 3 |
| Danger Causes severe skin burns and eye damage | Danger Causes severe skin burns and eye damage | Danger Causes severe skin burns and eye damage | Warning Causes skin irritation | No symbol Warning Causes mild skin irritation |
|  |  |  | # NOT REQUIRED UNDER THE UN Model Regulations Note: For UN Model Regulations pictogram colours: Symbol: black background: upper half: white; lower half: black with white border; Figure 8: white. |  |

| ## SEVERE EYE DAMAGE/ EYE IRRITATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2A | Category 2B | - | - |
| Danger Causes severe eye damage | Warning Causes severe eye irritation | No symbol Warning Causes eye irritation |  |  |
| NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |

| Respiratory SENSITIZATION * |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category 1 |  | - |  | - |  | - |  | - |  |
| Danger May cause allergy or asthma symptoms or breathing difficulties if inhaled |  | * Note: There has been considerable discussion about what to convey to those exposed about sensitisation effects, and at what point it should be conveyed. While the current cut-off for mixtures is 1%, it appears that the major systems all believe information should be conveyed below that level. This may be appropriate both to warn those already sensitised, as well as to warn those who may become sensitised. [Reserved]* |  |  |  |  |  |  |  |
| ## NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |  |  |  |  |  |
| DERMAL SENSITIZATION* |  |  |  |  |  |  |  |  |  |
| Category 1 | - |  | - |  | - |  | - |  |  |
| Warning May cause an allergic skin reaction | Note: There has been considerable discussion about what to convey to those exposed about sensitisation effects, and at what point it should be conveyed. While the current cut-off for mixtures is 1%, it appears that the major systems all believe information should be conveyed below that level. This may be appropriate both to warn those already sensitised, as well as to warn those who may become sensitised. [Reserved] * |  |  |  |  |  |  |  |  |
| NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |  |  |  |  |  |

| Germ cell mutagenicity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1A | Category 1B | Category 2 | - | - |

| Danger May cause genetic defects (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Danger May cause genetic defects (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) | Warning Suspected of causing genetic defects (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |  |
| --- | --- | --- | --- | --- |
| NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |

| CARCINOGENICITY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1A | Category 1B | Category 2 | - | - |
| Danger May cause cancer (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Danger May cause cancer (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Warning Suspected of causing cancer (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) |  |  |
| ## NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |

| ## TOXIC TO REPRODUCTION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1A | Category 1B | Category 2 | Additional Category | - |
| Danger May damage fertility or the unborn child (state specific effect if known or route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Danger May damage fertility or the unborn child (state specific effect if known or route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Warning Suspected of damaging fertility or the unborn child (state specific effect if known or route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Effects on or via lactation May cause harm to breast-fed children. |  |
| NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |

| TARGET ORGAN SYSTEMIC TOXICITY (SINGLE EXPOSURE) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | - | - | - |
| Danger Causes damage to (state all organs affected, or use a general statement where there is no definite evidence that other organs are not affected ) if (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Warning May cause damage to (state all organs affected, or use a general statement where there is no definite evidence that other organs are not affected ) if (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) |  |  |  |
| NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |

| TARGET ORGAN SYSTEMIC TOXICITY (REPEATED EXPOSURE) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | - | - | - |
| Danger Causes (state all organs affected, or use a general statement where there is no definite evidence that other organs are not affected ) damage through prolonged or repeated exposure (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) | Warning May cause (state all organs affected, or use a general statement where there is no definite evidence that other organs are not affected ) damage through prolonged or repeated exposure (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) |  |  |  |
| NOT REQUIRED UNDER THE UN Model Regulations |  |  |  |  |

| AQUATIC TOXICITY (ACUTE) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## Category 1 | Category 2 | Category 3 | - | Note |
| Warning Very toxic to aquatic life | No symbol No signal word Toxic to aquatic life | No symbol No signal word Harmful to aquatic life |  | Presently not covered under the UN Model Regulations if the substance presents any other hazards covered by UN Model Regulations . If no other hazard is presented, the UN Model Regulations Class 9 label is applicable. The UN Model Regulations requirements are being revised. |

| AQUATIC TOXICITY (CHRONIC) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1 | Category 2 | ## Category 3 | Category 4 | Note |
| Warning Very toxic to aquatic life with long lasting effects | No signal word Toxic to aquatic life with long lasting effects | No symbol No signal word Harmful to aquatic life with long lasting effects | No symbol No signal word May cause long lasting harmful effects to aquatic life | Presently not covered under the UN Model Regulations if the substance presents any other hazards covered by UN Model Regulations . If no other hazard is presented, UN Model Regulations Class 9 label is applicable. The UN Model Regulations requirements being revised. |

________________

* 	Further work is included in the work programme of the Sub-Committee of Experts on the GHS.

* 	Further work is included in the work programme of the Sub-Committee of Experts on the GHS.