UN/SCEGHS/3/INF.5/Add.6

Sub-Committee of Experts on the Globally

Harmonized System of  Classification

and Labelling of Chemicals

(Third session, 10-12 July 2002)

DRAFT GHS

ANNEX 2

[Blank page]

# ANNEX 2

# CLASSIFICATION AND LABELLING

# SUMMARY TABLES

[blank page]

## A2 - Classification and labelling summary tables

### A2.1	Explosives (see Chapter 2.1 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| Division 1.1 | According to the results of the test in Part I of the Manual of Tests and Criteria. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Explosive; mass explosion hazard |
| Division 1.2 | According to the results of the test in Part I of the Manual of Tests and Criteria. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Explosive; severe projection hazard |
| Division 1.3 | According to the results of the test in Part I of the Manual of Tests and Criteria. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Explosive; fire, blast or projection hazard |
| Division 1.4 | According to the results of the test in Part I of the Manual of Tests and Criteria. | Symbol | 1.4 |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Fire or projection hazard |
| Division 1.5 | According to the results of the test in Part I of the Manual of Tests and Criteria. | Symbol | 1.5 |
|  |  | Signal word | Warning |
|  |  | Hazard statement | May explode in fire |
| Division 1.6 | According to the results of the test in Part I of the Manual of Tests and Criteria. | Symbol | 1.6 |
|  |  | Signal word | No signal word |
|  |  | Hazard statement | No hazard statement |

# A2.2.	Flammable gases (See Chapter 2.2 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Gases and gas mixtures, which at 20 °C and a standard pressure of 101.3 kPa: - are ignitable when in a mixture of 13% or less by volume in air; or - have a flammable range with air of at least 12 percentage points regardless of the lower flammable limit. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Extremely flammable gas |
| 2 | Gases or gas mixtures, other than those of category 1, which, at 20 °C and a standard pressure of 101.3 kPa, have a flammable range while mixed in air. | Symbol | No symbol used |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Flammable gas |

# A2.3	Flammable aerosols (See Chapter 2.3 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | On the basis of its components, of its chemical heat of combustion and, if applicable, of the results of the foam test, for foam aerosols, and of the ignition distance test and enclosed space test, for spray aerosols (see decision logic in 2.3.4.1 of Chapter 2.3). | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Extremely flammable aerosol |
| 2 | On the basis of its components, of its chemical heat of combustion and, if applicable, of the results of the foam test, for foam aerosols, and of the ignition distance test and enclosed space test, for spray aerosols (see decision logic in 2.3.4.1 of Chapter 2.3). | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Flammable aerosol |

# A2.4	Oxidizing gases (See Chapter 2.4 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Any gas which may, generally by providing oxygen, cause or contribute to the combustion of other material more than air does. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May cause or intensify fire; oxidizer |

### A2.5	Gases under pressure (See Chapter 2.5 for details)

| Hazard category | Criteria | Hazard communication elements |  |  |
| --- | --- | --- | --- | --- |
| Compressed gas | A gas, which when packaged under pressure is entirely gaseous at -50 °C; including all gases with a critical temperature  -50 °C. |  | Symbol |  |
|  |  |  | #### Signal word | Warning |
|  |  |  | Hazard statement | Contains gas under pressure; may explode if heated |
| Liquefied gas | A gas which when packaged under pressure, is partially liquid at temperatures above -50 °C. A distinction is made between: i) High pressure liquefied gas: a gas with a critical temperature between –50 °C and +65 °C; and ii) Low pressure liquefied gas: a gas with a critical temperature above +65 °C. |  | Symbol |  |
|  |  |  | Signal word | Warning |
|  |  |  | Hazard statement | Contains gas under pressure; may explode if heated |
| Refrigerated liquefied gas | A gas which when packaged is made partially liquid because of its low temperature. |  | Symbol |  |
|  |  |  | Signal word | Warning |
|  |  |  | Hazard statement | Contains refrigerated gas; may cause cryogenic burns or injury |
| Dissolved gas | A gas which when packaged under pressure is dissolved in a liquid phase solvent. |  | Symbol |  |
|  |  |  | Signal Word | Warning |
|  |  |  | Hazard statement | Contains gas under pressure; may explode if heated |

### A2.6	Flammable liquids (See Chapter 2.6 for details)

| Hazard category | ## Criteria | Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Flash point < 23 °C and initial boiling point ≤ 35 °C | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Extremely flammable liquid and vapour |
| 2 | Flash point < 23 °C and initial boiling point >35 °C | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Highly flammable liquid and vapour |
| 3 | Flash point  23 °C and ≤ 60 °C | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Flammable liquid and vapour |
| 4 | Flash point > 60 °C and ≤ 93 °C | Symbol | #### No symbol used |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Combustible liquid |

### A2.7	Flammable solids (See Chapter 2.7 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Burning rate test: Substances other than metal powders: - wetted zone does not stop fire and - burning time < 45 seconds or burning rate > 2.2 mm/s Metal powders: - burning time  5 minutes | #### Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Flammable solid |
| 2 | Burning rate test: Substances other than metal powders: - wetted zone stops the fire for at least 4 minutes and - burning time < 45 seconds or burning rate > 2.2 mm/second Metal powders : - burning time > 5 minutes and  10 minutes | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Flammable solid |

### A2.8	Self-reactive substances (See Chapter 2.8 for details)

| Hazard category | ##### Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| ##### Type A | According to the results of tests in the Manual of Tests and Criteria, Part II and the application of the decision logic under 2.8.4.1 of Chapter 2.8. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Heating may cause an explosion |
| Type B | According to the results of tests in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.8.4.1 of Chapter 2.8. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Heating may cause a fire or explosion |
| Type C and D | According to the results of tests in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.8.4.1 of Chapter 2.8. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Heating may cause a fire |
| Type E and F | According to the results of tests in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.8.4.1 of Chapter 2.8. | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Heating may cause a fire |
| Type G | According to the results of tests in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.8.4.1 of Chapter 2.8. | Signal word | There are no label elements allocated to this hazard category. |
|  |  | Symbol |  |
|  |  | Hazard statement |  |

### A2.9	Pyrophoric liquids (See Chapter 2.9 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | The liquid ignites within 5 min when added to an inert carrier and exposed to air, or it ignites or chars a filter paper on contact with air within 5 min. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Catches fire spontaneously if exposed to air |

### A2.10	Pyrophoric solids (See Chapter 2.10 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | The solid ignites within 5 minutes of coming into contact with air. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Catches fire spontaneously if exposed to air |

### A2.11	Self-heating substances (See Chapter 2.11 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | A positive result is obtained in a test using a 25 mm sample cube at 140 °C | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Self-heating; may catch fire |
| 2 | (a) A positive result is obtained in a test using a 100 mm sample cube at 140 °C and a negative result is obtained in a test using a 25 mm cube sample at 140 °C and the substance is to be packed in packages with a volume of more than 3 m3; or (b) A positive result is obtained in a test using a 100 mm sample cube at 140 °C and a negative result is obtained in a test using a 25 mm cube sample at 140 °C, a positive result is obtained in a test using a 100 mm cube sample at 120 °C and the substance is to be packed in packages with a volume of more than 450 litres; or (c) A positive result is obtained in a test using a 100 mm sample cube at 140 °C and a negative result is obtained in a test using a 25 mm cube sample at 140 °C and a positive result is obtained in a test using a 100 mm cube sample at 100 °C | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Self-heating in large quantities; may catch fire |

### A2.12	Substances, which on contact with water, emit flammable gases (See Chapter 2.12 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Any substance which reacts vigorously with water at ambient temperatures and demonstrates generally a tendency for the gas produced to ignite spontaneously, or which reacts readily with water at ambient temperatures such that the rate of evolution of flammable gas is equal to or greater than 10 litres per kilogram of substance over any one minute. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | In contact with water releases flammable gases which may ignite spontaneously |
| 2 | Any substance which reacts readily with water at ambient temperatures such that the maximum rate of evolution of flammable gas is equal to or greater than 20 litres per kilogram of substance per hour, and which does not meet the criteria for category 1. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | In contact with water releases flammable gases |
| 3 | Any substance which reacts slowly with water at ambient temperatures such that the maximum rate of evolution of flammable gas is equal to or greater than 1 litre per kilogram of substance per hour, and which does not meet the criteria for categories 1 and 2. | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | In contact with water releases flammable gases |

### A2.13	Oxidizing liquids (See Chapter 2.13 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Any substance which, in the 1:1 mixture, by mass, of substance and cellulose tested, spontaneously ignites; or the mean pressure rise time of a 1:1 mixture, by mass, of substance and cellulose is less than that of a 1:1 mixture, by mass, of 50% perchloric acid and cellulose. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May cause fire or explosion; strong oxidizer. |
| 2 | Any substance which, in the 1:1 mixture, by mass, of substance and cellulose tested, exhibits a mean pressure rise time less than or equal to the mean pressure rise time of a 1:1 mixture, by mass, of 40% aqueous sodium chlorate solution and cellulose; and the criteria for category 1 are not met. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May intensify fire; oxidizer. |
| 3 | Any substance which, in the 1:1 mixture, by mass, of substance and cellulose tested, exhibits a mean pressure rise time less than or equal to the mean pressure rise time of a 1:1 mixture, by mass, of 65% aqueous nitric acid and cellulose; and the criteria for categories 1 and 2 are not met. | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | May intensify fire; oxidizer. |

### A2.14	Oxidizing solids (See Chapter 2.14 for details)

| Hazard category | ##### Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Any substance which, in the 4:1 or 1:1 sample-to-cellulose ratio (by mass) tested, exhibits a mean burning time less than the mean burning time of a 3:2 mixture, by mass, of potassium bromate and cellulose. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May cause fire or explosion; strong oxidizer |
| 2 | Any substance which, in the 4:1 or 1:1 sample-to-cellulose ratio (by mass) tested, exhibits a mean burning time equal to or less than the mean burning time of a 2:3 mixture (by mass) of potassium bromate and cellulose and the criteria for category 1 are not met. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May intensify fire; oxidizer |
| 3 | Any substance which, in the 4:1 or 1:1 sample-to-cellulose ratio (by mass) tested, exhibits a mean burning time equal to or less than the mean burning time of a 3:7 mixture (by mass) of potassium bromate and cellulose and the criteria for categories 1 and 2 are not met. | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | May intensify fire; oxidizer |

### A2.15	Organic peroxides (See Chapter 2.15 for details)

| Hazard category | Criteria | Hazard communication elements |  |
| --- | --- | --- | --- |
| Type A | According to the results of test series A to H in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.15.4.1 of Chapter 2.15. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Heating may cause an explosion |
| Type B | According to the results of test series A to H in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.15.4.1 of Chapter 2.15. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Heating may cause a fire or explosion |
| Type C and D | According to the results of test series A to H in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.15.4.1 of Chapter 2.15. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Heating may cause a fire |
| Type E and F | According to the results of test series A to H in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.15.4.1 of Chapter 2.15. | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Heating may cause a fire |
| Type G | According to the results of test series A to H in the Manual of Tests and Criteria, Part II and the application of the decision logic under para. 2.15.4.1 of Chapter 2.15. | Signal word | There are no label elements allocated to this hazard category. |
|  |  | Symbol |  |
|  |  | Hazard statement |  |

### A2.16	Corrosive to metals (See Chapter 2.16 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Corrosion rate on steel or aluminium surfaces exceeding 6.25 mm per year at a test temperature of 55 °C. | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | May be corrosive to metals |

### A2.17	Acute toxicity (See Chapter 3.1 for details)

| Hazard category | Criteria | Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | LD50 < 5 mg/kg bodyweight (oral) LD50 < 50 mg/kg bodyweight (dermal) LC50 < 100 ppm (gas) LC50 < 0.5 (mg/l) (vapour) LC50 < 0.05 (mg/l) (dust,mist) | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Fatal if swallowed. (oral) Fatal in contact with skin (dermal) Fatal if inhaled (gas, vapour, dust, mist) |
| 2 | LD50 between 5 and less than 50 mg/kg bodyweight (oral) LD50 between 50 and less than 200 mg/kg bodyweight (dermal) LC50 between 100 and less than 500 ppm (gas) LC50 between 0.5 and less than2.0 (mg/l) (vapour) LC50 between 0.05 and less than 0.5 (mg/l) (dust, mist) | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard Statement | Fatal if swallowed. (oral) Fatal in contact with skin (dermal) Fatal if inhaled (gas, vapour, dust, mist) |
| 3 | LD50 between 50 and less than 300 mg/kg bodyweight (oral) LD50 between 200 and less than 1000 mg/kg bodyweight (dermal) LC50 between 500 and less than 2500 ppm (gas) LC50 between 2.0 and less than 10.0 (mg/l) (vapour) LC50 between 0.5 and less than 1.0 (mg/l) (dust, mist) | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Toxic if swallowed. (oral) Toxic in contact with skin (dermal) Toxic if inhaled (gas, vapour, dust, mist) |
| Continued on next page |  |  |  |
| (cont'd) |  |  |  |
| 4 | LD50 between 300 and less than 2000 mg/kg bodyweight (oral) LD50 between 1000 and less than 2000 mg/kg bodyweight (dermal) LC50 between 2500 and less than 5000 ppm (gas) LC50 between 10.0 and less than 20.0 (mg/l) (vapour) LC50 between 1.0 and less than 5.0 (mg/l) (dust, mist) | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Harmful if swallowed. (oral) Harmful in contact with skin (dermal) Harmful if inhaled (gas, vapour, dust, mist) |
| 5 | LD50 between 2000 and 5000 (oral or dermal) For gases, vapours, dusts, mists, LC50 in the equivalent range of the oral and dermal LD50 (i.e., between 2000 and 5000 mg/kg bodyweight) See also the additional criteria - Indication of significant effect in humans - Any mortality at Category 4 - Significant clinical signs at Category 4 - Indication from other studies | Symbol | No symbol |
|  |  | Signal word | Warning |
|  |  | Hazard statement | May be harmful if swallowed (oral) May be harmful in contact with skin (dermal) May be harmful if inhaled (gas, vapour, dust, mist) |

### A2.18	Skin corrosion/irritation (See Chapter 3.2 for details)

| Hazard category | Criteria |  | Hazard communication elements |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 Corrosive Including sub-categories - A, B, and C; - see Chapter 3.2, Table 3.2.1 | 1. For Substances and Tested Mixtures: - Human experience showing irreversible damage to the skin; - Structure/activity or structure property relationship to a substance or mixture already classified as corrosive; - pH extremes of < 2 and > 11.5 including acid/alkali reserve capacity; - Positive results in a valid and accepted in vitro dermal corrosion test; or - Animal experience or test data that indicate that the substance/mixture causes irreversible damage to the skin following exposure of up to 4 hours (See Table 3.2.1) 2. If data for a mixture are not available, use bridging principles in sub-section 3.2.3.2. 3. If bridging principles do not apply, - For mixtures where substances can be added: Classify as corrosive if the sum of the concentrations of corrosive substances in the mixture is > 5% (for substances with additivity); or - For mixtures where substances cannot be added:> 1%. See paragraph 3.2.3.3.4. |  | Symbol |  |  |
|  |  |  | Signal word | Danger |  |
|  |  |  | Hazard statement | Causes severe skin burns and eye damage |  |
| Continued on next page |  |  |  |  |  |
| Hazard category (cont'd) |  | Criteria | Hazard communication elements |  |  |
| - 2 - Irritant - (applies to all authorities) |  | 1. For Substances and Tested Mixtures - Human experience or data showing reversible damage to the skin following exposure of up to 4 hours; - Structure/activity or structure property relationship to a substance or mixture already classified as an irritant; - Positive results in a valid and accepted in vitro dermal irritation test; or - Animal experience or test data that indicate that the substance/mixture causes reversible damage to the skin following exposure of up to 4 hours , mean value of > 2.3 < 4.0 for erythema/eschar or for oedema , or inflammation that persists to the end of the observation period, in 2 of 3 tested animals (Table3.2.2). - If data for a mixture are not available, use bridging principles in sub-section 3.2.3.2. - If bridging principals do not apply, classify as an irritant if: - For mixtures where substances can be added: the sum of concentrations of corrosive substances in the mixture is > 1% but < 5%; the sum of the concentrations of irritant substances is > 10%; or the sum of (10 x the concentrations of corrosive ingredients) + ( the concentrations of irritant ingredients) is > 10%; or - For mixtures where substances cannot be added: > 3%. (See paragraph 3.2.3.3.4) | - Symbol |  |  |
|  |  |  | Signal word | Warning |  |
|  |  |  | Hazard statement | Causes skin irritation |  |
| Continued on next page |  |  |  |  |  |
| Hazard category (cont'd) |  | Criteria | Hazard communication elements |  |  |
| - 3 - Mild Irritant - (applies to some authorities) |  | 1. For Substances and Tested Mixtures - Animal experience or test data that indicates that the substance/mixture causes reversible damage to the skin following exposure of up to 4 hours, mean value of > 1.5 < 2.3 for erythema/eschar in 2 of 3 tested animals (See Table 3.2.2) 2. If data for a mixture are not available and the bridging principles in sub-section 3.2.3.2. 3. If bridging principles do not apply, classify as mild irritant if: - For mixtures where substances can be added the sum of the concentrations of irritant substances in the mixture is > 1% but < 10%; - For mixtures where substances cannot be added: the sum of the concentrations of mild irritant substances is > 10%; - the sum of (10 x the concentrations of corrosive substances) + (the concentrations of irritant substances) is > 1% but < 10%; or - the sum of (10 x the concentrations of corrosive substances) + (the concentrations of irritant substances) + (the concentrations of mild irritant substances ) is > 10%. | Symbol |  | None |
|  |  |  | Signal word |  | Warning |
|  |  |  | Hazard statement |  | Causes mild skin irritation |

### A2.19	Serious eye damage / eye irritation (See Chapter 3.3 for details)

| Hazard category | Criteria | Hazard communication elements |  |  |
| --- | --- | --- | --- | --- |
| 1 Irrever-sible Effects | 1. For Substances and Tested Mixtures - Classification as corrosive to skin; - Human experience or data showing damage to the eye which is not fully reversible within 21 days; - Structure/activity or structure property relationship to a substance or mixture already classified as corrosive; - pH extremes of < 2 and > 11.5 including buffering capacity; - Positive results in a valid and accepted in vitro test to assess severe damage to eyes; or - Animal experience or test data that the substance or mixture produces either (1) in at least one animal, effects on the cornea, iris or conjunctiva that are not expected to reverse or have not reversed; or (2) in at least 2 of 3 tested animals a positive response of corneal opacity > 3 and/or iritis >1.5. ( See Table3.3.1) 2. If data for a mixture are not available, use bridging principles in sub-section 3.3.3.2. 3. If bridging principles do not apply, - For mixtures where substances can be added: Classify as Category 1 if the sum of the concentrations of substances classified as corrosive to the skin and/or eye Category 1 substances in the mixture is > 3% or - For mixtures where substances cannot be added:> 1 See paragraph 3.3.3.3.4. | Symbol |  |  |
|  |  | Signal word | Danger |  |
|  |  | Hazard statement | Causes severe eye damage |  |
| Continued on next page |  |  |  |  |
| Hazard category (cont'd) | Criteria | Hazard communication elements |  |  |
| 2A Irritant | 1. Substances and tested mixtures - Classification as severe skin irritant; - Human experience or data showing production of changes in the eye which are fully reversible within 21 days; - Structure/activity or structure property relationship to a substance or mixture already classified as an eye irritant; - Positive results in a valid and accepted in vitro eye irritation test; or - Animal experience or test data that indicate that the substance/mixture produces a positive response in at least 2 of 3 tested animals of : corneal opacity >1, iritis >1, or conjunctival edema (chemosis) >2 (Table 3.3.2). 2. If data for a mixture are not available, use bridging principles in sub-section 3.3.3.2. 3. If bridging does not apply, classify as an irritant (2A) if: (a) For mixtures where substances can be added: the sum of the concentrations of skin and/or eye Category 1 substances in the mixture is > 1% but < 3%; the sum of the concentrations of eye irritant substances is > 10%; or the sum of (10 x the concentrations of skin and/or eye category 1 substances) + ( the concentrations of eye irritants) is > 10% (b) For mixtures where substances cannot be added: the sum of the concentrations of eye irritant ingredients is > 3% (See paragraph 3.3.3.3.4) | Symbol |  |  |
|  |  | Signal word | Warning |  |
|  |  | Hazard statement | Causes severe eye irritation |  |
| 2B Mild Irritant | 1. For Substances and tested mixtures - Human experience or data showing production of mild eye irritation; - Animal experience or test data that indicate that the lesions are fully reversible within 7 days. (See Table 3.3.2) 2. If data for a mixture are not available, use bridging principles in sub-section 3.3.3.2. 3. If bridging does not apply, classify as an irritant (2A) if: (a)For mixtures where substances can be added: the sum of the concentrations of skin and/or eye Category 1 substances in the mixture is > 1% but < 3%; the sum of the concentrations of eye irritant substances is > 10%; or the sum of (10 x the concentrations of skin and/or eye category 1 substances) + ( the concentrations of eye irritants) is > 10% (b) For mixtures where substances cannot be added: the sum of the concentrations of eye irritant ingredients is > 3% (See paragraph 3.3.3.3.4) | Symbol | No symbol |  |
|  |  | Signal word | Warning |  |
|  |  | Hazard statement | Causes eye irritation |  |

### A2.20	Respiratory sensitiser (See Chapter 3.4 for details)

| Hazard category | Criteria | Hazard communication element |  |
| --- | --- | --- | --- |
| 1 | 1. For Substances and Tested Mixture If there is human evidence that the individual substance induces specific respiratory hypersensitivity, and/or Where there are positive results from an appropriate animal test 2. If these mixture meets the criteria set forth in the “Bridging Principles” through one of the following: - Dilution - Batching - Substantially Similar Mixture 3. If bridging principles do not apply, classify if any individual respiratory sensitiser in the mixture has a concentration of: ≥ 1.0% Solid/Liquid ≥ 0.2% Gas | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May cause allergic or asthmatic symptoms or breathing difficulties if inhaled |

### A2.21	Skin sensitiser  (See Chapter 3.4 for details)

| Hazard category | Criteria | Hazard communication element |  |
| --- | --- | --- | --- |
| 1 | 1. For Substances and tested mixture If there is evidence in humans that the individual substance can induce sensitisation by skin contact in a substantial number of persons, or Where there are positive results from an appropriate animal test 2. If the mixture meets the criteria set forth in the “Bridging Principles” through one of the following: - Dilution - Batching - Substantially similar mixture 3. If bridging principles do not apply, Classify if any individual skin sensitiser in the mixture has a concentration of: ≥ 1.0% Solid/Liquid/Gas | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard Statement | May cause allergic skin reaction |

### A2.22	Mutagenicity  (See Chapter 3.5 for details)

| ## Hazard Category | Criteria for classification | Hazard communication elements |  |
| --- | --- | --- | --- |
| 1A | Known to induce heritable mutations or regarded as if it induces heritable mutations in the germ cells of humans (see criteria in section 3.5.2) or mixtures containing 0.1 % of such a substance | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May cause genetic defects (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |
| 1B | to be regarded as if inducing heritable mutations in the germ cells of humans (see criteria in section 3.5.2) or mixture containing 0.1/ of such a substance | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | May cause genetic defects (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard |
| 2 | Causes concern for man owing to the possibility that it may induce heritable mutations in the germ cells of humans (see criteria in section 3.5.2) or mixtures containing 1.0 % of such a substance | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard Statement | Suspected of causing genetic defects (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |

### A2.23	Carcinogenicity  (See Chapter 3.6 for details)

| Hazard category | Criteria | Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 (both 1A and 1B) | Known or Presumed Human Carcinogen including mixtures containing  0.1% of such a substance | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard satement | May cause cancer (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard. |
| 2 | Suspected human carcinogen Including mixtures containing more than  0.1 or 1.0 % of such a substance (See Notes 1 and 2 in Table 3.6.1 of Chapter 3.6) | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Suspected of causing cancer (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard*) |

*	 Some authorities will choose to label according to this provision, others may not.

### A2.24 (a)	Toxic to reproduction  (See Chapter 3.7 for details)

| Hazard category | # Criteria | # Hazard communication elements |  |  |
| --- | --- | --- | --- | --- |
| 1 (Both 1A and 1B) | Known or presumed human reproductive toxicants (see criteria in paragraphs 3.7.2.2.1 to 3.7.2.6.0 of Chapter 3.7) or mixtures containing  0.1% or 0.3 % of such a substance (See notes1 and 2 of Table 3.7.1, Chapter 3.7) | Symbol |  |  |
|  |  | Signal word | Danger |  |
|  |  | Hazard statement | May damage fertility or the unborn child (state specific effect if known or route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |
| 2 | Suspected human reproductive toxicants (see criteria in paragraphs 3.7.2.2.1 to 3.7.2.6.0 of Chapter 3.7) or mixtures containing  0.1% or 3.0 % of such a substance (See Notes 3 and 4 of Table 3.7.1, Chapter 3.7) | Symbol |  |  |
|  |  | Signal word | Warning |  |
|  |  | Hazard statement | Suspected of damaging fertility or the unborn child (state specific effect if known or route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |
| Continued on next page |  |  |  |  |

### A2.24 (b)	Effects on or via lactation (See Chapter 3.???)

| Hazard category | # Criteria | # Hazard communication elements |  |
| --- | --- | --- | --- |
| Special category | Substances which cause concern for the health of breastfed children (see criteria in paragraphs 3.7.2.2.1 to 3.7.2.6.0 and 3.7.3.4 of Chapter 3.7) | Symbol | No symbol |
|  |  | Signal word | No signal word |
|  |  | Hazard Statement | May cause harm to breast-fed children. |

### A2.25	Target organ systemic toxicity following single exposure (See Chapter 3.8 for details)

| # Hazard category | Criteria | Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Reliable evidence on the substance or mixture (including bridging) of an adverse effect on specific organ/systems or systemic toxicity in humans or animals. May use guidance values in Table 3.8.1, Category 1 criteria as part of weight of evidence evaluation. May be named for specific organ/system.] Mixture that lacks sufficient data, but contains Category 1 ingredient at a concentration of > 1.0 to < 10.0% for some authorities; and >10.0% for all authorities. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Causes damage to organs (state all organs affected, if known) through prolonged or repeated exposure (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |
| 2 | Evidence on the substance or mixture (including bridging) of an adverse effect on specific organ/systems or systemic toxicity from animal studies or humans considering weight of evidence and guidance values in Table 3.8.1, Category 2 criteria. May be named for specific organ/system affected. Mixture that lacks sufficient data, but contains Category 1 ingredient: > 1 but <10% for some authorities; and /or contains Category 2 ingredient: > 1 to <10% for some authorities; and >10% for all authorities | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | May causes damage to organs (state all organs affected, if known) through prolonged or repeated exposure (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |

### A2.26	Target organ systemic toxicity following repeat exposure (See Chapter 3.9 for details)

| # Hazard category | Criteria | Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | Reliable evidence on the substance or mixture (including bridging) of an adverse effect on specific organ/systems or systemic toxicity in humans or animals. May use guidance values in Table 3.91 as part of weight of evidence evaluation. May be named for specific organ/system Mixture that lacks sufficient data, but contains Category 1 ingredient: > 1 to < 10% for some authorities; and >10% for all authorities. | Symbol |  |
|  |  | Signal word | Danger |
|  |  | Hazard statement | Causes damage to organs (state all organs affected, if known) through prolonged or repeated exposure (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard ) |
| 2 | Evidence on the substance or mixture (including bridging) of an adverse effect on specific organ/systems or systemic toxicity from animal studies or humans considering weight of evidence and guidance values in Table 3.9.2 criteria. May be named for specific organ/system. Mixture that lacks sufficient data, but contains Category 1 ingredient: > 1.0 but <10% for some authorities (See Note 3 of Table 3.9.3) and /or contains Category 2 ingredient: > 1.0 or >10% | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | May cause damage to organs (state all organs affected, if known) through prolonged or repeated exposure (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |

_____________