UN/SCEGHS/3/INF.16

Sub-Committee of Experts on the Globally

Harmonized System of  Classification

and Labelling of Chemicals

(Third session, 10-12 July 2002)

## Amendment to the GHS

Transmitted by the Organisation for Economic Co-Operation and Development (OECD)

Proposal

Annex 3 of document ST/SG/AC.10/C.4/2001/26 shows classification and labelling summary tables. The classification and labelling summary tables regarding chapter 3.10 on hazards to the aquatic environment were missing and are proposed here below.

# Acute hazards to the aquatic environment (See Chapter 3.10 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | 1. For Substances and Tested Mixtures: - L(E)C50  1mg/L where L(E)C50 is either fish 96hr LC50, crustacea 48hr EC LC50 or aquatic plant 72 or 96hr ErC50 2. If data for a mixture are not available, use bridging principles (see paragraphs 33-39) 3. If bridging principles do not apply, - For mixtures with classified ingredients: The summation method (see paragraph 44-50) reveals: - [Concentration of Acute 1] x M > 25% where M is a multiplying factor (see paragraph 56). - For mixtures with tested ingredients: The additivity formula (see paragraph 41-42) reveals: - L(E)C50  1mg/L - For mixtures with both classified and tested ingredients: The combined additivity formula and summation method (see paragraphs 41-50) reveal: - [Concentration of Acute 1] x M > 25% 4. For mixtures with no usable information for one or more relevant ingredients, classify using the available information and add the statement: "x percent of the mixture consists of component(s) of unknown hazards to the aquatic environment". | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Very toxic to aquatic life |

| 2 | 1. For Substances and Tested Mixtures: - 1mg/L < L(E)C50  10mg/L where L(E)C50 is either fish 96hr LC50, crustacea 48hr EC LC50 or aquatic plant 72 or 96hr ErC50 2. If data for a mixture are not available, use bridging principles (see paragraphs 33-39) 3. If bridging principles do not apply, - For mixtures with classified ingredients: The summation method (see paragraph 44-50) reveals: - [Concentration of Acute 1] x M x 10 + [Concentration of Acute 2] > 25% where M is a multiplying factor (see paragraph 56). - For mixtures with tested ingredients: The additivity formula (see paragraph 41-42) reveals: - 1mg/L < L(E)C50  10mg/L - For mixtures with both classified and tested ingredients: The combined additivity formula and summation method (see paragraphs 41-50) reveal: - [Concentration of Acute 1] x M x 10 + [Concentration of Acute 2] > 25% 4. For mixtures with no usable information for one or more relevant ingredients, classify using the available information and add the statement: "x percent of the mixture consists of component(s) of unknown hazards to the aquatic environment". | Symbol | No symbol used |
| --- | --- | --- | --- |
|  |  | Signal word | No signal word |
|  |  | Hazard statement | Toxic to aquatic life |
| 3 | 1. For Substances and Tested Mixtures: - 10mg/L < L(E)C50  100mg/L where L(E)C50 is either fish 96hr LC50, crustacea 48hr EC LC50 or aquatic plant 72 or 96hr ErC50 2. If data for a mixture are not available, use bridging principles (see paragraphs 33-39) 3. If bridging principles do not apply, - For mixtures with classified ingredients: The summation method (see paragraph 44-50) reveals: - [Concentration of Acute 1] x M x 100 + [Concentration of Acute 2] x 10 + [Concentration of Acute 3] > 25% where M is a multiplying factor (see paragraph 56). - For mixtures with tested ingredients: The additivity formula (see paragraph 41-42) reveals: - 10mg/L < L(E)C50  100mg/L - For mixtures with both classified and tested ingredients: The combined additivity formula and summation method (see paragraphs 41-50) reveal: - [Concentration of Acute 1] x M x 100 + [Concentration of Acute 2] x 10 + [Concentration of Acute 3] > 25% 4. For mixtures with no usable information for one or more relevant ingredients, classify using the available information and add the statement: "x percent of the mixture consists of component(s) of unknown hazards to the aquatic environment". | Symbol | No symbol used |
|  |  | Signal word | No signal word |
|  |  | Hazard statement | Harmful to aquatic life |

# Chronic hazards to the aquatic environment (See Chapter 3.10 for details)

| Hazard category | Criteria | ##### Hazard communication elements |  |
| --- | --- | --- | --- |
| 1 | 1. For Substances: - L(E)C50  1mg/L; and - Lack the potential to rapidly biodegrade and/or have the potential to bioaccumulate (BCF 500 or if absent log Kow  4). where L(E)C50 is either fish 96hr LC50, crustacea 48hr EC LC50 or aquatic plant 72 or 96hr ErC50 2. For Mixtures, use bridging principles (see paragraphs 33-39). 3. If bridging principles do not apply, - [Concentration of Chronic 1] x M > 25% where M is a multiplying factor (see paragraph 56). 4. For mixtures with no usable information for one or more relevant ingredients, classify using the available information and add the statement: "x percent of the mixture consists of component(s) of unknown hazards to the aquatic environment". | Symbol |  |
|  |  | Signal word | Warning |
|  |  | Hazard statement | Very toxic to aquatic life with long lasting effects |
| 2 | 1. For Substances: - 1 mg/L < L(E)C50  10 mg/L; and - Lack the potential to rapidly biodegrade and/or have the potential to bioaccumulate (BCF 500 or if absent log Kow  4); unless - Chronic NOECs > 1mg/L 2. For Mixtures, use bridging (see paragraphs 33-39). 3. If bridging principles do not apply, - [Concentration of Chronic 1] x M x 10 + [Concentration of Chronic 2] > 25% where M is a multiplying factor (see paragraph 56). 4. For mixtures with no usable information for one or more relevant ingredients, classify using the available information and add the statement: "x percent of the mixture consists of component(s) of unknown hazards to the aquatic environment". | Symbol |  |
|  |  | Signal word | No signal word |
|  |  | Hazard statement | Toxic to aquatic life with long lasting effects |
| 3 | 1. For Substances: - 10 mg/L < L(E)C50  100 mg/L; and - Lack the potential to rapidly biodegrade and/or have the potential to bioaccumulate (BCF 500 or if absent log Kow  4); unless - Chronic NOECs > 1mg/L 2. For Mixtures, use bridging principles (see paragraphs 33-39). 3. If bridging principles do not apply, - [Concentration of Chronic 1] x M x 100 + [Concentration of Chronic 2] x 10 + [Concentration of Chronic 3] > 25% where M is a multiplying factor (see paragraph 56). 4. For mixtures with no usable information for one or more relevant ingredients, classify using the available information and add the statement: "x percent of the mixture consists of component(s) of unknown hazards to the aquatic environment". | Symbol | No symbol used |
|  |  | Signal word | No signal word |
|  |  | Hazard statement | Harmful to aquatic life with long lasting effects |
| 4 | 1. For Substances: - poorly soluble and no acute toxicity is observed up the water solubility - Lack the potential to rapidly biodegrade and/or have the potential to bioaccumulate (BCF 500 or if absent log Kow  4); unless - Chronic NOECs > 1mg/L 2. For Mixtures, use bridging principles (see paragraphs 33-39). 3. If bridging principles do not apply, - Sum of concentrations of components classified as Chronic 1, 2, 3 or 4 > 25% 4. For mixtures with no usable information for one or more relevant ingredients, classify using the available information and add the statement: "x percent of the mixture consists of component(s) of unknown hazards to the aquatic environment". | Symbol | No symbol used |
|  |  | Signal word | No signal word |
|  |  | Hazard statement | May cause long lasting harmful effects to aquatic life |