| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL TRANS/SC.3/WP.3/2002/9 4 January 2002 ENGLISH Original: RUSSIAN |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on Inland Water Transport

Working Party on the Standardization of

Technical and Safety Requirements in

Inland Navigation

(Twenty-third session, 19-21 March 2002,

agenda item 4)

## Harmonization of the Requirements concerning anchors for inland navigation vessels

# Submitted by the Governments of the Russian Federation and Ukraine

Note:  At its twenty-first session, the Working Party took note of the comments by the delegation of Ukraine and the Chairman of the Working Party concerning the general concept for the anchor equipment of vessels other than self-propelled cargo vessels (TRANS/SC.3/WP.3/2001/10 and Add.1) and invited Governments and river commissions to state whether in their opinion, these documents could serve as a basis for future work on this question.  Replies received from the Governments of the Russian Federation and Ukraine are reproduced below by the secretariat.

GE.02-20009  (E)    110202    130202

## GENERAL CONCEPT FOR ANCHOR EQUIPMENT REQUIREMENTS FOR

## PASSENGER VESSELS, PUSHERS, SELF-PROPELLED PUSHER VESSELS

## AND PUSHED BARGES

RUSSIAN FEDERATION

1.	UN/ECE document TRANS/SC.3/WP.3/2001/10/Add.1 submitted by the Chairman of the Working Party, which sets out the general concept for the anchor requirements for various types of vessels, may serve as a basis for future work.  Experts from the Russian Federation have concluded, however, that formula (1) for the passenger vessels referred to in the above-mentioned document should be written without brackets; the use of brackets yields extremely high and quite unrealistic values.  The results of the calculations applying the formulae proposed by the Chairman (without brackets in the case of formula (1) for passenger vessels), as compared with the anchor equipment of vessels in the Russian Federation, are shown in tables 1-4 below.

2.	We propose that at the twenty-third session the Working Party should begin an article-by-article discussion of the document submitted by the Chairman.

UKRAINE

3.	Having reviewed the ideas in documents TRANS/SC.3/WP.3/2001/10 and Add.1 concerning the concept for the anchor requirements of vessels other than self-propelled cargo vessels, we are of the opinion that these documents could serve as a basis for future work in this field.

# Table 1

# Passenger vessels

| Name (type of vessel) | Displace- ment | Dimensions |  |  | Mean height above water-line | Number, type and weight of anchors calculated according to national regulations |  | Length of chain of bow/ stern anchors | Main region (zone) of navigation | C1** | Calculated aggregate mass of bow anchors |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D (t) | L (m) | B (m) | d* | H (m) | MB (kg) | MS (kg) | L (m) |  |  | M1 (kg) |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| Dmitry Furmanov | 3 850 | 129 | 16 | 2.85 | 13.8 | 2 x 1 575 with increased holding power | 1 x 855 with increased holding power | 175, 150/125 | Navigational zone 1 (“M” basin) | 65 | 3 660 |
| Oktyabrskaya revolyutsiya | 1 390 | 90.2 | 13.5 | 1.66 | 11 | 2 x 1 000, Hall | 1 x 500, Hall | 125, 100/75 | Navigational zone 2 (“O” basin) | 65 | 1 590 |
| Moskvich | 35.0 | 24.3 | 3.96 | 0.68 | 5.2 | 2 x 35 with increased holding power | - | 60, steel anchor cable | Navigational zone 3 (“R” basin) | 35 | 116 |

*  L - Length, B - Beam and d - Draught of vessels.

**  The empirical factor C1 is taken to be the same as for self-propelled cargo vessels, but it varies with full displacement rather

than carrying capacity and reflects the dimensions of the vessel.

# Table 2

# Pushers

| Name (type of vessel) | Power | Designed maximum carrying capacity of convoy pushed | Number, type and weight of stern anchors calculated according to national regulations | Length of chain of stern anchors | Main region (zone) of navigation | C2 | Calculated aggregate mass of bow anchors |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | P(кВт) | CC(t) | MS (kg) | L (m) |  |  | M2 (kg) |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Arkus | 810 | 5 000 | 2 x 675, Hall | 200, steel anchor cable | Navigational zone 2 (“O” basin) | 20 | 1 227 |
| OT | 1 765 | 15 000 | 2 x 1 250, Hall | 125 | Navigational zone 2 (“O” basin) | 30 | 2 340 |
| BTM | 220 | 1 000 | 1 x 125 with increased holding power | 120, steel anchor cable | Navigational zone 3 (“R” basin) | 30 | 237 |

# Table 3

# Pushed barges

| Name (type of vessel) | Dimensions |  |  | Carrying capacity of the convoy | Number, type and weight of bow anchors calculated according to national regulations* | Length of chain of bow anchors | Main region (zone) of navigation | C2 | Calculated aggregate mass of bow anchors** |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L (m) | B (m) | d (m) | CC (t) | MB (kg) | L (m) |  |  | M1 (kg) |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1681 | 85.7 | 16.5 | 2.55 | 2 x 2 500 | 2 x 1 000, Hall | 150 и 150 | Navigational zone 1 (“M” basin) | 30 | 1 740 |
| 81300 | 113 | 16.5 | 3.48 | 2 x 5 000 | 2 x 1 250, Hall | 100 и 100 | Navigational zone 2 (“O” basin) | 30 | 2 210 |
| P165 | 91.0 | 15.5 | 2.6 | 2 x 2 000 | 2 x 800, Hall | 102 и 77 | Navigational zone 3 (“R” basin) | 25 | 1 450 |

*  The anchor equipment of the head and centre pushed barges is identical in view of their potential inter-changeability.

**  The calculation has been performed as for head pushed barges in accordance with paragraphs 9 and 13 of document

TRANS/SC.3/WP.3/2001/10/Add.1.

# Table 4

# Self-propelled cargo pusher vessels

| Name (type of vessel) | Power of engine | Designed maximum carrying capacity of convoy pushed | Number, type and weight of anchors calculated according to national regulations |  | Length of chain of bow/stern anchors | Main region (zone) of navigation | C3 | Calculated aggregate mass of vessel’s bower anchors |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P(кВт) | CC(t) | MS (kg) | MS (kg) | 1 (m) |  |  | M2 (kg) |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Olenek | 2 x 331 | 2 000 | 1 x 1 500 1 x 1 250, Hall | 1 x 1 000, Hall | 175 и 175/75 | Navigational zone 1 (“M” basin) | 90 | 2 685 |
| Volzhsky | 2 x 880 | 11 400 | 2 x 1 750, Hall | 2 x 1 250, Hall | 155 и 155/75 | Navigational zone 2 (“O” basin) | 105 | 6 050 |
| Melkosidyashchy | 166.5 | 300 | 1 x 150 with increased holding power | 1 x 100 with increased holding power | 75/75, steel anchor cable | Navigational zone 3 (“R” basin) | 65 | 320 |

-----