UNITED

E

NATIONS

|  | Economic and Social Council | Distr. GENERAL TRANS/SC.3/WP.3/2002/21 18 April 2002 ENGLISH Original: ENGLISH, FRENCH AND RUSSIAN |
| --- | --- | --- |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on Inland Water Transport

Working Party on the Standardization of Technical and Safety Requirements in Inland Navigation

(Twenty-fourth session, 5-7 June 2002,

agenda item 3)

# AMENDMENT OF THE EUROPEAN CODE FOR INLAND WATERWAYS (cevnI)

Note:  The secretariat reproduces below the text of amendments to CEVNI as reflected in TRANS/SC.3/115/Rev.2, provisionally agreed by the Working Party at its twenty-second session (TRANS/SC.3/WP.3/44, paras. 4, 9, 10, 12, 14 and 15).

_________________

###### Chapter 1

1.	The text of article 1.02 (7) should read as follows:

“7.	In the case of a moored vessel or assembly of floating material having no boatmaster:

(a)	the operator and owner of such vessel or assembly,

(b)	the person responsible for keeping watch and surveillance under article 7.08,

shall be responsible for ensuring compliance with the provisions of these regulations.”

###### Chapter 3

- 2.	Modify the text of article 3.32 as follows:

“Article 3.32 - Prohibition of smoking or using an unprotected light or flame

1.	If other regulations prohibit

(a)	smoking,

(b)	using an unprotected light or flame,

on board, the prohibition shall be indicated by

circular white boards bordered with red, with a red diagonal and a picture of a cigarette emitting smoke.

The boards shall be placed on the vessel or on the gangway, as circumstances require.  By derogation from article 3.03, they shall be about 60 cm in diameter.

2.	By night, the boards shall be illuminated so as to be clearly visible on both sides of the vessel.”

3.	Paragraph 3 of article 3.10 should be supplemented with a footnote as follows:

“1	In the event that the competent authorities prescribe that the boatmaster of a pusher of a higher propelling power shall be the boatmaster of the convoy, this pusher shall carry the lights prescribed in paragraph 1 (c) (i) above.”

###### Chapter 6

4.	Modify the last sentence of article 6.09 as follows:

“This provision shall not apply to a small craft overtaking a vessel other than a small craft”.

5.	Add new paragraph 5 to article 6.21 as follows:

“5.	Passenger vessels with passengers on board shall not sail in side-by-side formation.”

Chapter 7

6.	Modify the text of paragraph 3 of article 7.08 as follows:

“3.	When the vessel has no boatmaster, the responsibility for setting up such a watch or surveillance shall lie with the operator and, in case the operator is not identifiable, with the owner.”

Annex 3

7.	Modify the sketch 66 as follows:

|  | 66 |  |
| --- | --- | --- |
| Article 3.32 Prohibited to smoke, or use an unprotected light or flame. |  |  |

- Add sketch 75 as follows:

|  | 75 |  |
| --- | --- | --- |
| Article 3.08 Paragraph 5: High-speed motorised vessel proceeding alone. |  |  |

_________________