Distr.

GENERAL

TRANS/WP.29/GRSG/2002/1

20 January 2002

ENGLISH

Original:  FRENCH

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on General Safety Provisions (GRSG)

(Eighty-second session, 29 April-3 May 2002,

agenda item 8.3)

## PROPOSED AMENDMENT TO THE NEW DRAFT REGULATION

## UNIFORM TECHNICAL PROVISIONS CONCERNING THE

## FORWARD FIELD OF VISION OF DRIVERS

# Transmitted by the expert from France

Note:	The text reproduced below was prepared by the expert from France, with a view to improving the definition of the A pillar and its specifications.  It is based on the text of a document distributed without a symbol (informal document No. 12) during the eighty-first session of GRSG (TRANS/WP.29/GRSG/60, para. 52).

Note:	This document is distributed to the Experts on General Safety Provisions only.

GE.02-20439  (E)    180302    190302

###### A.	PROPOSAL

Paragraph 2.18, read:

“2.18	‘A pillars’ means all areas of visual obstruction linking the upper part to the lower part of the daylight opening through 180° and located forward of the vertical transverse plane located 68 mm in front of the V points. These areas of visual obstruction may be caused, for example, by the roof supports, non-transparent items attached or contiguous to those supports, such as windscreen mouldings and door frames, or any continuous area with a light transmission coefficient of less than 70%.

There are two classes of pillars:

- principal A pillars:  these generate the largest angle of obstruction (measured as a

unit);

- secondary A pillars: these generate the smallest angle of obstruction (measured as

a unit).”

Add a new paragraph 5.1.2.2, to read:

“5.1.2.2	A vehicle may have a maximum of two principal A pillars and four secondary A pillars.  The sum of the angles of obstruction of each secondary pillar measured as a unit may not exceed the angle of obstruction of the principal A pillar.

In special cases in which the architecture of the vehicle does not permit of measurement as a unit (common part of the area of sections S1 and S2), it is permissible to make a theoretical measurement using a CAD construction which breaks down the pillar into two virtual pillars.  The pillar in question will then be counted as two pillars.”

Paragraph 5.1.4, read:

“… within the meaning of paragraph 5.1.3:

- obstructions … equal to 6°;

- radio aerials …”

*   *   *

###### B.	JUSTIFICATION

Document TRANS/WP.29/GRSG/2000/19, based on European Community Directive 77/649/EEC, does not define the A pillars permitted within the forward field of vision of drivers.

The purpose of this proposal is to incorporate a definition and requirements applicable when the vehicle has one or more A pillars.

-----