# Transmitted by the expert from the United States of America

# National Highway Traffic Safety Administration

### U.S. Department of Transportation

# Federal Motor Vehicle Safety Standard No. 202, Head Restraints

Summary of Jan. 4, 2001 Notice of Proposed Rulemaking

Reference:  http://dms.dot.gov, NHTSA Docket No. NHTSA-2000-8570

## Dimensional Requirements

- Front and Rear Outboard Seats.

- Height:

  - Front - Must achieve > 800 mm.

  - Front and Rear - No adjustment < 750 mm.  Consistent with ECE 17/25.

- Backset < 50 mm.

- Min. Width: Unchanged - Consistent with ECE for bucket seats and exceeds ECE for bench seats.

- Max. Gap: Consistent with ECE - 60 mm in HR.  25 mm between lowered adjustable HR and seat.  Limit consistent with ECE, but measurement method is not.

## Energy Absorption

- Head form strike at 24.2 km/h peak deceleration of 80g.  Consistent with ECE; however FMVSS No. 202 proposes a linear impactor.

## Strength and Lock Requirements

- Test height and backset locks at multiple adjustment positions.

- 102 mm deflection limit unchanged, but test procedure harmonized with ECE.

- 890 N force requirement altered such that seat back failure no longer acceptable.

## Dynamic Compliance Option - No similar requirement for ECE.

- Limit head-torso rotation < 12 degrees for a 50th male dummy in the front and rear, and < 20 degrees for a 95th male in the front.

- More realistic acceleration corridor.