PROPOSAL TO AMEND REGULATION No. 44 (2002/10)

# Transmitted by the Netherlands, Sweden, Germany and Consumers International

Insert a new paragraph 7.1.4.1.9., to read:

A child restraint with a support leg shall be tested as follows:  In the case of semi-universal category the tests shall be conducted with the support leg adjusted to both its maximum and minimum positions.  During the tests the support leg shall be supported by the trolley floor pan as described in Annex 6.  In the case of support legs out of the plane of symmetry, the worst case shall be selected by the Technical Service for the tests above.  In the case of specific vehicle category the support leg shall be adjusted as specified by the child restraint manufacturer.

Exchange paragraph 3.3.1.1. in Annex 6, to read:

The floor pan shall be rigidly mounted on the trolley.  The height of the floor pan relative to the CR-point, dimension X in figure 2, shall be adjusted to meet the requirements of paragraph 7.1.4.1.9.

Annex 6, Appendix 3, figure 2, amend to read:

Exchange floor pan height value 200 against X.

___________