### ECONOMIC COMMISSION FOR EUROPE				INF.15

INLAND TRANSPORT COMMITTEE

# Working Party on the Transport

of Dangerous Goods

(Seventy-fourth session, Geneva, 19-23 May 2003)

REPORT

of an ad-hoc ADR Expert Meeting on Dangerous Goods Groupings

for Road Tunnel Regulations

Feldkirch 12th-14th May 2003

Transmitted by the Government of Germany

###### Attendance

- The ad-hoc ADR expert meeting on dangerous goods groupings for road tunnel regulations held its session in Feldkirch (Austria) from 12 to 14 May 2003. Experts from the following countries and Organizations took part in its work: Austria, Finland, France, Germany; Netherlands; Norway, Portugal, Sweden, Switzerland, United Kingdom, CEFIC, FIATA and IEEC. The List of Participants is shown in Annex 1.

- Mr. Kafka from Austria acted as Chairman for the meeting.

###### MANDATE OF THE EXPERT MEETING

- The mandate given to the Expert Group by WP. 15 contained the following items:

- To define in greater depth the substances and types of load (packages, bulk, tanks) to be included in each group, taking into account the OECD/PIARC criteria;

- To introduce provisions explaining more clearly the meaning of the table;

- Possibly to introduce these provisions into other sections, e.g. Part 8, so as to facilitate their implementation by drivers;

- Possibly to provide the Working Party on Road Traffic Safety (WP.1) with a system of road signs and signals at the entrance to tunnels so as to enable the groups permitted in the tunnel to be identified;

- To provide a system to allow checks, e.g. by vehicle marking, documentation, or otherwise.

###### Working documents

- The deliberations were based on the ADR 2003, the OECD / PIARC study on Transport of Dangerous Goods through Road Tunnels, Document TRANS/WP.15/2002/21 from Austria as well as on written comments and proposals submitted in advance by Finland and  Norway, and during the meeting by the Netherlands and Switzerland.

###### General Debate

- The Chairman referred to the history and present status of the issue of Dangerous Goods Groupings for Road Tunnel Regulations addressing in particular Chapter 1.9 of the ADR, the OECD / PIARC study, the recommendations of the Ad hoc Multidisciplinary Group of Experts on Safety in Tunnels (AC 7) and the ongoing work on the EU Commission's proposal for a Directive on Minimum Safety Requirements for Tunnels in the Trans-European Road Network. He reviewed document TRANS/WP.15/2002/21and underlined its two main intentions namely to implement, in an updated form, the system of Dangerous Goods Groupings recommended in the OECD / PIARC study and to promote a minimum degree of harmonisation with regard to the types of dangerous goods subject to national tunnel regulations in the framework of Chapter 1.9 of ADR.

- The Expert from Finland in her document asked several general questions. As to the question about the relationship of the OECD/PIARC models [Quantitative Risk Assessment Model (QRAM) and Decision Support Model (DMS)] to the risk analysis and categories of tunnels (I-V) proposed in the EU directive, it was pointed out that there is no direct relationship of these Models to the categories (equipment classes) in Annex I of the draft directive. The Chairman also informed the meeting that it is not yet clear whether or not these provisions of the Annex I in the Commission Draft will  be replaced by those of an alternative Annex I proposed by Austria, France, Germany and Italy which does not link measures to equipment classes but specifies measures applicable for all tunnels and others applicable only to tunnels with certain characteristics.

- The consequence of the application of the OECD/PIARC models, which should be generally available shortly, would be a classification of tunnels only with regard to restrictions on the passage of transport units carrying dangerous goods. Therefore the answer to the second question posed by Finland, whether the risk analyses according to OECD and PIARC are recommended to be done before using the table proposed by Austria, is affirmative. The regulations in which it is stated which dangerous goods grouping (A-E) are allowed to be transported in different kind of tunnels  would always be the national regulations covered by Chapter 1.9 of the ADR. However these national regulations could in future be based on the dangerous goods groupings appearing in Chapter 1.9. Carriers of dangerous goods would then know from Chapter 1.9 which dangerous goods belong to certain dangerous goods groupings referred to in national tunnel regulations. An indication of the relevant grouping for a tunnel on a road sign and/or additional panel would give support to the implementation of such national regulations.

- The meeting noted the importance to carriers of ready availability of information on local transport conditions as required in section 1.9.3. It was hoped that the UN/ECE website could provide links to the sources of such information.

- The expert from Switzerland regretted that the groupings recommended did not cover the possibility to ban dangerous goods below the threshold of grouping E.

###### Definition of Substances and Types of Load to be included in each Group

- The meeting carefully examined the table from OECD/PIARC as developed in the Austrian proposal and finally agreed on a comprehensively revised version whilst maintaining the principles of the OECD/PIARC grouping system based on accident scenarios (see annex 2).

###### Introduction of Explanatory Provisions to the Table

- In the proposal from Austria, explanations to the table were given in two "NOTES" and the table was preceded by an introductory sentence. Both were revised (see annex 2).

###### Introduction of THE Provisions referring to the Table in other Parts e.g. Part 8

- There was agreement among the experts that chapter 1.9 was the right place for the table. It could be envisaged to add new provisions in Part 8 in order to make carriers and drivers more aware of the provisions in chapter 1.9. However in order to pursue such an approach written proposals to WP. 15 would be necessary.

###### Recommendation for WP.1 on Road Signs and Signals at the Entrance to Tunnels

- It was recalled that prohibition of entry for vehicles carrying dangerous can be expressed by signs C 3h, C 3m or C 3n covered by the Vienna Convention on Road Signs and Signals and the European Agreement supplementing that Convention. This signing is also mentioned in Annex III, 2.1 of the draft Directive on Minimum Safety Requirements for Tunnels in the Trans-European Road Network. A new sign and/or additional plate containing one of the capital letters A, B, C, D or E on an orange background could express the scope of the prohibition or restriction with regard to the dangerous goods concerned. Since the issue is connected to the ongoing work in Geneva (WP. 1) and in Brussels (tunnel safety directive) no explicit recommendation for a standardised additional plate was to be made at present.

###### Provisions in Support for Checks e.g. Vehicle Marking, Documentation or otherwise

- The majority of the experts could not imagine any practical provisions in this regard.

###### ADOPTION OF THE REPORT

- The Expert Meeting adopted the report and its annexes.

# Annex 1

List of Participants

# Annex 2

Draft text proposed by the expert – meeting for introduction into chapter 1.9.

Annex 1

# INTERNATIONAL ADR EXPERT - MEETING ON DANGEROUS GOODS GROUPINGS FOR ROAD TUNNEL REGULATIONS

Feldkirch (Vorarlberg) 12th to 14th May 2003

List of Participants

| FAMILY NAME | First Name(s) | Country / Organization | Position | Permanent official address | Telephone No | Fax No | E-mail address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BERG | Göran | FIATA | Chairman, FIATA-ABDG | Baumackerstr. 24, P.O.Box CH-8050 Zürich | + 46 31-703 81 14 | + 46 31-703 89 06 | goran.berg@Schenker.com |
| DÖRING | Ingo | Germany | Expert | Bundesanstalt für Material-forschung und -prüfung D-12200 Berlin | + 49 30 81043407 | + 49 30 81041226 | ingo.doering@bam.de |
| FRANCO | José Alberto | Portugal | Head of Department of the Directorate-General on Land Transport | Av. Forças Armadas, 40 P 1649-022 LISBOA | +351 21 794 90 19 | +351 21 797 37 77 | jafranco@dgtt.pt |
| GILABERT | David Manuel | Suisse | adjoint scientifique | Office fédéral des routes Worblentalstrasse 68, Ittigen CH-3003 Bern | + 41 3132342901 | +41 31 3234290 | david.gilabert@astra.admin.ch |
| HART | Jeffrey | United Kingdom | Head, TDG Policy Branch, DOT | Department for Transport, Zone 2/34, Great Minster House, 76 Marsham Street, LONDON SW1P 4DR | +44 20 79 44 27 58 | +44 20 79 44 20 39 | jeff.hart@dft.gsi.gov.uk |
| HUURDEMAN | Paul | Netherlands | policy advisor, Ministry of Transport | Nieuwe Uitleg, Postbus 20904 2500 Ex Den Haag | +31 70-351 1582 | + 31 70 - 351 1479 | paul.huurdeman@dgg.minvenw.nl |
| JOHANSEN | Arne | Norway | Senior Coordinating Officer for TDG, Directorate for Fire and Electrical Safety | POB. 2014, N-3103 Tonsberg Norway | + 47 333 98 840 | +47 333 10 660 | Arne.johansen@dbe.dep.no |
| KAFKA | Gustav | Austria | Head of unit, Federal Ministry for Transport, Innovation and Technology | Stubenring 1, A - 1010 Wien | +43 1 71162 5152 | +43 1 71162 15723 | gustav.kafka@bmvit.gv.at |

| FAMILY NAME | First Name(s) | Country / Organization | Position | Permanent official address | Telephone No | Fax No | E-mail address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| KORDEK | Marie-Astrid | France | INERIS | Ineris Parc Technologique ALATA 60550 Verneuil-en-Halatte, France | + 33 3 44 55 6139 | + 33 3 44 55 6510 | Marie-astrid.kordek@ineris.fr |
| LAUFHÜTTE | Klaus | Germany | Assistant Head of Division | Federal Ministry of Transport, Building and Housing D-53170 Bonn | + 49 228 300 2752 | + 49 228 300 3428 | klaus.laufhuette@bmvbw.bund.de |
| MCCULLOCH | Alexander | IEEC | Dangerous Goods Manager, International Express Carriers Conference | 9 Hollingworth Road, Orpington Kent BR 5 IAG United Kingdom | +44 208 325 6638 |  | alex.mcculloch@europe.ups.com |
| MIETTINEN | Seija | Finland | Director of Unit for Transport of Dangerous goods | Government, B.O.Box 31 Fin - 00023 | +358 9 6028563 | +358 9 6028597 | seija.Miettinen@mintc.fi |
| READER | Nigel | United Kingdom | TDG Policy Branch, DOT | Department for Transport, Zone 2/34, Great Minster House, 76 Marsham Street, LONDON SW1P 4DR | +44 20 79 44 27 72 | +44 20 79 44 20 39 | nigel.reader@dtlr.gsi.gov.uk |
| SIGRIST | Erwin | CEFIC | SGCI Chemie Pharma Schweiz, Leiter Fachbereich "Regelwerke für den Transport gefährlicher Güter" | Nordstraße 15, 8035 Zürich, CH | + 41 1 368 1742 | + 41 1 368 1743 | erwin.sigrist@sgci.ch |
| STRIDSBERG | Bengt | Sweden | Swedish Rescue Services Agency, Department for Risk Management and Environmental Impact | Swedish Rescues Service Agency, SE-651 80 Karlstad, Sweden | + 46 54 13 53 05 | + 46 54 13 5620 | bengt.stridsberg@srv.se |
| STRÖM | Katarina | Sweden | Swedish Rescue Services Agency, Department for Risk Management and Environmental Impact | Swedish Rescues Service Agency, SE-651 80 Karlstad, Sweden | + 46 54 13 53 41 | + 46 54 13 5620 | Katarina.strom@srv.se |

Annex 2

Draft text proposed by the expert meeting for introduction into chapter 1.9

Re – number existing section 1.9.4 as 1.9.5 and insert  a new section 1.9.4 as follows:

"1.9.4	Where additional provisions concerning carriage of dangerous goods through road tunnels, in accordance with 1.9.3 (a),  are applied, reference [shall] be made to the groupings of dangerous goods loadings in the table in 1.9.4.1.

      - <TABLE>

NOTE 1: The groupings in this table are based on accident scenarios modelled for the purposes of the joint OECD/PIARC Report on the Transport of Dangerous Goods through Road Tunnels (ITRD number: E 110101). A quantitative risk assessment model and decision support model are proposed in the Report to determine the groupings.

NOTE 2: The table reads across with the least restrictive grouping A to the most restrictive grouping E.

1.9.4.2	Where a transport unit carries dangerous goods of more than one class, the most restrictive grouping shall apply to the whole load. When a transport unit carries dangerous goods below the transport category thresholds of sub-section 1.1.3.6, the most restrictive grouping shall apply to the whole load after application of the provisions of 1.1.3.6.4 and 1.1.3.6.5. Empty uncleaned tanks / tank containers shall be treated as if full or part full.”

| Class | Groupings (see NOTES 1 and 2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E |
| 1 | Unrestricted | Restricted for - Divisions 1.1, 1.2 and 1.5 above [1000] kg maximum permissible net mass of explosives and - explosives belonging to compatibility groups A, K and L, above ADR subsection 1.1.3.6 threshold | Restricted for - Divisions 1.1, 1.2 and 1.5 above ADR subsection 1.1.3.6 threshold - division 1.3 above [5000] kg maximum permissible net mass of explosives and - explosives belonging to compatibility groups A, H, J, K and L above ADR subsection 1.1.3.6 threshold | Restricted for - Divisions 1.1, 1.2, 1.3 and 1.5 above ADR subsection 1.1.3.6 threshold - explosives belonging to compatibility groups A, H, J, K and L above ADR subsection 1.1.3.6 threshold | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 2 | Unrestricted | Restricted for - flammable gases (classification codes F, TF and TFC) in tanks | Restricted for - loadings in grouping B and - toxic gases (classification codes T, TC, TO and TOC) in tanks | Restricted for - loadings in grouping C and - flammable and toxic gases (classificatrion codes F, FC, T, TC, TF, TO, TFC and TOC) in packages above ADR subsection 1.1.3.6 threshold | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 3 | Unrestricted | Restricted for - classification code D [UN numbers XXXX] above ADR subsection 1.1.3.6 threshold | Restricted for - loadings in grouping B - PG I of classification code FC and FTC in tanks | Restricted for - loadings in grouping C and - PG I and II in tanks and - classification code F2 in tanks | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 4.1 | Unrestricted | Restricted for - classification code D, DT [UN numbers XXXX] above ADR subsection 1.1.3.6 threshold - self-reactive substances Type B [UN 3221, 3222, 3231 and 3232] above ADR subsection 1.1.3.6 threshold | Restricted for - loadings in grouping B | Restricted for - loadings in grouping C and - self-reactive substances of Type C to F above ADR subsection 1.1.3.6 threshold - [related substances to self-reactive substances above ADR subsection 1.1.3.6 threshold] | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 4.2 | Unrestricted | Restricted for - PG I in tanks | Restricted for - loadings in grouping B | Restricted for - loadings in grouping C - [PG II in bulk/tanks] | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 4.3 | Unrestricted | Restricted for - PG I in tanks | Restricted for - loadings in grouping B | Restricted for - loadings in grouping C and - [PG II in bulk/tanks] | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 5.1 | Unrestricted | Restricted for - PG I in tanks | Restricted for - loadings in grouping B | Restricted for - loadings in grouping C | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 5.2 | Unrestricted | Restricted for - Type B [UN 3101, 3102, 3111 and 3112] above ADR subsection 1.1.3.6 threshold | Restricted for - loadings in grouping B | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 6.1 | Unrestricted | Unrestricted | Restricted for - PG I of classification codes TF1 and TFC in tanks | Restricted for - loadings in grouping C - PG I of classification codes TF2 and TW1 in tanks - PG I of classification codes TF1 and TFC in packages above ADR subsection 1.1.3.6 threshold - PG II of classification codes TF1, TF2, TFC and TW1 in tanks | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 6.2 | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Restricted for - UN 2814 and 2900 above ADR subsection 1.1.3.6 threshold. |
| 7 | Unrestricted | Unrestricted | Restricted for - UN 2977 and 2978 | Restricted for - loadings in grouping C | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| 8 | Unrestricted | Unrestricted | Restricted for - PG I of classification code CT1 in tanks | Restricted for - loadings in grouping C and - PG I of classification codes CF1, CFT and CW1 in tanks - PG I of classification code CT1 in packages above ADR subsection 1.1.3.6 threshold | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |

| 9 | Unrestricted | Unrestricted | Unrestricted | Restricted for - classification code M2 (UN 2315, 3151 and 3152) in tanks - classification code M3 (UN 2211 and 3314) in bulk/tanks - classification code M10 (UN 3258) | Restricted for - goods of this class above ADR subsection 1.1.3.6 threshold |
| --- | --- | --- | --- | --- | --- |

__________