UNITED

E

NATIONS

Economic and Social

Council

Distr.

GENERAL

- TRANS/WP.29/2003/15

###### 17 December 2002

ENGLISH

- Original: ENGLISH AND FRENCH

### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-twenty-ninth session,

11-14 March 2003, agenda item 4.2.13.)

# PROPOSAL FOR DRAFT SUPPLEMENT 4 TO REGULATION No. 107

(Double-deck large passenger vehicles)

Transmitted by the Working Party on General Safety Provisions (GRSG)

Note:	The text reproduced below was adopted by GRRF at its eighty-third session, and is transmitted for consideration to WP.29 and to AC.1.  It is based on document TRANS/WP.29/GRSG/2002/7, not amended (TRANS/WP.29/GRSG/62, para. 12).

____________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Paragraph 5.7.9.6.2.2., amend to read:

"5.7.9.6.2.2.	Intrusion of a structural member provided that the intrusion is included within a triangle whose peak is situated 70 cm from the top and whose base is 10 cm in width and situated in the upper part of the space in question, adjacent to the side wall of the vehicle (see annex 3, figure 10);"

Annex 3, Figure 10, amend to read:

"Figure 10

PERMITTED INTRUSION OF A STRUCTURAL MEMBER

(see paragraph 5.7.9.6.2.2.)

|  | I (cm) min. |
| --- | --- |
| Class I | 40-50 (for classes I and II min 35 cm at wheel arches and engine compartment) |
| Class II |  |
| Class III |  |

"

__________