UNITED

- NATIONS

Distr.

Economic and Social	GENERAL

Council					TRANS/WP.29/2003/33

4 April 2003

ENGLISH

- Original:  ENGLISH and FRENCH

# ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirtieth session,

24-27 June 2003, agenda item 4.2.25.)

PROPOSAL FOR DRAFT SUPPLEMENT 2 TO REGULATION No. 99

(Gas-discharge light sources)

Transmitted by the Working Party on Lighting and Light-Signalling (GRE)

Note :	The text reproduced below was adopted by GRE at its forty-ninth session, and is transmitted for consideration to WP.29 and AC.1.  It is based on document TRANS/WP.29/GRE/2002/40, as amended (TRANS/WP.29/GRE/49, paras. 120 and 121).

__________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

List of contents,

ANNEXES, amend the list, to read:

"Annex 1	Sheets for gas-discharge light sources

Annex 2	Communication concerning .……

………….."

Text of the Regulation,

Annex 1 (former), replace by new annex 1 (see next pages)

Annex 4, paragraph 10, amend to read:

″...

The following figure shows the colour tolerance area for colour white and the restricted tolerance area for the gas-discharge light sources D1R, D1S, D2R, D2S, D3R, D3S, D4R and D4S.”

Annex 5 (former), replace by new annex 5 (see the last page)

Annex 1, amend to read:

"Annex 1

SHEETS FOR GAS-DISCHARGE LIGHT SOURCES

List of categories of gas-discharge light sources and their sheet numbers:

| Light source category |  | Sheet numbers |
| --- | --- | --- |
| D1R |  | DxR/1 to 7 |
| D1S |  | DxS/1 to 6 |
| D2R |  | DxR/1 to 7 |
| D2S |  | DxS/1 to 6 |
| D3R |  | DxR/1 to 7 |
| D3S |  | DxS/1 to 6 |
| D4R |  | DxR/1 to 7 |
| D4S |  | DxS/1 to 6 |

List of sheets for gas-discharge light sources and their sequence in this annex:

| #### Sheet numbers |  | Applicable for light source categories |
| --- | --- | --- |
| DxR/1 to 7 |  | D1R, D2R, D3R and D4R |
| Dx/1 to 6 |  | D1S, D2S, D3S and D4S |

CATEGORIES D1R, D2R, D3R AND D4R	Sheet DxR/1

The drawings are intended only to illustrate the essential dimensions (in mm)

of the gas-discharge light source

Figure 1 - Category D1R - Type with cables - Cap PK32d-3

Figure 2 - Category D2R - Type with connector - Cap P32d-3

1/	The reference plane is defined by the positions on the surface of the holder on which the three supporting bosses of the cap ring will rest.

2/	See sheet DxR/3.

3/	With respect to the reference axis, when measured at a distance of 27.1 mm from the reference plane the eccentricity of the outer bulb shall be less than  0.5 mm in direction B and less than + 1 mm /- 0.5 mm in direction A.

CATEGORIES D1R, D2R, D3R AND D4R	Sheet DxR/2

The drawings are intended only to illustrate the essential dimensions (in mm)

of the gas-discharge light source

Figure 3 – Category D3R - Type with starter – Cap PK32d-6

Figure 4 - Category D4R - Type with connector – Cap P32d-6

1/	The reference plane is defined by the positions on the surface of the holder on which the three supporting bosses of the cap ring will rest.

2/	See sheet DxR/3.

3/	With respect to the reference axis, when measured at a distance of 27.1 mm from the reference plane the eccentricity of the outer bulb shall be less than  0.5 mm in direction B and less than + 1 mm /- 0.5 mm in direction A.

CATEGORIES D1R, D2R, D3R AND D4R	Sheet DxR/3

Figure 5

Definition of reference axis 1/

The cap shall be pushed in this direction

Figure 6

Maximum lamp outline 2/

1/	The reference axis is perpendicular to the reference plane and crosses the intersection of the two parallel lines as indicated in figure 5.

2/	Glass bulb and supports shall not exceed the envelope, as indicated in figure 6. The envelope is concentric with the reference axis.

CATEGORIES D1R, D2R, D3R AND D4R	Sheet DxR/4

| Dimensions |  |  |  | Production light sources |  |  | Standard light sources |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Position of electrodes |  |  |  | Sheet DxR/5 |  |  |  |  |
| Position and form of the arc |  |  |  | Sheet DxR/6 |  |  |  |  |
| Position of the black stripes |  |  |  | Sheet DxR/7 |  |  |  |  |
| 1 1/ |  |  |  | 45°  5° |  |  |  |  |
| 2 1/ |  |  |  | 45° min. |  |  |  |  |
| D1R: Cap PK32d-3 D2R: Cap P32d-3 D3R: Cap PK32d-6 D4R: Cap P32d-6 |  | in accordance with IEC Publication 60061 (sheet 7004-111-2) |  |  |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARCTERISTICS |  |  |  |  |  |  |  |  |
|  |  |  |  | D1R/D2R | D3R/D4R | D1R/D2R |  | D3R/D4R |
| Rated voltage of the ballast |  |  | V | 12 2/ |  | 12 |  |  |
| Rated wattage |  |  | W | 35 |  | 35 |  |  |
| Test voltage |  |  | V | 13.5 |  | 13.5 |  |  |
| Lamp voltage | Objective |  | V | 85 | 42 | 85 |  | 42 |
|  | Tolerance |  |  |  17 |  9 |  8 |  |  4 |
| Lamp wattage | Objective |  | W | 35 |  | 35 |  |  |
|  | Tolerance |  |  |  3 |  |  0.5 |  |  |
| Luminous flux | Objective |  | lm | 2800 |  | 2800 |  |  |
|  | Tolerance |  |  |  450 |  |  150 |  |  |
| Colour co-ordinates | Objective |  |  | x = 0.375 |  |  | y = 0.375 |  |
|  | Tolerance area 3/ |  |  | x ≥ 0.345 y ≤ 0.150 + 0.640 x x ≤ 0.405 y ≥ 0.050 + 0.750 x |  |  |  |  |
| Hot-restrike switch-off time |  |  | s | 10 |  |  | 10 |  |

1/	The part of the bulb within the angles 1 and 2 shall be the light emitting part. This part shall be as homogeneous in form as possible and shall be optically distortion free. This applies to the whole bulb circumference within the angles 1 and 2 except for the black stripes.

2/	Application voltages of ballasts may differ from 12 V.

3/	See annex 4.

CATEGORIES D1R, D2R, D3R AND D4R	Sheet DxR/5

Position of the electrodes

This test is used to determine whether the electrodes are correctly positioned relative to the reference axis and the reference plane.

Measuring direction: light source side and top view

| Dimension in mm | Production light sources | Standard light sources |
| --- | --- | --- |
| a1 | d + 0.5 | d + 0.2 |
| a2 | d + 0.7 | d + 0.35 |
| b1 | 0.4 | 0.15 |
| b2 | 0.8 | 0.3 |
| c | 4.2 | 4.2 |

d = diameter of the electrode;

d < 0.3 for D1R and D2R;

d < 0.4 for D3R and D4R.

The top of the electrode nearest to the reference plane shall be positioned in the area defined by a1 and b1. The top of the electrode furthest from the reference plane shall be positioned in the area defined by a2 and b2.

CATEGORIES D1R, D2R, D3R AND D4R	Sheet DxR/6

Position and form of the arc

Measuring direction C as defined on sheet DxR/7

The form of the arc is for illustration purpose only.

This test is used to determine the form and sharpness of the arc and its position relative to the reference axis and plane by measuring its bending and diffusion in the central cross section D and by measuring stray light intensities in Zone A and at lines B and C.

When measuring the relative luminance distribution in the central cross section D as indicated in the drawing above, the maximum value Lmax has the distance r from the reference axis.  The points of 20% of Lmax have the distance s, as shown in the drawing below.

| Dimension in mm | Production light sources | Standard light sources |
| --- | --- | --- |
| r | 0.50  0.25 | 0.50  0.20 |
| s | 1.10  0.25 | 1.10  0.25 |

Relative luminance distribution

in the central cross section D.

Determination of:

arc bending r

arc diffusion s

Luminance Lmax

When measuring the luminances from measuring direction B as defined on sheet DxR/7 with a set-up as outlined in annex 5, however with a circular field of 0.2M mm diameter, the relative luminance expressed as a percentage of L max (at cross section D) shall be:

| Zone A | ≤ 4.5 % | Line B | ≤ 15 % | Line C | ≤ 5.0 % |
| --- | --- | --- | --- | --- | --- |

The area of zone A is defined by the black coating, the outer bulb and a plane at 24.5 mm from the reference plane.

CATEGORIES D1R, D2R, D3R AND D4R	Sheet DxR/7

Position of black stripes

This test is used to determine whether the black stripes are correctly positioned relative to the reference axis and the reference plane.

When measuring the luminance distribution of the arc in the central cross section as defined on sheet DxR/6, after having turned the light source so that the black stripe is covering the arc, the measured luminance shall be ≤ 0.5 % of Lmax.

In the area defined by 1 and 3 the black coating may be replaced by any other means which prevents light transmission through the specified area.

| Dimensions | Production light sources | Standard light sources |
| --- | --- | --- |
| 1 | 45°  5° |  |
| 3 | 70° min. |  |
| 4 | 65° min. |  |
| 1/24, 1/30, 2/24, 2/30 | 25°  5° |  |
| f1/24, f2/24 1/ | 0.15  0.25 | 0.15  0.20 |
| f1/30 1/ | f1/24 mv  0.15 2/ | f1/24 mv  0.1 |
| f2/30 1/ | f2/24 mv  0.15 2/ | f2/24 mv  0.1 |
| f1/24 mv - f2/24 mv |  0.3 max. |  0.2 max. |
| d | 9  1 |  |

1/ "f1/.." means dimension f1 to be measured at the distance from the reference plane indicated in mm after the stroke.

2/ "  /24 mv" means the value measured at a distance of 24 mm from the reference plane.

CATEGORIES D1S, D2S, D3S AND D4S	Sheet DxS/1

The drawings are intended only to illustrate the essential dimensions (in mm)

of the gas-discharge light source

Figure 1 - Category D1S - Type with cables - Cap PK32d-2

Figure 2 - Category D2S - Type with connector - Cap P32d-2

1/	The reference plane is defined by the positions on the surface of the holder on which the three supporting bosses of the cap ring will rest.

2/	See sheet DxS/3.

3/	When measured at a distance of 27.1 mm from the reference plane and with respect to the mid-point of the inner bulb, the outer bulb shall have an eccentricity of 1 mm max.

CATEGORIES D1S, D2S, D3S AND D4S	Sheet DxS/2

The drawings are intended only to illustrate the essential dimensions (in mm)

of the gas-discharge light source

Figure 3 - Category D3S - Type with starter - Cap PK32d-5

Figure 4 - Category D4S - Type with connector - Cap P32d-5

1/	The reference plane is defined by the positions on the surface of the holder on which the three supporting bosses of the cap ring will rest.

2/	See sheet DxS/3.

3/	When measured at a distance of 27.1 mm from the reference plane and with respect to the mid-point of the inner bulb, the outer bulb shall have an eccentricity of 1 mm max.

CATEGORIES D1S, D2S, D3S AND D4S	Sheet DxS/3

Figure 5

Definition of reference axis 1/

The cap shall be pushed in this direction

Figure 6

Maximum lamp outline 2/

1/	The reference axis is perpendicular to the reference plane and crosses the intersection of the two parallel lines as indicated in figure 5.

2/	Glass bulb and supports shall not exceed the envelope, as indicated in figure 6. The envelope is concentric with the reference axis.

CATEGORIES D1S, D2S, D3S AND D4S	Sheet DxS/4

| Dimensions |  |  |  | Production light sources |  |  | Standard light sources |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Position of electrodes |  |  |  | Sheet DxS/5 |  |  |  |  |
| Position and form of the arc |  |  |  | Sheet DxS/6 |  |  |  |  |
| 1, 2 1/ |  |  |  | 55° min. |  |  | 55° min. |  |
| D1S: Cap PK32d-2 D2S: Cap P32d-2 D3S: Cap PK32d-5 D4S: Cap P32d-5 |  | in accordance with IEC Publication 60061 (sheet 7004-111-2) |  |  |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARCTERISTICS |  |  |  |  |  |  |  |  |
|  |  |  |  | D1S/D2S | D3S/D4S | D1S/D2S |  | D3S/D4S |
| Rated voltage of the ballast |  |  | V | 12 2/ |  | 12 |  |  |
| Rated wattage |  |  | W | 35 |  | 35 |  |  |
| Test voltage |  |  | V | 13.5 |  | 13.5 |  |  |
| Lamp voltage | Objective |  | V | 85 | 42 | 85 |  | 42 |
|  | Tolerance |  |  |  17 |  9 |  8 |  |  4 |
| Lamp wattage | Objective |  | W | 35 |  | 35 |  |  |
|  | Tolerance |  |  |  3 |  |  0.5 |  |  |
| Luminous flux | Objective |  | lm | 3200 |  | 3200 |  |  |
|  | Tolerance |  |  |  450 |  |  150 |  |  |
| Colour co-ordinates | Objective |  |  | x = 0.375 |  |  | y = 0.375 |  |
|  | Tolerance area 3/ |  |  | x ≥ 0.345 y ≤ 0.150 + 0.640 x x ≤ 0.405 y ≥ 0.050 + 0.750 x |  |  |  |  |
| Hot-restrike switch-off time |  |  | s | 10 |  |  | 10 |  |

1/	The part of the bulb within the angles 1 and 2 shall be the light emitting part. This part shall be as homogeneous in form as possible and shall be optically distortion free. This applies to the whole bulb circumference within the angles 1 and 2.

2/	Application voltages of ballasts may differ from 12 V.

3/	See annex 4.

CATEGORIES D1S, D2S, D3S AND D4S	Sheet DxS/5

Position of the electrodes

This test is used to determine whether the electrodes are correctly positioned relative to the reference axis and the reference plane.

Measuring direction:  light source side and top view

| Dimension in mm | Production light sources | Standard light sources |
| --- | --- | --- |
| a1 | d + 0.2 | d + 0.1 |
| a2 | d + 0.5 | d + 0.25 |
| b1 | 0.3 | 0.15 |
| b2 | 0.6 | 0.3 |
| c | 4.2 | 4.2 |

d = diameter of the electrode;

d < 0.3 for D1S and D2S;

d < 0.4 for D3S and D4S.

The top of the electrode nearest to the reference plane shall be positioned in the area defined by a1 and b1. The top of the electrode furthest from the reference plane shall be positioned in the area defined by a2 and b2.

CATEGORIES D1S, D2S, D3S AND D4S	Sheet DxS/6

Position and form of the arc

This test is used to determine the form of the arc and its position relative to the reference axis and the reference plane by measuring its bending and diffusion in the cross section at a distance 27.1 mm from the reference plane.

The form of the arc is for illustration purposes only.

Measuring direction: light source side view

When measuring the relative luminance distribution in the central cross section as indicated in the drawing above, the maximum value shall be located within the distance r from the reference axis. The point of 20% of the maximum value shall be within s.

| Dimension in mm | Production light sources | Standard light sources |
| --- | --- | --- |
| r | 0.50  0.40 | 0.50  0.20 |
| s | 1.10  0.40 | 1.10  0.25 |

Relative luminance distribution

in the central cross section.

Determination of:

arc bending  r

arc diffusion  s

__________

Annex 5, amend to read:

″Annex 5

Optical setup for the measurement of the position and form
of the arc and of the position of the electrodes 1/

The gas-discharge light source shall be positioned as shown:

- in figure 1 or figure 2 on sheet DxR/1 or sheet DxS/1;

- in figure 3 or figure 4 on sheet DxR/2 or sheet DxS/2.

An optical system shall project a real image A' of the arc A with a magnification of preferably  M = s'/s = 20 on a screen.  The optical system shall be aplanatic and achromatic.  In the focus-length f of the optical system a diaphragm d shall cause a projection of the arc with nearly parallel observation directions.  To get the angle of the half divergence not larger than  = 0.5°, the diameter of the focus-diaphragm with respect to the focus-length of the optical system shall be not more than d = 2f tan().  The active diameter of the optical system shall be not more than:

D = (1 + 1/M)d + c + (b1 + b2)/2. (c, b1 and b2 are given on sheet DxS/5, respectively sheet DxR/5).

A scale on the screen shall enable to measure the position of the electrodes.  The calibration of the arrangement advantageously can be done by using a separate projector with a parallel beam in connection with a gauge whose shadow is projected to the screen.  The gauge shall show the reference axis and the plane parallel to the reference plane and at distance "e" mm from it (e = 27.1 for D1R, D1S, D2R, D2S, D3R, D3S, D4R and D4S).

In the plane of the screen a receiver has to be mounted movable in a vertical direction on a line corresponding to the plane at "e" from the reference plane of the gas discharge light source.

The receiver shall have the relative spectral sensitivity of the human eye.

The size of the receiver shall be not more than 0.2 M mm in the horizontal and not more than 0.025 M mm in the vertical direction (M = the magnification).  The range of measurable movement shall be such that the required measures of the arc bending r and arc diffusion s can be measured.

__________

1/	This method is an example of a measurement method; any method with equivalent measurement accuracy may be used.

_________"