UNITED

- NATIONS

Distr.

Economic and Social	GENERAL

Council					TRANS/WP.29/2003/40

4 April 2003

ENGLISH

- Original:  ENGLISH and FRENCH

# ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirtieth session,

24-27 June 2003, agenda item 4.2.19.)

DRAFT CORRIGENDUM 2 TO THE 05 SERIES

OF AMENDMENTS TO REGULATION No. 83

(Emissions of M1 and N1 categories of vehicles)

Transmitted by the Working Party on Pollution and Energy (GRPE)

Note :	The text reproduced below was adopted by GRPE at its forty-fifth session, and is transmitted for consideration to WP.29 and AC.1.  It is based on the text of document TRANS/WP.29/GRPE/2003/7,  not amended (TRANS/WP.29/GRPE/45, para. 28).

__________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Paragraph 11.1.3.2., correct to read (English and Russian only):

"...... meets the requirements of this Regulation as required by paragraph 11.1.2.2. above."

Paragraph 11.1.4.1., correct to read:

"......

shall be considered, for the purposes of paragraphs 11.1.3.1. and 11.1.3.2. as vehicles in category N1."

Annex 4, Appendix 3,

Paragraph 5.1.1.2.8., correct to read:

"......

RT	=	total driving resistance = RR + RAERO

KR	=	temperature correction factor of rolling resistance, taken to be equal to: 8.64 . 10-3/°C, or the manufacturer’s correction factor that is approved by the authority

t	=	road test ambient temperature in °C

......

where:

M =	vehicle mass in kg,

and for each speed the coefficients a and b are shown in the following table:

| V (km/h) | a | b |
| --- | --- | --- |
| 20 40 60 80 100 120 | 7.24 . 10-5 1.59 . 10-4 1.96 . 10-4 1.85 . 10-4 1.63 . 10-4 1.57 . 10-4 | 0.82 0.54 0.33 0.23 0.18 0.14 |

"

__________