| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/2003/52 11 April 2003 Original: ENGLISH |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirtieth session, 24-27 June 2003,
 agenda item 4.2.12.)

PROPOSAL FOR DRAFT SUPPLEMENT 7
TO THE 02 SERIES OF AMENDMENTS TO REGULATION No. 48

(Installation of Lighting and light-signalling devices)

Transmitted by the Working Party on Lighting and Light-Signalling (GRE)

Note:	The text reproduced below was adopted by GRE at its fiftieth session, and is transmitted for consideration to WP.29 and to AC.1.  It is based on the text of a document distributed during the session without a symbol (informal document No. 12 bis.).  It complements the proposal contained in document TRANS/WP.29/2003/25/Rev.1 (TRANS/WP.29/GRE/50, paras. [   ]).

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Insert a new paragraph 5.23., amend to read:

"5.23.	Lamps shall be fitted in a vehicle in such a way that the light source can be correctly replaced according to the instructions of the vehicle manufacturer without the use of special tools, other than those provided with the vehicle by the manufacturer. This requirement is not applicable to:

(a)	devices approved with a non-replaceable light source;

(b)	devices approved with light sources according to Regulation No. 99."

Paragraph 5.23. (former), renumber as paragraph 5.24.

Paragraph 12.11., should be deleted.

Paragraph 12.12., renumber as paragraph 12.11.

Inserrt new paragraphs 12.12. to 12.15., to read:

"12.12.	As from the date of entry into force of Supplement 7 to the 02 series of amendments, no Contracting Party applying this Regulation shall refuse to grant approvals under this Regulation as amended by Supplement 7 to the 02 series of amendments.

12.13.	As from 30 months after the date of entry into force of Supplement 7 to the 02 series of amendments, Contracting Parties applying this Regulation shall grant ECE approvals only if the vehicle type to be approved meets the requirements of this Regulation as amended by Supplement 7 to the 02 series of amendments.

12.14.	Contracting Parties applying this Regulation shall not refuse to grant extensions of approvals to the preceding series of amendments to this Regulation, including Supplement 6 to the 02 series of amendments.

12.15.	ECE approvals granted under this Regulation before the date mentioned in paragraph 12.13. above, including extensions of such approvals, shall remain valid indefinitely."

_________