UNITED

E

NATIONS

Economic and Social

Council

Distr.

GENERAL

- TRANS/WP.29/2003/94

###### 31 July 2003

- Original: ENGLISH

### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirty-first session,

11-14 November 2003, agenda item 4.2.32.)

# DRAFT CORRIGENDUM 1 TO SUPPLEMENT 6 TO REGULATION No. 91

(Side-marker lamps)

Transmitted by the Working Party on Lighting and Light-Signalling (GRE)

Note:	The text reproduced below was adopted by GRE at its fiftieth session, and is transmitted for consideration to WP.29 and to AC.1.  It is based on documents TRANS/WP.29/GRE/2003/13, as amended (TRANS/WP.29/GRE/50, para. 89 and annex 3).

____________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Paragraph 2.3., amend to read:

"2.3.	"Side-marker lamps of different types" means lamps which do not differ in such essential respects as:

-	the trade name or mark;

-	the characteristics of the optical system (level of intensity, light distribution angles, category of filament lamp, light source module, etc.).

A change of the colour of the filament lamp or the colour of any filter does not constitute a change of type."

Paragraph 3.2.2., amend to read:

"3.2.2.	a brief technical description stating, in particular, with the exception of lamps with non-replaceable light sources:

-	the category or categories of filament lamp(s) prescribed; this filament lamp category shall be one of those contained in Regulation No. 37; and/or

-	the light source module specific identification code."

Paragraph 4.3., amend to read:

"4.3.	with the exception of lamps with non-replaceable light sources must bear a clearly legible and indelible marking indicating:

-	the category or categories of filament lamp(s) prescribed; and/or

-	the light source module specific identification code."

Paragraph 4.5., amend to read:

"4.5.	In the case of lamps with non-replaceable light sources or light source module(s), must bear the marking of the rated voltage and rated wattage."

Add new paragraphs 4.6., 4.6.1., 4.6.2. and 4.6.3., to read:

"4.6.	Light source module(s) submitted along with the approval of lamp must bear:

4.6.1.	the trade name or mark of the applicant; this marking must be clearly legible and indelible;

4.6.2.	the specific identification code of the module; this marking must be clearly legible and indelible.

This specific identification code shall comprise the starting letters "MD" for "MODULE" followed by the approval marking without the circle as prescribed in paragraph 5.4.1.1. below; this specific identification code shall be shown in the drawings mentioned in paragraph 3.2.1. above.  The approval marking does not have

to be the same as the one on the lamp in which the module is used, but both markings shall be from the same applicant.

4.6.3.	the marking of the rated voltage (and rated wattage)."

Add new paragraphs 6.3., 6.3.1. and 6.3.2., to read:

"6.3.	Light source module

6.3.1.	The design of the light source module(s) shall be such that even in darkness the light source module(s) can be fitted in no other position, but the correct one.

6.3.2.	The light source module(s) shall be tamperproof."

Annex 2, item 9., amend to read:

"9.	Concise description:  3/

Colour of light emitted: amber / red  2/

Number and category(ies) of filament lamp(s):

Light source module:	yes/no  2/

Light source module specific identification code:

Geometrical conditions of installation and relating variations,

if any: 	 "

Annex 3, insert a new example 4, to read:

"Example 4

Light source modules

### MD E3 17325

The light source module bearing the identification code shown above has been approved together with a lamp approved in Italy (E3) under approval number 17325."

________________