PROPOSAL FROM THE GTB-AFS GROUP

Ref: Informal Document No.1 (Draft Reg. on AFS), "Cut-Off" Evaluation

Annex 9

Amend paragraph 3.2.1. to read

3.2.1.	No double lines allowed:

In case a "cut-off" serves, according to the Applicants specification, for aiming purposes, no secondary "cut-off line" (double line) shall be produced from the same lighting unit.

A double line shall be deemed not being detrimental if its sharpness factor G, as defined in the paragraph 3.2.2. below, is less than [0.10].

Justification

Adaptive frontlighting systems must produce at least two cut-off lines for the passing beam aiming on either side. Other cut-off lines may be present, intended for aiming, too and/or serving for compliance with the photometric requirements, only. Forbiddance of a secondary cut-off line should be specified therefore different than for a conventional headlamp.