| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. RESTRICTED TRANS/WP.29/GRE/2003/9 27 January 2003 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

(Fiftieth session, 7-11 April 2003, agenda item 2.2.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 37

(Filament lamps)

Transmitted by the expert from the Working Party "Brussels 1952" (GTB)

Note:	The text reproduced below was prepared by the expert from GTB in order to insert into the Regulation provisions for a new double filament lamp of category HS5 for motorcycles.  The suggested amendments to the Regulation are marked in bold characters.  The text is based upon :

- The text of Revision 3;

- Including (draft) Supplements 21 and 22;

- Including draft Corrigendum 1 to Revision 3;

- Draft proposals to introduce red filament light source (TRANS/WP.29/GRE/2002/36);

- Draft proposals for editorial amendments (TRANS/WP.29/GRE/2002/31);

- Informal document No. 2 distributed during the forty-ninth session of GRE.

_______

__________________

Note:	This document is distributed to the Experts on Lighting and Light-Signalling only.

A.	PROPOSAL

Text of the Regulation,

Annex 1,

List of categories of filament lamps, amend to read:

“...

|  | Category | Sheet number(s) |  |
| --- | --- | --- | --- |

...

|  | HS2 | HS2/1..3 |  |
| --- | --- | --- | --- |
|  | HS5 | HS5/1..4 |  |
|  | R2 | R2/1..3 |  |

...”

List of sheets for filament lamps, amend to read:

“...

|  | Sheet number(s) |  |
| --- | --- | --- |

...

|  | HS2/1..3 |  |
| --- | --- | --- |
|  | HS5/1..4 |  |
|  | P19W/1..3 |  |

...”

New sheets HS5, insert in between sheet HS2/3 and sheet P19W/1 as indicated in the List of sheets above, to read:

(see next pages)

*      *     *

B.	JUSTIFICATION

The proposed draft amendments concern the addition of a new double filament lamp for motorcycles. The proposed introduction of a new harmonized symmetrical beam pattern was the immediate cause to this development.  The proposal also corrects a few editorial errors.

_________________

CATEGORY HS5 	Sheet HS5/1

The drawings are intended only to illustrate the essential dimensions (in mm) of the filament lamp

FILAMENT LAMP FOR MOTORCYCLES

Figure 1  Main drawing

A

e

B

Reference plane  1 /

50max

Ø24.5

max

Reference axis  2 /

3 /

Passing - beam

Driving - beam

Earth

Figure 2  Distortion free area 4 / and black top 5 /







4 /

5 /

Reference axis  2 /

1/	The reference plane is defined by the three ramp inside surface.

2/	The reference axis is perpendicular to the reference plane and passing through the centre of the 23 mm cap diameter.

3/	Glass bulb and supports shall not exceed the envelope as indicated in figure 1. The envelope is concentric to the reference axis.

4/	Glass bulb shall be optically distortion free within the angles 1 and 2. This requirement applies to the whole bulb circumference within the angles 1 and 2.

5/	The obscuration shall extend at least to angle 3 and shall extend at least to the cylindrical part of the bulb on the whole top circumference.

CATEGORY HS5 	Sheet HS5/2

Filament centre

View A of passing-beam filament

lC

k

h1

h3

e

View B of driving-beam filament

lR

j

g1

g3

e

Filament centre

Figure 3

Filament position and dimensions

Reference axis

Top view of driving-beam filament

g2

g4

Top view of passing-beam filament

h2

h4

Reference axis

CATEGORY HS5 	Sheet HS5/3

| Dimensions in mm |  |  |  |  | Filament lamps of normal production |  |  | Standard filament lamp |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 12V |  |  | 12V |  |
| e |  | 26 |  |  | 6 / |  |  |  0.15 |  |
| lC 7 / |  | 4.6 |  |  |  |  |  |  0.3 |  |
| k |  | 0 |  |  |  |  |  |  0.2 |  |
| h1, h3 |  | 0 |  |  |  |  |  |  0.15 |  |
| h2, h4 |  | 0 |  |  |  |  |  |  0.20 |  |
| lR 7 / |  | 4.6 |  |  |  |  |  |  0.3 |  |
| j |  | 0 |  |  |  |  |  |  0.2 |  |
| g1, g3 |  | 0 |  |  |  |  |  |  0.30 |  |
| g2, g4 |  | 2.5 |  |  |  |  |  |  0.40 |  |
|  |  | 50 min. |  |  | - |  |  | - |  |
|  |  | 23 min. |  |  | - |  |  | - |  |
|  |  | 50 min. |  |  | - |  |  | - |  |
| Cap P23t in accordance with IEC Publication 60061 (sheet 7004-....-1) |  |  |  |  |  |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARACTERISTCS |  |  |  |  |  |  |  |  |  |
| Rated values | Voltage |  | V | 12 |  |  | 12 |  |  |
|  | Wattage |  | W | 35 |  | 30 | 35 |  | 30 |
| Test voltage |  |  | V | 13.2 |  |  | 13.2 |  |  |
| Objective Values | Wattage |  | W | 40 max. |  | 37 max. | 40 max. |  | 37 max. |
|  | Luminous flux |  | lm | 620 |  | 515 |  |  |  |
|  |  |  | % | 15 |  | 15 |  |  |  |
| Reference luminous at approximately 12V |  |  |  |  |  |  | 460 lm |  | 380 lm |

6/	To be checked by means of a “box-system”. Sheet HS5/4.

7/	The positions of the first and the last turn of the filament are defined by the intersections of the outside of the first and the outside of the last light-emitting turn, respectively, with the plane parallel to and 26 mm distant from the reference plane.

CATEGORY HS5 	Sheet HS5/4

Screen projection requirement

This test is used to determine whether a filament lamp complies with the requirements by checking whether:

(a)  the passing-beam filament is correctly positioned relative to the reference axis and the reference plane; and whether

(b)  the driving-beam filament is correctly positioned relative to the passing-beam filament.

Reference axis

c

d

v

b

a

26.0 to reference plane

Passing-beam filament

Driving-beam filament

Dimensions in millimeters

Side elevation

| Reference | a | b | c | d | v |
| --- | --- | --- | --- | --- | --- |
| Dimensions | d1+0.6 | d1+0.8 | d2+1.2 | d2+1.6 | 2.5 |

d1 : Diameter of the passing-beam filament

d2 : Diameter of the driving-beam filament

Reference axis

k    k

//

h

26.0 to reference plane

//

Front elevation

| Reference | h | k |
| --- | --- | --- |
| Dimensions | 6.0 | 0.5 |

The filaments shall lie entirely within the limits shown.

The centre of the filament shall lie within the limits of dimension k.

________________