| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. RESTRICTED TRANS/WP.29/GRE/2003/34 7 July 2003 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

(Fifty-first session, 15-19 September 2003,

agenda item 1.3.1.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 98

(Headlamps with gas-discharge light sources)

Transmitted by the expert from the Working Party "Brussels 1952" (GTB)

Note:		The text reproduced below was prepared by the secretariat (see TRANS/WP.29/GRE/50, paras. 39 and 40).  It based on the document without a symbol (informal document No. 20), distributed during the fiftieth GRE session.  The document aims to incorporate in the Regulation the specifications of the harmonized driving beam pattern (TRANS/WP.29/GRE/47, para. 60).  With regard to the request by GRE (see TRANS/WP.29/GRE/50, para. 40), GTB informed the secretariat on its decision to maintain the use of the letters as proposed in the informal document.

_______

__________________

Note:	This document is distributed to the Experts on Lighting and Light-Signalling only.

A.	PROPOSAL

Paragraph 4.2.2.4., amend to read:

"4.2.2.4.	on headlamps meeting the requirements of this Regulation in respect of the driving beam only, the letter "DR"; or on headlamps meeting the requirements of the worldwide harmonized driving beam specified in tables D & E in annex 3, the letter "DH";"

Paragraph 4.2.2.5., amend to read:

"4.2.2.5.	on headlamps meeting the requirements of this Regulation in respect of both the passing beam and the driving beam, the letters "DCR"; or on headlamps meeting the requirements of the worldwide harmonized driving beam specified in tables D & E in annex 3, the letter "DCH";"

Paragraph 6.3.2.3., amend to read:

"6.3.2.3.	Starting from point HV, horizontally to the right and left, the illuminance shall be not less than 40 lux up to a distance of 1.125 m and not less than 10 lux up to a distance of 2.25 m.

In the case of a headlamp designed to produce a worldwide-harmonized driving beam, the intensities shall conform to the tables "A" and "B" in annex 3.  Table "A" applies in the case where a primary driving beam is being produced with a single light source.  Table "B" applies in the case where the driving beam is being produced by a secondary driving beam headlamp operated with a passing beam headlamp or a primary driving beam headlamp."

Annex 1, footnote 3/, add the markings "DH, DH PL," and "DCH, DCH PL" to the existing list.

# Annex 3,

# Insert after the title tables A and B, to read:

"Annex 3

Table A - Primary driving beam headlamp

Refer to Figure D for details of test point positions

| TEST POINT NUMBER | ### TEST POINT LOCATION | Required illumination in lux |  |
| --- | --- | --- | --- |
|  |  | Min. | Max. |
| 1 | H-V (1/) | (1/) | --- |
| 2 | H-3R & 3L | 40.0 | --- |
| 3 | H-6R & 6L | 10.0 | --- |
| 4 | H-9R & 9L | 4 | --- |
| 5 | H-12R & 12L | 1.2 | --- |
| 6 | 2U-V | 2 | --- |
| 7 | 4D-V | --- | (2/) |
|  | MAX. LUMINOUS INTENSITY | 70.0 | 180.0 |

(1/)	Intensity at H-V shall be equal to or greater than 80 per cent of the maximum intensity in the beam pattern.

(2/)	Intensity at 4D-V shall be equal to or less than 30 per cent of the maximum intensity in the beam pattern.

Table B - Secondary driving beam headlamp operated

with a passing beam headlamp or a primary driving beam headlamp

## Refer to Figure E for details of test point positions

| TEST POINT NUMBER | TEST POINT LOCATION | Required illumination in lux |  |
| --- | --- | --- | --- |
|  |  | Min. | Max. |
| 1 | H-V (1/) | (1/) | --- |
| 2 | H-3R & 3L | 40.0 | --- |
| 3 | H-6R & 6L | 10.0 | --- |
| 6 | 2U-V | 2 | --- |
| 7 | 4D-V | --- | (2/) |
|  | MAX. LUMINOUS INTENSITY | 70 | 180 |

(1/)	Intensity at H-V shall be equal to or greater than 80 per cent of the maximum intensity in the beam pattern.

(2/)	Intensity at 4D-V shall be equal to or less than 30 per cent of the maximum intensity in the beam pattern."

# Add at the end of annex 3 figures D and E, to read:

"Figure D

Primary Driving Beam

Figure E

Secondary Driving Beam

______________"