Informal document No.  25

(50th GRE, 7-11 April 2003,

agenda item 1.4.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48, 
02 SERIES OF AMENDMENTS

# Transmitted by the Experts from the Czech Republic and Germany

### A.	PROPOSAL

Insert a new paragraph 5.24., to read:

"5.24.	Electrical connections between the towing and the towed vehicles

The electrical connection of lighting and light – signalling equipment of the towed vehicle to the towing vehicle shall be provided by a 7 – pole black (dark) coloured socket (on towing vehicle) and plug (on towed vehicle) according to ISO 1724 Standard, type 12 N for the 12 V system, or to ISO 1185 Standard, type 24 N for the 24 V system; the socket and plug for other purposes (supplementary) shall be white (light) coloured and shall meet the requirements of ISO 3732 Standard, type 12 S for the 12 V system or to ISO 3731 Standard, type 24 S for the 24 V system.

5.24.1.	On category M1 and N1 vehicles and on the category O1 and O2 trailers installation of a 13 – pole connector ISO 11 446 instead of a 7 – pole socket and plug is permissible.

5.24.2.	On category N2 and N3 vehicles the connection between the towing and towed vehicles may be provided by a 15 – pole socket and plug according to ISO 12 098.

5.24.3.	If a vehicle is equipped with a 15 – pole socket and the trailer with two 7 – pole plugs:

5.24.3.1.	an adaptor according to ISO 12 098, Annex A – Example 1 shall be used.

5.24.4.	If a vehicle is equipped with two 7 – pole sockets and the trailer with a 15 – pole plug:

5.24.4.1.	an adaptor according to ISO 12 098, Annex A – Example 2 shall be used."

B.	JUSTIFICATION

Unlikely Regulations Nos. 13 and 48 do not determine the means of electrical connection between the towing and the towed vehicle for supplying and control of the lighting and light – signalling devices of the trailer.  The Czech delegate was asked by the German delegation to repeat his proposal tabled in 1994.  The proposal was amended according to the state of the ISO standards, which the Czech delegate has had to his disposal (see below).

The state of standards is the following:

12 V system: 	7 – pole connector, type 12N (normal)	ISO 1724, Third edition, 1997

7 – pole connector, type 12S (supplementary)	ISO 3732, Third edition, 1997

24 V system:	7 – pole connector, type 24N (normal)	ISO 1185, Second edition, 1997

7 – pole connector, type 24S (supplementary)	ISO 3731, Third edition, 1997

12 V system:	13 – pole connector	ISO 11 446, Second edition, 1995

24 V system:	15 – pole connector	ISO 12 098, First edition, 1994