GLOBAL HARMONIZATION OF PTW, AND 3W BRAKING

DEVELOPMENT OF DRAFT GTR PROPOSAL

Conclusions reached at the 1st Informal meeting on the motorcycle braking GTR, Pisa, 03/07/16-17

Document revision date: 03/07/29

| # 1. SCOPE | # 1. Scope | Wait for the outcome of GRSG vehicle classification. Intended to apply to two and three-wheeled motorcycles including mopeds | T Canada to consider how to separate out 3-wheelers which are not considered motorcycles |
| --- | --- | --- | --- |
| # 2. DEFINITIONS | # 2. Definitions | To be determined. Need to add: brake, transmission, control, CBS, split service, ABS, laden | Add during GTR development |
| # 3. REQUIREMENTS # 3.1 Performance | # 3. Requirements | - Performance requirement to be based on MFDD or [the corrected stopping distance, taken from measured data]. - Test the brakes separately, unless otherwise specified - Clarify requirements for split brake systems where necessary | - T Canada and USA to review whether or not to include stopping distance. - Define conversion formula |
| 3.2 General: |  |  |  |
| 3.2.1 Design | 19. Design durability | Needs to be specific and in principle testable | IMMA to propose a text based on FMVSS and 13H |
|  | 4. Type of service brake system | see definitions | see definitions |
|  | # 10. Parking brake characteristics | ECE or FMVSS | USA to review the current test procedures |
|  | # 5. Mechanical service brake system | A general requirement for component integrity would be appropriate | Need to define general test procedures (so that the requirement can be enforced) or delete the item |
|  | 6. Hydraulic service brake system |  |  |
|  | 7. Master cylinder reservoirs | Use text from FMVSS 122 (S5.1.2.1) |  |
|  | 8. Reservoir labelling | Make a requirement to have a warning statement and not specify the language (if not done by symbols) based on FMVSS | IMMA to research how this would be treated by the EU, e.g. using symbols and an explanation in the owners manual |
| 3.2.2 Operation | 9. Failure indicator lamp | Use text from FMVSS ( S5.1.3, S5.1.3.1) |  |
|  | 11. Inspection of pad/lining | Use FMVSS wording. | To agree when this should be done |
| 4. TESTS |  |  |  |
| 4.1 General | 23. Transmission | Unless otherwise specified, all stops are to be made with the clutch disengaged. (Not applicable to auto gearbox) |  |
|  | 25. Ambient temperature | Ambient temperature between 4 and 38°C. (to avoid frozen surface) |  |
|  | 26. Wind velocity | Not more than 5 m/s. |  |
|  | 28. Vehicle position and wheel lock | All stops to be made without deviation from the test lane and without wheel lockup (not applicable to ABS equipped vehicles <10 km/h). Vehicle shall start in the middle of the lane |  |
|  | 30. Brake actuation force | T Canada does not think minimum values are necessary ECE/Japanese more stringent | USA to review need for minimum values and clarify current requirements IMMA to provide clear specification of the application point, e.g. ISO |
|  | 29. Thermocouples | Brake temperature to be measured on the disc or shoe with thermocouples. |  |
|  | 31. Test procedure and sequence | Not necessary but to simplify testing, the heat fade test could be done last | #### To agree any sequence requirement |
| 4.2 Preparation |  |  |  |
| 4.2.1 Track | 27. Road surface | Test area to be clean, dry and level road surface. Test surface to have a skid number of 81. Maximum lane width of 2.5 m for two-wheeled motorcycles; vehicle width plus 2.5m for two-wheeled motorcycles with sidecar or three wheeled motorcycles. [For ABS, see ECE 78 Annex 4] For the parking brake test, surface shall be Portland cement concrete . |  |
| 4.2.2 Vehicle | 21. Vehicle weight | Wait for the definition of mass from GRSG (if time allows) Final decision will depend on agreed test procedures | USA to consider any consequences from the Phase 2 testing |
|  | 22. Tyre pressure | Manufacturer recommendation |  |
|  | Brake burnishing | Include a requirement that the manufacturer will carry out the burnishing and show records to the test house on request | IMMA to draft a proposal with justification |
| 4.3 Dry Stop Tests | 13. Dry stop tests | Separate the basic and the high speed tests | To agree on the number of stops needed IMMA to review possible use of ISO method (5 tests average the 3 in the middle) |
| 4.4 Wet Brake | 18. Wet braking | the ECE test for disc brake systems | USA and T Canada to consider need for a drum brake test IMMA to consider a revised immersion test for drum brakes, particularly for scooters. |
| 4.5 Heat Fade | 14. Fade and recovery | Mopeds (L1 & L2) would be excluded Provisionally, the ECE method would be used | T Canada to review low speed motorcycles (not mopeds) USA to review requirements |
| 4.6 High Speed | 13. High Speed Test | Use test procedure from Japanese standard for the high speed test. | USA to review high speed test |
| 4.7 Parking Brake | 17. Parking brake system efficiency | No conclusion | USA to review relative severity |
| 4.8 ABS | 37. Anti-lock (ABS) Systems | ECE Annex 4 is the only procedure available. May be altered following future tests with ABS systems. | Need to wait for the results of the USA's ABS testing |
| 4.9 Partial Failure | 16. Partial failure | Only needed for split service brake systems. | IMMA to consider the FMVSS test procedure and prepare an adaptation if necessary (e.g. to include MFDD) |
| ITEMS PROVISIONALLY DELETED : |  |  |  |
|  | 12. Pre burnish test | Provisionally, not necessary | USA to review the need for a pre-burnishing test |
|  | 15. Final effectiveness test | Not necessary because no sequence of testing specified | USA and T Canada to review need for test |
|  | 24. Engine | Not applicable to motorcycles | USA to review need |
|  | 33. Pre-test instrumentation check | Test house responsibility |  |
|  | 36. Final inspection | Not necessary because no sequence of testing specified |  |