Informal document No. 24

(85th  GRSG, 21-24 October 2003

agenda item 2)

- OICA PROPOSAL FOR DRAFT AMENDMENTS TO DOCUMENT TRANS/WP.29/GRSG/2003/19

- UN/ECE R52 (SMALL BUS CONSTRUCTION)

A. Proposal

Para. 5.7.8.5.1 of part 5, amend to read:

"5.7.8.5.1	For a seat behind a partition or other rigid structure other than a seat, a minimum clear space in front of each passenger seat shall be provided as shown in Annex 3, figure 16a. Alternatively A partition whose contour corresponds approximately to that of the inclined seat back may intrude into this space as provided by paragraph 5.7.8.4.  Intrusion of the dashboard, instrument panel, windscreen, sun visor, seat belts and seat belt anchorages shall be allowed.”

New Inserted paragraph, amend to read:

"5.7.8.5.2.	For a seat behind a seat and/or a seat facing the gangway, a minimum clear foot space of at least 300 mm depth and a width equal to that specified for a seat cushion in paragraph 5.7.8.2.1., reduced where appropriate by paragraphs 5.7.8.1.3.3 and 5.7.8.1.3.4, shall be provided as shown in Annex 3, figure 16b. The local presence in this space of seat legs shall be permitted provided that adequate space remains for the passengers' feet. This foot space may partly be situated in and/or above the gangway but shall not create any obstruction when measuring the gangway-width in accordance with paragraph 5.7.5.  Intrusion of the seat belts and seat belt anchorages shall be allowed."

Annex 3, figure 16, amend to read:

"Figure 16a

Space for seated passengers behind a partition

or other rigid structure other than a seat"

(see paragraph 5.7.8.5.1)

Insert a new drawing:

"Figure 16b

Space for seated passengers behind a seat

and or/or a seat facing the gangway

(see paragraph 5.7.8.5.2) "

60 cm max

30 cm min

30 ° max

– 8 ° min

70 cm min

B. Justification:

The intention of the proposal is:

- to clarify the interpretation of paragraph 5.7.8.5.1. and Annex 3 figure 16, which was intended not to impose any additional requirements on seat spacing but instead to determine the clear space between a passenger seat and a partition or other rigid structure other than a seat, and

- to clarify that the foot space in front of each passenger seat may be so located such that parts of the foot space "intrude" into the gangway, either on the floor or above it. In any case, while measuring the gangway width with the respective testing gauge, the foot space shall not obstruct movement of the gauge.

_____________