Paragraph 6.1.6., the reference to "paragraph 1.42.", amend to read "paragraph 1.20."

Paragraph 7.2.1.2., amend to read:

"  ........... in the position of actual unlocking and when projected into a plane perpendicular to the button's initial direction of motion:  for enclosed devices ............ two dimensions forming the prescribed area."

Paragraph 8.2.1.1.2., amend to read:

" .......... or the vehicle without opening the buckle.  A tension of 200 + 2 N shall be applied to the buckle.  If the buckle is attached to a rigid part, the force shall be applied reproducing the angle formed between the buckle and that rigid part during the dynamic test."

Annex 6, appendix 3, figures 1 and 2, the text reading:

" DISTANCE  C - Re = 550  mm

ANGLE "ANG" = 30° MAXIMUM"

amend to read (in each figure):

" DISTANCE C - Re = 530  mm

Re is located on the centerline of the retractor spool

ANGLE "ANG" = 30° MAXIMUM"

Annex 13,

Paragraph 2., amend to read: */

" ...............   two anchorages points (A1 and A2) (see Figure 1), and a central part (N, in detail in Figure 3).  The retractor ......

Paragraph 3., amend to read:

" .............

Belt retractor R shall be fitted to trolley anchorage such that the spool centre-line is positioned on Re

The value of X in Figure 1 below is 200 + 5 mm.  The effective strap length between A1 and the centre-line of the retractor spool Re (when the webbing is fully extracted including the minimum length of 150 mm*) for testing of universal and semi-universal categories shall be 2820 + 5 mm when measured in a straight line without load and on a horizontal surface; this length may be increased for testing of restricted category; for all categories with the child restraint installed there shall be a minimum of 150 mm* of strap length on the retractor spool."

Figure 1, amend to read :

“

“

*          *          *