Informal Document No. GRSP-34-26

(34th GRSP, 8-12 December 2003,

agenda item B.1.5.)

Draft Amendment to Regulation No. 44

Corrigendum to Document TRANS/WP29/GRSP/2003/15

# Transmitted by the expert from CLEPA

A. PROPOSAL

Paragraph 7.1.4.1.9. a), add at the end:

"…in annex 6.  The floor pan will be adjusted to these minimum and maximum lengths of the support leg. If there is clearance between the shortest length and the highest floor pan, the leg is adjusted to the highest floor pan level of 70 mm. If the maximum support leg length is more than the lowest floor pan height, 210 mm length adjustment is used.  Length adjustments of support legs will be tested according to 7.1.4.1.9. "

B. JUSTIFICATION

If the maximum support leg length is more than the lowest floor pan height would allow to be used, the floor pan is set to this lowest height (280 mm below the Cr axis (see figure 2) and the support leg adjusted accordingly.