| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/2004/64 20 July 2004 Original: ENGLISH |
| --- | --- | --- |

##### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirty-fourth session,

16-19 November 2004, agenda item 5.2.11.)

#### DRAFT CORRIGENDUM 2 TO SUPPLEMENT 5 TO THE 03 SERIES OF AMENDMENTS TO REGULATION No.44

(Safety-belts)

Transmitted by the Working Party on Passive Safety (GRSP)

Note:  The text reproduced below was adopted by GRSP at its thirty-fifth session and is transmitted for consideration to WP.29 and AC.1 (TRANS/WP.29/GRSP/35, para. 27).  It is based on annex 5 to  the report.

_____________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Paragraph 2.1.3., amend to read:

"2.1.3. 	The retention system of Child restraint systems may be of two classes:

an integral class if the retention of the child within the restraint system is independent of any means directly connected to the vehicle;

a non-integral class if the retention of the child within the restraint system is dependent upon any means directly connected to the vehicle;"

Paragraph 6.1.3., amend to read:

"6.1.3.	According to the category which it belongs to, the child restraint shall be secured to the vehicle structure or to the seat structure.

POSSIBLE CONFIGURATIONS FOR APPROVAL

GROUPS / CATEGORIES TABLE

| GROUP CATEGORY |  | Universal (1) |  | Semi-universal (2) |  | Restricted |  | Specific Vehicle |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CRS | ISOFIXCRS | CRS | ISOFIXCRS | CRS | ISOFIXCRS | CRS | ISOFIXCRS |
| 0 | Carry-cot | A | NA | A | A | A | NA | A | A |
|  | Rearward facing | A | NA | A | A | A | NA | A | A |
| 0+ | Rearward facing | A | NA | A | A | A | NA | A | A |
| I | Rearward facing | A | NA | A | A | A | NA | A | A |
|  | Forward facing (integral) | A | A | A | A | A | NA | A | A |
|  | Forward facing (non integral) | A | NA | A | NA | A | NA | A | A |
| II | Rearward facing | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (integral) | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (non integral) | A | NA | A | NA | A | NA | A | A |
| III | Rearward facing | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (integral) | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (non integral) | A | NA | A | NA | A | NA | A | A |

"

_______________