| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/2004/67 11 August 2004 ENGLISH Original: ENGLISH AND FRENCH |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirty-fourth session,

16-19 November 2004, agenda item 5.2.17.)

#### PROPOSAL FOR DRAFT SUPPLEMENT 4 TO REGULATION No. 85

(Measurement of net power)

Transmitted by the Working Party on Pollution and Energy (GRPE)

Note:  The text reproduced below was adopted by GRPE at its forty-eighth session and is transmitted for consideration to WP.29 and AC.1 (TRANS/WP.29/GRPE/48, para. 32).  It is based on document TRANS/WP.29/GRPE/2004/9, not amended.

____________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Insert new paragraphs 2.5. to 2.5.3., to read:

"2.5.	"Hybrid vehicles (HV)":

2.5.1.	"Hybrid vehicle (HV)" means a vehicle with at least two different energy converters and two different energy storage systems (on vehicle) for the purpose of vehicle propulsion.

2.5.2.	"Hybrid electric vehicle (HEV)" means a vehicle that, for the purpose of mechanical propulsion, draws energy from both of the following on-vehicle sources of stored energy/power:

-	a consumable fuel

-	an electrical energy/power storage device (e.g.: battery, capacitor, flywheel/generator ...)

2.5.3.	For a hybrid electric vehicle the "power train" comprises a combination of two different drive train types:

-	an internal combustion engine, and

-	one (or several) electric drive train(s)."

Paragraph 3.2., amend to read:

"3.2.	It shall be accompanied by the description, in triplicate, of the drive train comprising all the relevant particulars referred to:

-	in Annex 1 for vehicles powered by an internal combustion engine only, or

-	in Annex 2 for pure electric vehicles, or

-	in Annexes 1 and 2 for hybrid electric vehicles."

Insert a new paragraph 3.3., to read:

"3.3.	For hybrid electric vehicles (HEV), the tests shall be carried out separately on the internal combustion engine (according to Annex 5) and on the electric drive train(s) (according to Annex 6)."

Paragraph 3.3. (former), renumber as paragraph 3.4. and amend to read:

"3.4.	A drive train (or set of drive trains), representative of the (set of) drive train type(s) to be approved, shall, with the equipment prescribed in Annexes 5 and 6 to this Regulation, be submitted to the technical service conducting the approval tests."

Annex 3, (COMMUNICATION)

Items 1 to 5, amend to read:

"1.	Trade name or mark of drive train or set of drive trains:…......……………….

2.	Internal combustion engine:

2.1.	Make: ............................................................………………….......………….

2.2.	Type: .................................................................................................…………

2.3.	Manufacturer's name and address: ....................................................................

3.	Electric drive train(s):

3.1.	Make: ............................................................…………………….....………..

3.2.	Type: ...............................................................................................………….

3.3.	Manufacturer's name and address: .......................…........................................

5.	Drive train or set of drive trains submitted for approval on:……..…………"

Item 12, amend to read:

"12.	Electric drive train(s):………………………………….....………………….."

______________