| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/GRE/2004/23 16 July 2004 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

(Fifty-third session, 4–8 October 2004,

agenda item 2.2.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48

(Installation of lighting and light-signalling devices)

Transmitted by the expert from Canada

Note:		The text reproduced below was prepared by the expert from Canada in order to limit the projection of white light towards the rear of the vehicle.  It is based on a document without a symbol (informal document No. GRE-52-7), distributed during the fifty-second GRE session which was revised by the expert from Canada (TRANS/WP.29/GRE/52, para. 10).  The modifications to the existing text of the Regulation are marked in bold characters.

_______

__________________

Note:	This document is distributed to the Experts on Lighting and Light-Signalling only.

A.	PROPOSAL

Paragraphs 5.10. to 5.10.2., amend to read:

"5.10.	Visibility of coloured light forward and rearward of a vehicle

Note: No account shall be taken of lighting devices fitted for the interior lighting of a vehicle.

5.10.1.	Visibility of light other than white towards the front of a vehicle

With the exception of a red rearmost side-marker lamp, there must be no direct visibility of the apparent surface of a lamp capable of projecting or retro-reflecting light of any colour other than white if viewed by an observer moving within Zone 1 as specified in annex 4.

5.10.2.	Visibility of light other than red towards the rear of a vehicle

With the exception of a reversing lamp, cornering lamp and direction indicator lamp, there must be no direct visibility of the apparent surface of a lamp capable of projecting or retro-reflecting light of any colour other than red if viewed by an observer moving within Zone 2 as specified in annex 4."

*     *     *

B.	JUSTIFICATION

This document was developed as a response to informal document No. GRE-52-6 tabled by the expert from the United Kingdom, in which the problem of reflecting light of questionable colour was raised.  It also expands the French proposal adopted during the forty-second GRE (TRANS/WP.29/2004/51).  The French proposal addressed only the visibility of red light projected through the apparent surface of a lamp; this Canadian proposal would prohibit the projection through the apparent surface of a lamp of any colour other than white towards the front of a vehicle.

_________