| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/GRE/2004/25 16 July 2004 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

(Fifty-third session, 4–8 October 2004,

agenda item 8.)

PROPOSAL FOR DRAFT COLLECTIVE AMENDMENTS TO

REGULATIONS Nos. 37 and 99

(with regard to colour definitions)

Transmitted by the expert from the United Kingdom

Note:	The text reproduced below was prepared by the expert from the United Kingdom to introduce into the Regulations collective amendments regarding colour specifications.  It is based on a document without a symbol (informal document No. GRE-52-14), distributed during the fifty-second GRE session (TRANS/WP.29/GRE/52, para. 39).  The modifications to the existing text of the Regulations are marked in bold characters.

_______

__________________

Note:	This document is distributed to the Experts on Lighting and Light-Signalling only.

A.	PROPOSAL

###### ECE REGULATION No. 37 – Filament lamps

(The following text is based upon Supplement 24 to the 03 series of amendments)

Paragraph 3.6.2., amend to read:

"3.6.2.	The colour of the light emitted shall have chromaticity coordinates (x,y) (CIE Publication 15.2, 1986, Colorimetry, the CIE 1931 standard colorimetric observer), that lie inside the chromaticity areas defined by the following boundaries:

finished filament lamps emitting white light:

| W12 | green boundary: | y = 0.150 + 0.640 x |
| --- | --- | --- |
| W23 | yellowish green boundary: | y = 0.440 |
| W34 | yellow boundary: | x = 0.500 |
| W45 | reddish purple boundary: | y = 0.382 |
| W56 | purple boundary: | y = 0.050 + 0.750 x |
| W61 | blue boundary: | x = 0.310 |

with intersection points:

|  | x | y |
| --- | --- | --- |
| W1: | 0.310 | 0.348 |
| W2: | 0.453 | 0.440 |
| W3: | 0.500 | 0.440 |
| W4: | 0.500 | 0.382 |
| W5: | 0.443 | 0.382 |
| W6: | 0.310 | 0.283 |

finished filament lamps emitting selective-yellow light:

| SY12 | green boundary: | y = 1.290 x - 0.100 |
| --- | --- | --- |
| SY23 | the spectral locus |  |
| SY34 | red boundary: | y = 0.138 + 0.580 x |
| SY45 | yellowish white boundary: | y = 0.440 |
| SY51 | white boundary: | y = 0.940 - x |

with intersection points:

|  | x | y |
| --- | --- | --- |
| SY1: | 0.454 | 0.486 |
| SY2: | 0.480 | 0.519 |
| SY3: | 0.545 | 0.454 |
| SY4: | 0.521 | 0.440 |
| SY5: | 0.500 | 0.440 |

finished filament lamps emitting amber light:

| A12 | green boundary: | y = x - 0.120 |
| --- | --- | --- |
| A23 | the spectral locus |  |
| A34 | red boundary: | y = 0.390 |
| A41 | white boundary: | y = 0.790 - 0.670 x |

with intersection points:

|  | x | y |
| --- | --- | --- |
| A1: | 0.545 | 0.425 |
| A2: | 0.557 | 0.442 |
| A3: | 0.609 | 0.390 |
| A4: | 0.597 | 0.390 |

finished filament lamps emitting red light:

| R12 | yellow boundary: | y = 0.335 |
| --- | --- | --- |
| R23 | the spectral locus |  |
| R34 | the purple line | (its linear extension across the purple range of colours between the red and the blue extremities of the spectral locus). |
| R41 | purple boundary: | y = 0.980 – x |

with intersection points:

|  | x | y |
| --- | --- | --- |
| R1: | 0.645 | 0.335 |
| R2: | 0.665 | 0.335 |
| R3: | 0.735 | 0.265 |
| R4: | 0.721 | 0.259 |

"

*     *     *

###### ECE REGULATION No. 99 – Gas-discharge light sources

(The following text is based upon Supplement 2 to the Regulation)

Annex 1, sheets DxR/4 and DxS/4, amend the tables to read:

"…

| Luminous flux | … | … | … | … |
| --- | --- | --- | --- | --- |
| Chromaticity co-ordinates | Objective |  | x = 0.375 | y = 0.375 |
|  | Tolerance area 3/ | ### within boundaries | x = 0.345 x = 0.405 | y = 0.150 + 0.640 x y = 0.050 + 0.750 x |
|  |  | Intersection points x,y of boundaries | x = 0.345 | y = 0.371 |
|  |  |  | x = 0.405 | y = 0.409 |
|  |  |  | x = 0.405 | y = 0.354 |
|  |  |  | x = 0.345 | y = 0.309 |
| Hot-restrike switch off time |  | s | 10 | 10 |

…"

*     *     *

B.	JUSTIFICATION

To avoid an unnecessary complication of the sheet, it is suggested to keep the design as it is.  The above-mentioned examples were given for Regulations Nos. 37 and 99.  For the amendments to other Regulations, the texts need to be rephrased.

Alternatives

As an alternative the chromaticity co-ordinates could be:

1.	annexed to each relevant Regulation;

2.	annexed to Regulation No. 48, (or Regulations Nos. 53, 74 and 86) concerning the installation of lighting and light-signalling devices on vehicles;

3.	annexed to Regulations Nos. 37 (filament lamps) and/or 99 (gas-discharge light sources);

4.	adopted as a new Regulation concerning general specifications, definitions, colour boundaries, test procedures, etc.

For these purposes, the legal and practical consequences should be considered.

Relevant Regulations (to be completed):

|  | UNECE Regulation |  | body/ annex | paragraph | colour of light |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ##### ACTIVE devices |  |  | white | selective yellow | amber | red |
| x | 5 | sealed beam headlamps |  |  | x | x |  |  |
| x | 6 | direction indicators |  |  |  |  | x |  |
| x | 7 | stop, position, markers |  |  | x |  |  | x |
| x | 19 | front fog |  |  | x | x |  |  |
| x | 23 | reversing |  |  | x |  |  |  |
| x | 31 | halogen sealed beam headlamps |  |  | x | x |  |  |
| x | 37 | filament light sources |  |  | x | x | x | x |
| x | 38 | rear fog |  |  |  |  |  | x |
| x | 50 | position, stop, direction indicators |  |  | x? |  | x | x |
| ? | 65 | special warning lamps |  |  |  |  |  |  |
| ? | 77 | parking lamps |  |  |  |  |  | x |
| x | 87 | DRL |  |  |  |  |  |  |
| x | 91 | side marker |  |  |  |  | x | x |
| x | 98 | gas-discharge headlamps |  |  | x | x |  |  |
| x | 99 | gas-discharge light sources |  |  | x | x |  |  |
| x | 112 | asymmetrical passing beam |  |  | x |  |  |  |
| x | 113 | symmetrical passing beam |  |  |  | x |  |  |
|  |  | # PASSIVE devices |  |  |  |  |  |  |
| P | 3 | retroreflective devices |  |  |  |  | x | x |
| P | 4 | illumination of rear registration plates |  |  |  |  |  |  |
| P | 27 | warning triangles |  |  |  |  |  |  |
| P | 69 | rear marking plates |  |  |  |  |  | tables 1,2,3 |
| P | 70 | rear marking plates heavy and long vehicles |  |  |  |  | yellow tables 1,2 | tables 1,2,3 |
| P | 88 | retroreflective tyres |  |  |  |  |  |  |
| P | 104 | retroreflective contour markings |  |  |  | yellow table 1 | yellow table 2 |  |

*     *     *

C.	INFORMATION

# The current texts of the Regulations and standards including the amendments for selective yellow read:

# Regulation No. 37, paragraph 3.6.2.:

"3.6.2.	The colourimetric characteristics of the light emitted, expressed in CIE trichromatic coordinates, shall lie within the following limits:

finished filament lamps emitting white light:

limit towards blue:	x ≥ 0.310

limit towards yellow:	x ≤ 0.500

limit towards green:	y ≤ 0.150 + 0.640 x

limit towards green:	y ≤ 0.440

limit towards purple:	y ≥ 0.050 + 0.750 x

limit towards red:	y ≥ 0.382;

finished filament lamps emitting selective-yellow light:

limit towards red	y ≥ 0.138 + 0.580 x

limit towards green	y ≤ 1.290 x - 0.100

limit towards white:	y ≥ 0.940 - x and y ≥ 0.440;

finished filament lamps emitting amber light:

limit towards green:	y ≤ x - 0.120

limit towards red:	y ≥ 0.390

limit towards white:	y ≥ 0.790 - 0.670 x;

finished filament lamps emitting red light:

limit towards yellow:	y ≤ 0.335

limit towards purple:	y ≥ 0.980 – x."

# Regulation No. 99, annex 1, sheets DxR/4 and DxS/4:

"…

| …. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Colour co-ordinates | Objective |  | x = 0.375 | y = 0.375 |
|  | Tolerance area 3/ |  | x ≥ 0.345 x ≤ 0.405 | y ≤ 0.150 + 0.640 x y ≥ 0.050 + 0.750 x |
| …. |  |  |  |  |

…."

CIE standard S 004/E-2001:

"4.2	Specified chromaticity areas:

The colours of light signals shall have chromaticity coordinates (x, y) that lie inside the chromaticity areas defined by the boundaries specified in Table 1 and, where applicable, by the spectral locus and its linear extension across the purple range of colours between the red and blue extremities of the spectral locus.

NOTE 1:	For convenience in plotting the chromaticity areas, the intersection points of the chromaticity boundaries, spectral locus and purple line are given in Table 2 and the areas are illustrated in Figures 1 to 5.

NOTE 2:	The chromaticity coordinates for plotting the spectral locus in Figures 1 to 5 are given in ISO/CIE 10527-1991.

Table 1 - Boundaries of the allowed chromaticity areas for the colours of light signals

| ## Colour | ## Notation | ## Equation of boundary |
| --- | --- | --- |
| RED LIGHT SIGNAL COLOURS |  |  |
| Class A |  |  |
| Yellow boundary | AB | y = 0.320 |
| Purple boundary | AD | y = 0.980 – x |
| etc. |  |  |

Table 2 - Coordinates of intersection points of allowed chromaticity area boundaries

| Colour | ### Chromaticity coordinates |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RED LIGHT SIGNAL COLOURS |  |  |  |  |  |
| ## Class A |  | ## A | ## B | ## C | ## D |
|  | x | 0.660 | 0.680 | 0.735 | 0.721 |
|  | y | 0.320 | 0.320 | 0.265 | 0.259 |
| etc. |  |  |  |  |  |

…."

_________