| ## UNITED NATIONS |  | # E |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr. GENERAL TRANS/WP.29/GRE/2004/47 26 July 2004 Original: ENGLISH ENGLISH AND FRENCH ONLY |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

(Fifty-third session, 4-8 October 2004,

agenda item 13.2.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 37

(Filament lamps)

Transmitted by the expert from France

Note:	The text reproduced below was prepared by the expert from France, in order to clarify its position with regard to the category of filament lamp to use in front fog lamps.  It is a complement to document TRANS/WP.29/GRE/2004/19/Rev.1.

The original proposal was modified by the secretariat in order to align it with the new structure of the Regulation.  The modification to the existing text of the Regulation is marked in bold characters.

_______

__________________

Note:	This document is distributed to the Experts on Lighting and Light-Signalling only.

A.	PROPOSAL

Annex 1, the list of categories of filament lamps and their sheets, amend to read (inserting a new footnote ***/ to exclude the use of categories HIR1 and H9 for the use in front fog lamps, and moving the categories P24W and PS24W from the list of categories for signalling lamps only to a new inserted list):

| "…. |  |  |
| --- | --- | --- |
| Category |  | Sheet number(s) |
| …. |  |  |
| H9 | ***/ | H9/1 to 4 |
| H9B | ***/ | H9/1 to 4 |
| … |  |  |
| HIR1 | ***/ | HIR1/1 to 3 |
| … |  |  |
| S3 |  | S3/1 |
| only for signalling lamp and front fog lamps |  |  |
| P24W |  | P24W/1 to 3 |
| PS24W |  | P24W/1 to 3 |
| only for signalling lamps |  |  |
| … |  |  |
| P21/5W |  | P21/5W/1 to 3 |
| P27W |  | P27W/1 to 2 |
| … |  | … |
| PS19W |  | P19W/1 to 3 |
| PSR19W | **/ | P19W/1 to 3 |
| ….." |  |  |

Insert a new footnote ***/, to read:

"***/	Not for use in front fog lamps."

*          *          *

B.	JUSTIFICATION

During the fifty-second GRE session, it was decided to revise TRANS/WP.29/GRE/2004/19, in order to allow a front fog lamp with non-removable filament lamp according to Regulation No. 37, using the wording of Regulation No. 112.  This has been done with proposal TRANS/WP.29/GRE/2004/19/Rev.1.  In addition, France proposes to modify Regulation No. 37 in order to allow or to forbid some categories of filament lamps.

_________________