| ## UNITED NATIONS |  | # E |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr. GENERAL TRANS/WP.29/GRPE/2004/10 31 March 2004 ENGLISH ONLY |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Pollution and Energy (GRPE)

(Forty-eighth session, 1-4 June 2004,

agenda item 3.)

WORLDWIDE HARMONIZED MOTORCYCLE EMISSIONS

CERTIFICATION PROCEDURE (WMTC)

TECHNICAL REPORT

Transmitted by the expert from Germany

Note:	The document reproduced below has been transmitted by the expert from Germany on behalf of the informal group developing the worldwide-harmonized motorcycle emissions certification procedure (WMTC).

______________

__________________

Note:	This document is distributed to the Experts on Pollution and Energy only.

CONTENT	Page

1.	Introduction	3

2.	Objective	4

3.	Structure of the project	4

4.	Cycle development	5

4.1.	Approach	5

4.2.	In-use driving behaviour data	6

4.3.	Fleet composition and vehicle use	9

4.4.	The reference database	12

4.5.	Modifications of the draft test cycle and final version	15

5.	Gearshift procedure development	19

5.1.	Approach	19

5.2.	Gearshift criteria, additional requirements	21

5.3.	Gearshift prescriptions	23

5.3.1.	Step 1 – Calculation of shiftspeeds	23

5.3.2.	Step 2 – Gear choice for each cycle sample	24

5.3.2.1.	Gear lever in neutral and clutch disengaged	24

5.3.2.2.	Gear choice for acceleration phases	24

5.3.2.3.	Gear choice for deceleration or cruise phases	25

5.3.3.	Step 3 – Corrections according to additional requirements	25

5.4.	Calculation example	25

6.	Driveability tests	27

7.	Vehicle classification	28

8.	Weighting factors for an overall WMTC emissions result	30

9.	Emissions validation tests	32

9.1.	General information	32

9.2.	Results of the emissions validation programme	37

10.	Test protocol	46

11.	Round robin test	46

12.	Off cycle emissions	50

13.	Summary and conclusions	50

14.	Literature	51

15.	Annex A - Description of the modification work on the WMTC cycle	52

16.	Annex B – Final cycle version	63

____________

1.	INTRODUCTION

At its beginning (1999), the development of the Worldwide-harmonized Motorcycle Test Cycle (WMTC) was a tripartite project between the Netherlands Ministry of the Environment (VROM), TNO Automotive and the International Motorcycle Manufacturer Association (IMMA).  Within this tripartite project, VROM looked after the political aspects of generating a worldwide test cycle.  TNO Automotive, funded by VROM, did the development work and the technical management of the project.  IMMA contributed by the collection of in-use driving behaviour data worldwide.

In a later stage (May 2000), the project was brought under the umbrella of the UNECE/WP.29.  Under the guidance of WP.29, the Group of Experts on Pollution and Energy (GRPE) mandated the ad-hoc group WMTC with the development of a "World-wide Harmonized Motorcycle Emissions Certification/Test ProCedure" and to establish it in the framework of the 1998 Agreement on global technical regulations (gtr).  Since October 2000, RWTUEV Fahrzeug GmbH joined the WMTC group.  RWTUEV Fahrzeug developed a gearshift procedure closely linked to the test cycle.  This work was funded by the German Bundesanstalt für Straßenwesen (BASt).  Since May 2001, RWTUEV Fahrzeug got the responsibility for the cycle development work, the development of the test protocol and the coordination of the validation programme by order of the Netherlands Ministry of the Environment (VROM) and with support by the German Federal Ministry of Transport, Building and Housing (BMVBW).

The WMTC group was formed by members of the following countries/organisations:

- European Commission,

Directorate General Enterprise (DG ENTR),

Joint Research Centre (JRC).

- Germany,

Federal Ministry of Transport, Building and Housing (BMVBW),

Federal Highway Research Institute (BASt),

RWTUEV Fahrzeug GmbH, institute for vehicle technology,

TÜV NORD Straßenverkehr GmbH.

- Italy,

Ministry of Infrastructure and Transport (MoT).

- Japan,

Ministry of Land, Infrastructure and Transport (former Ministry of Transport),

Ministry of Environment (former Environment Agency),

National Traffic Safety and Environment Laboratory (NTSEL),

Japan Automobile Research Institute (JARI),

Japan Automobile Standards Internationalization Center (JASIC).

- Netherlands,

Netherlands Ministry of the Environment (VROM),

TNO Automotive, Delft.

- Spain,

National Institute of Aerospace Technique (INTA).

- Switzerland,

Swiss Federal Roads Authority (ASTRA), Bern,

Swiss Federal Laboratories for Materials Testing and Research (EMPA), Dubendorf,

University of Applied Sciences, Lab. for Exhaust Emission Control (HTA), Biel.

- United Kingdom,

Department of Transport,

Ricardo Ltd.

- United States of America (USA),

US Environmental Protection Agency.

- Association for Emissions Control by Catalyst (AECC).

- Federation of European Motorcyclists Association (FEMA).

- International Motorcycle Manufacturers Association (IMMA).

With respect to the tight timeframe of the work and in order to increase the efficiency the WMTC group established a subgroup of experts that met more frequently than the WMTC group and prepared all necessary material for the discussions and decisions in the WMTC group.  This subgroup was called WMTC FE (fundamental elements).

2.	OBJECTIVE

The objective of the research program is to develop a worldwide-harmonized motorcycle emissions test procedure, consisting of:

-	a test cycle,

-	a gearshift procedure,

-	sampling, measurement and analysis procedures (with support from/in collaboration with ISO)

The test procedure needs to be:

-	representative of world-wide on-road vehicle operation,

-	able to provide the highest possible level of efficiency in controlling on-road emissions,

-	corresponding to state-of-the-art testing, sampling and measurement technology,

-	applicable in practice to existing and foreseeable future exhaust emissions abatement technologies,

-	capable of providing a reliable ranking of exhaust emission levels from different engine types,

-	consistent with the development of appropriate emission factors,

-	inclusive of adequate cycle-bypass prevention provisions.

The test procedure has to cover the cycle and the accompanying gearshift procedure for the test bench measurements and the prescription of test bench settings like determination of road load resistance, inertia mass, cooling requirements, exhaust gas sampling procedure and other test bench specifications.

3.	STRUCTURE OF THE PROJECT

The development of the cycle and the gearshift procedure belongs to the tasks of the WMTC group; the prescription of test bench settings was developed in working group 17 of ISO TC22 in close liaison with the WMTC group.  Table 1 gives an overview of the tasks of the whole project.

In the work schedule of the WMTC group two validation steps were foreseen after the development of test cycle and gearshift procedure.  A first step, in which the driveability was evaluated and a second step, in which the emissions measurement results were evaluated and compared with results from existing certification procedures.  The development work and the two validation steps are finished.  Finally, a round robin test was carried out starting in Spring 2003.

Table 1: The Structure of the whole project

4.	CYCLE DEVELOPMENT

4.1.	Approach

The basis of the cycle development was the collection and analysis of driving behaviour data and statistical information about motorcycle use for the different regions of the world.  These data had to include all relevant real life vehicle operations and built the basis for the cycle development.  In a second step the in-use driving behaviour data were combined with the statistics on vehicle use in order to create a reference database that is representative for worldwide motorcycle driving behaviour.  This was achieved using a classification matrix for the most important influencing parameters.  In the final classification matrix three different regions (Europe, Japan, USA), three different vehicle classes and three different road categories were included.

The next step was to compact this reference cycle into a test cycle of the desired length.  A computer search programme then selects a number of modules (speed/time sequences between two stops) to represent by approximation this length.  The statistical characteristics of this number of modules are then compared to those of the database.  The comparison is done on the basis of the chi-squared method, an accepted statistical criterion.

Finally, a first draft of the World-wide Motorcycle Test Cycle (WMTC) was produced.  It was foreseen that this first draft needed to be modified on the basis of an evaluation concerning driveability and practical points concerning the measurement procedure.  Since this process is iterative by nature, several adaptation rounds including the driveability tests were carried out.

A flow chart of the development process is shown in Table 2.

Table 2: Flow chart of the cycle development work

4.2.	In-use driving behaviour data

The basis of the cycle development was the collection and analysis of driving behaviour data and statistical information about motorcycle use for the different regions of the world.  These data had to include all relevant real life vehicle operations and built the basis for the cycle development.  In a second step the in-use driving behaviour data were combined with the statistics on vehicle use in order to create a reference database that is representative for worldwide motorcycle driving behaviour.  This was achieved using a classification matrix for the most important influencing parameters.  In the final classification matrix three different regions, three different vehicle classes and three different road categories were included.

The in use driving behaviour data used for the WMTC project consist of the following subsets:

- Data measured in Europe

1994, ACEM-group. The measurements were carried out in Paris and Pisa.

1994, JAMA-group. The measurements were carried out in Amsterdam and Frankfurt.

1999, ACEM. The measurements were carried out in the area around Pisa (Italy), the area around Mandeure (France) and the area around Munich (Germany).

Technische Fachhochschule Biel in Switzerland, in and around Biel.

Technical University of Darmstadt (Germany), around Darmstadt.

- Data measured in Japan

1992, JMOE project. The measurements were carried out in the Tokyo area.

1997, JAMA project. The measurements were carried out in the Tokyo area.

2000, JAMA/JARI project. The measurements were carried out in the Tokyo area including highway. The data is only used for the gearshift model.

- Data measured in China

In the Ji Nan area by the Tianjin Motorcycle Technical Center of the Tianjin Internal Combustion Engine Research Institute, belonging to the Tianjin University in China.

- Data measured in the USA

1999, USMMA run. The measurements were carried out in Birmingham, Alabama.

The time duration and the total mileage of these in-use data subsets are shown in Table 3.   gives an overview about the vehicle sample.

Table 3: Duration and mileage of the in-use data subsets

Table 4: The vehicle sample of the in-use driving behaviour database

4.3.	Fleet composition and vehicle use

The content of this chapter is adopted from [1].

Vehicle Stock

From the industrial partners statistical data have been received as to the respective local vehicle fleets in different regions of the world.  One should bear in mind, however, that the available data are not very "hard".  One obvious problem with statistics from widely different sources is that usually they are not coherent.  It has been attempted to limit the data to motorcycles of more than 50 cc, although it is not always clear whether the available statistics include or exclude the class below 50 cc.  From an evaluation of the statistical data received, sufficient conclusions can nevertheless be derived as to the composition of the vehicle fleet in different regions of the world.

-	Europe has a fleet that mainly consists of either small capacity vehicles, mainly scooters, on the one hand, and of large sports and touring bikes on the other, with few vehicles in between. The distribution is very country dependant.  Generally speaking northern countries possess a large percentage of big bikes, whereas southern countries tend to have mainly small vehicles.

-	Japan has a similar share of small vehicles as Europe, but has a more or less equal number in the middle category.  Really big bikes are rare in Japan.

-	The USA has mainly big bikes (most of them above 750 cc) and very few in the small and middle categories.

The situation is shown in Table 5.

Table 5:	The composition of the fleet in Europe, Japan and the USA in 1997.  Note that the subdivision is not equivalent for all three regions, due to the structure of the available data. (from [1])

The total fleet size for Europe (that is including non-European Union Member States Switzerland and Norway), Japan and USA and the total annual mileage are shown in the following figure.

Table 6:	The total fleet size in the three regions, numbers of vehicles (left) and total annual mileage (right), (from [1])

For Japan and the USA no figures about scooters are available.

Vehicle Use

One obvious problem when comparing the use on different road types in different parts of the world, is that the definitions, and often even the road types themselves, do not compare on a 1:1 basis.  So any comparison can only be approximate.  The following information was supplied by the IMMA members.

The characteristic use in Europe depends heavily on the class and the country.  According to a user’s survey by a major scooter manufacturer the small machines (mainly scooters) are almost exclusively used in an urban environment.  The average trip length is small.  This is the main pattern in south-European countries.  According to a survey by the TU Darmstadt [2] the big machines are mainly used for recreational trips.  The average trip length is large.  This use is mainly on country roads, because the users are enthusiasts that like a challenging route.  This agrees with available information from the Netherlands and it seems to be the dominant use in north-European countries.

Table 7: Characteristic use per type of road (from [1], but corrected for US data)

The information that is available about Japan suggests that the majority of the mileage is performed on secondary roads, including urban roads.  The maximum speed on country roads is 60 km/h.  On motorways since October 2000 the posted speed is 100 km/h; before that date it was 80 km/h.  Vehicles below 125 cc are not allowed on motorways, but even for bigger bikes the motorway share is small.  Commuting and shopping are important motives in Japan.  For the class < 125 cc ‘business’ is an important motive as well (26 per cent), whereas for the classes above 125 cc recreational use is an important motive (34 per cent for the class 125-250 cc, and 49 per cent for the class > 250 cc).  The total annual mileages are low, suggesting small average trip lengths and low average speeds.

In the USA there is a larger share of urban use than in Europe and a smaller share of country roads.  The use of motorways is roughly equal to that in Northern Europe.  The annual mileages are relatively low and so are, presumably, the trip lengths.

Annual mileages are difficult to obtain, but the following table gives some indications.

| Class | Region | Annual mileage |
| --- | --- | --- |
| Engine capacity < 150 cm³ | Europe Japan USA | 2500 – 5000 km Approx. 1400 km 3000 – 3500 km |
| Engine capacity 150 – 450 cm³ | Europe Japan USA | Approx. 3500 km Approx. 2300 km 3750 – 4250 km |
| Engine capacity > 450 cm³ | Europe Japan USA | Approx. 5000 km Approx. 2400 km 5000 – 5500 km |

Table 8:	Approximate annual mileages per capacity class and region (from [1], but corrected for USA data)

4.4.	The reference database

It was originally planned to create the reference database by combining the in-use driving behaviour data with the statistics on vehicle use (see Table 9).  This should be done using a classification matrix for the most important influencing parameters.  In the classification matrix three different regions, three different vehicle classes and three different road categories should be included.

The reference database would then have been a combination of representative in-use data expressed in terms of vehicle speed for each cell of the classification matrix and with the corresponding weighting factors.

But since the data about vehicle use were not reliable enough and since the WMTC group could not find a compromise for the vehicle classification the following alternative approach was chosen:

The cycle should be designed as consisting of three parts, each part representing a separate road category.  Part 1 should be a low speed part, mainly representative for urban traffic; part 2 should be a medium speed part and represent slower country road type of traffic, and part 3 should be a high-speed part and represent faster country roads and motorways.  Part 1 should incorporate a cold start.

Table 9	Flowchart for the construction of a test cycle out of a large randomly distributed database, from [1].

By measuring the emissions separately for each part, this approach allows to perform validation tests with a provisional vehicle classification and take into account vehicle use statistics by applying weightings to the results for each part.  Consequently the in-use data were separated into 3 classes corresponding to the above-mentioned 3 road categories.

Since information of road categories was not available for all in-use data and since the classification may vary from region to region, it was attempted to characterise the road categories, for a vehicle for which these categories were known, on the basis of the length of vehicle speed modules and the average speed of these modules.  A module is a speed sequence between two stops.  This was not very successful, partly because of the routes chosen.  A second attempt on the basis of average speed and maximum speed per module looked more promising.  There was a reasonable distinction between the road categories.  The main overlap was between fast country and motorway traffic, but this was acceptable, as both categories should build one single class.

Eventually an approach was chosen that characterises the modules on the basis of speed distribution. This approach was used earlier for a similar problem in characterising the road use of hybrid driven passenger cars.  To this end for every module the share of speeds was calculated below 60 km/h, between 60 and 90 km/h and above 90 km/h.  Subsequently the following allocation was used:

Table 10: Allocation of driving types for the three cycle parts

This approach was selected as a pragmatic solution to the problem.  The additional requirement of Part 1 was included to prevent "creeping" from being selected as a representative driving mode.  Subsequently this way of allocation was applied to all data.

The second step then was to compact these reference cycles into test cycle parts of the desired length in time, which was set to 600 s for each part.  A computer search programme was the developed by TNO that selected a number of modules (speed/time sequences between two stops) to represent by approximation this length.  The statistical characteristics of this number of modules were then compared to those of the corresponding database.

The comparison was done on the basis of the chi-squared method for the acceleration versus speed matrix (v*a-matrix).  The selection of modules with the lowest value for chi-squared was then selected as the ideal combination.  So first the ideal lengths of the various modules were determined, and then the most representative modules corresponding with those lengths were selected.  After the module selection the stops were added.  The total stop time was taken from statistics.  This total stop time was then divided into stops on the basis of the statistical distribution of the stop lengths in the in-use database.  Finally a first draft of the World-wide Motorcycle Test Cycle (WMTC) was produced.

The cycle development work was done in close cooperation between JARI, TNO and RWTUEV.  JARI contributed with an extensive in-use data analysis with respect to cycle development work, dealing with statistical distributions of vehicle speed, idling time, trip length, acceleration, deceleration and cruising phases (see [5] and [6]).

4.5.	Modifications of the draft test cycle and final version

It was foreseen that the first draft needed to be modified on the basis of an evaluation concerning driveability and practical points concerning the measurement procedure.  Since this process is iterative by nature, several adaptation rounds including the first step of the validation programme were carried out.

The following modifications were made during the validation phase and the driveability tests:

-	Deletion of an ultra short module with operational speeds below 20 km/h in part 1,

-	Separation of an extremely long module in part 1 into 3 parts that were more representative for urban driving,

-	Replacement of 3 part 1 modules by more representative modules,

-	Correction of the idle time distribution for part 1,

-	Rearrangement of the rank order of the modules in part 1 with respect to cold start requirements,

-	Smoothing of ripples in the quasi-constant cycle phases of parts 2 and 3 to delete unrealistic fluctuations in the speed signal caused by the speed measurement uncertainty,

-	The maximum speed of part 3 was set to 125 km/h,

-	The acceleration time pattern was smoothed in order to delete unrealistic high changes of the acceleration rate that could cause driveability problems like tyre slip,

-	The highest deceleration values were reduced to take into account that only the drive wheel brake is working on the roller bench.

-	To reduce the risk for tyre slip the da/dt values were limited to:

-0.8 m/s²/s <= da/dt <= +0.8 m/s²/s.

Consequently the vehicle speed pattern were modified until da/dt falls into that range.

-	A special part 1 cycle, called "part 1, 50 cc" was created for low powered motorcycles, whose technical specifications are close to mopeds, by limiting the top speed of part 1 to 50 km/h and limiting the acceleration/deceleration values to +2/-2 m/s².

Although version 7 of the WMTC cycle was based on a smoothed acceleration pattern to avoid excessive changes in acceleration over time, the question of tyre slip was raised again at the WMTC meetings in Madrid (17.12.2001) and Geneva (16.01.2002).  Consequently an additional analysis of this problem was executed.  This analysis was based on the results of 4 vehicles (2 from the US and 2 from Japan), for which second by second data of roller speed as well as tyre speed had been delivered for the WMTC as well as for the US-FTP and TRIAS/ECE Regulation No. 40.  The latter was only measured in Japan.  The Japanese vehicles belong to Pclass 1 and 2, the US vehicles to Pclass 3 (see chapter 9.).  The word "Pclass" stands for the provisional classification used for the emissions validation tests (see chapter 7.).

The tyre slip was calculated as difference between tyre speed and roller speed divided by the roller speed and expressed in per cent.  That means tyre slip gives positive values, tyre lock negative values.  In addition the change of acceleration rate per second was calculated for the acceleration values of the set speed.

The majority of tyre slip values >= 50 per cent occurred during the transition from standstill to driving, surprisingly not only for the WMTC but also for the US-FTP and the TRIAS/ECE Regulation No. 40.  The analysis results show that the dynamics of the WMTC do not cause higher risks for tyre slip than existing certification cycles.  As already concluded from interim results and the analysis of the answers to the questionnaire about the tyre slip, the problem is related to individual tyre/roller combinations (see chapter 6.).  Nevertheless it was decided in the WMTC group that the tyre slip risk should be minimised for the WMTC by limiting the change of acceleration rate in order to improve driveability, independent of the situation for existing cycles.

During the WMTC FE meeting in Tokyo in April 2002, the Japanese delegation proposed to create a special version of part 1 with a top speed of 50 km/h for low powered motorcycles whose technical specifications are close to mopeds.  Since there is only one module in part 1 with vehicle speeds above 50 km/h (module 3), only this module was modified.  The first part was replaced by a module taken from in-use measurements of vehicle 6 (this module was closest to the existing one) and the second part was lowered in speed to fulfil the limitation of 50 km/h.

The compromise found on vehicle classes made it necessary to create additional cycle versions with reduced speed also for part 2 and part 3.  In order to achieve that the dynamics of the cycle parts with reduced speed were only changed slightly compared to the normal versions, the speed pattern was not simply limited by the reduced maximum but the whole module containing the maximum speed was lowered at its first acceleration phase and its last deceleration phase (e.g. see Table 13.  For consistency reasons this approach was then also applied to part 1, so that the second version of part 1, reduced speed is slightly different from the first version.

All modifications done in the adaptation rounds are described in detail in 15. ANNEX A.  Tables containing version 9 are shown in 16. ANNEX B – FINAL CYCLE VERSION.

The final result is shown in Table 11 to Table 13.  Each part is 600 s long. Part 1 representing urban driving consists of 8 modules interrupted by standstill at idling.  The top speed is 60 km/h in the normal version and 50 km/h in the reduced speed version.  Part 2 representing driving on secondary rural roads consists of 2 modules, the top speed is 95 km/h in the normal version and 85 km/h in the reduced speed version.  Part 3 representing primary rural roads and motorways consists of 1 module, the top speed is 125 km/h in the normal version and 111 km/h in the reduced speed version.

The characteristics of the WMTC driving cycle(s) are shown in Table 14 and Table 15.  The definitions for the driving modes are given in Table 16.

Table 11: WMTC driving cycle, part 1

Table 12: WMTC driving cycle, part 2

Table 13: WMTC driving cycle, part 3

Table 14: Characteristics of WMTC driving cycle (1)

Table 15: Characteristics of WMTC driving cycle (2)

Table 16: Definition of driving modes

5.	GEARSHIFT PROCEDURE DEVELOPMENT

5.1.	Approach

The development of the gearshift procedure was based on an analysis of the gearshift points in the in-use data.  In order to get generalised relations between technical specifications of the vehicles and gearshift speeds the engine speeds were normalised to the utilisable band between rated speed and idling speed.

In a second step the end speeds (vehicle speed as well as normalised engine speed) for upshifts and downshifts were determined and collected in a separate table.  The averages of these speeds for each gear and vehicle were calculated and correlated with technical specifications of the vehicles.

A flow chart of the development procedure is shown in Table 17.

Table 17: Flow chart of the gearshift prescription development

The results of these analyses and calculations can be summarised as follows:

-	The gearshift behaviour is engine speed related rather than vehicle speed related.

-	The best correlation between gearshift speeds and technical data was found for normalised engine speeds and the power to mass ratio (rated power/(kerb mass + 75 kg), see Table 18.

-	The residual variations cannot be explained by other technical data or by different transmission ratios.  They are most probably assigned to differences in traffic conditions and individual driver behaviour.

-	The best approximation between gearshift speeds and power to mass ratio was found for exponential functions, see Table 18.

-	The gearshift function for the first gear is significantly lower than for all other gears.

-	The gearshift speeds for all other gears can be approximated by one common function.

-	No differences were found between 5-speed and 6-speed gearboxes.

-	The gearshift behaviour in Japan is significantly different from the equal-type gearshift behaviour in Europe and in USA, see Table 18.

In order to find a balanced compromise between the three regions a new approximation function for normalised up shift speeds vs. power to mass ratio was calculated as weighted average of the Europe/USA curve (with 2/3 weighting) and the Japanese curve (with 1/3 weighting), see Table 19.

Table 18:	Correlation between normalised max engine speeds for upshifts and power to mass ratio

Table 19:	The final approximation function for upshift speeds in gears higher than 1 (1/3 Europe, 1/3 Japan, 1/3 USA)

5.2.	Gearshift criteria, additional requirements

Based on this, the gearshift prescriptions can be summarized as follows:

For acceleration phases manual transmissions shall be shifted from 1. to 2. gear when the engine speed reaches a value according to the following formula:

| n_max_acc (1)=( 0.5753×e (−1.9×Pnmk+75)−0.1 )×(s−n idle)+n idle | Equation 1 |
| --- | --- |

Upshifts for higher gears have to be carried out during acceleration phases when the engine speed reaches a value according to the following formula:

| n_max_acc (i)=( 0.5753×e (−1.9×Pnmk+75) )×(s−n idle)+n idle | Equation 2 |
| --- | --- |

Pn - rated power in kW

mk – kerb mass in kg

n – engine speed in min-1

nidle – idling speed in min-1

s - rated engine speed in min-1 at maximum power

i – gear number (>= 2)

The minimum engine speeds for acceleration phases in gear 2 or higher gears are accordingly defined by the following formula:

| n_min_acc (i)=n_max_acc (i−1)×r(i)r(i−1) | Equation 3 |
| --- | --- |

r(i) – ratio of gear i

The minimum engine speeds for deceleration phases or cruising phases in gear 2 or higher gears are defined by the following formula:

| n_min_dec (i)=n_min_dec (i−1)×r(i)r(i−1) | Equation 4 |
| --- | --- |

r(i) – ratio of gear I

When reaching these values during deceleration phases the manual transmission has to be shifted to the next lower gear (see Table 20).

The engine speed values resulting from the formulas above can be rounded to multiples of 100 min-1 for practical applications.

Table 20 shows an example for a gearshift sketch for a small vehicle.  The solid lines demonstrate the gear use for acceleration phases; the dotted lines show the downshift points for deceleration phases.  During cruising phases the whole speed range between downshift speed and upshift speed may be used.

Table 20:	Example of a gearshift sketch for a small vehicle

In order to avoid driveability problems these prescriptions had to be supplemented by the following additional requirements, (some of them are general, some are assigned to particular cycle phases):

-	There are fixed allocations for acceleration, cruising and deceleration phases (see annex B).

-	Gearshifts are prohibited for indicated cycle sections (see annex B).

-	No gearshift if a deceleration phase follows immediately after an acceleration phase.

-	Idle modes shall be run with manual transmissions in the first gear with the clutch disengaged.

-	Downshifts to the first gear are prohibited for those modes, which require the vehicle to decelerate to zero.

-	Manual transmissions gearshifts shall be accomplished with minimum time with the operator closing the throttle during each shift.

-	The first gear should only be used when starting from standstill.

-	For those modes that require the vehicle to decelerate to zero, manual transmission clutches shall be disengaged when the speed drops below 10 km/h, when the engine speed drops below nidle + 0.03*(s – nidle), when engine roughness is evident, or when engine stalling is imminent.

-	While the clutch is disengaged the vehicle shall be shifted to the appropriate gear for starting the next mode.

-	The minimum time span for a gear sequence is 2 seconds.

To give the test engineer more flexibility and to assure driveability the gearshift regression functions should be treated as lower limits.  Higher engine speeds are permitted in any cycle phase.

These criteria and additional requirements were used to calculate the gearshift schedules for the test vehicles in the emissions validation programme.  The definitions for acceleration, deceleration and cruising phases are given in Table 16.

5.3.	Gearshift prescriptions

5.3.1.	Step 1 – Calculation of shiftspeeds

Calculate upshift and downshift speeds for all gears according to the following formulas:

Upshift speeds in km/h during acceleration phases:

| v 1 → 2= (0.5753×e (−1.9×Pnmk+75)−0.1 )×(s−n idle)+n idle1ndv1 | Equation 5 |
| --- | --- |

| v i → i+1=( 0.5753×e (−1.9×Pnmk+75))×(s−n idle)+n idle1ndv1 | , i = 2 to ng-1 |
| --- | --- |
|  | Equation 6 |

Where

i is the gear number (>= 2),

ng is the total number of forward gears,

Pn is the rated power in kW,

mk is the kerb mass in kg,

n is the engine speed in min-1,

nidle is the idling speed in min-1,

s is the rated engine speed in min-1,

ndvi is the ratio between engine speed in km/h and vehicle speed in min-1 in gear i.

Downshift speeds in km/h during cruise or deceleration phases in gears 3 to n:

| v i → i−1= (0.5753×e (−1.9×Pnmk+75) )×(s−n idle)+n idle1ndvi-2 | , i = 3 to ng-1 |
| --- | --- |
|  | Equation 7 |

the gear lever shall set to first gear and the clutch shall be disengaged, if:

-	the vehicle speed drops below 10 km/h, or

-	the engine speed drops below nidle + 0.03*(s – nidle),

-	engine roughness is evident,

-	engine stalling is imminent.

5.3.2.	Step 2 – Gear choice for each cycle sample

Calculate the appropriate gear for each sample according to phase indicators in the tables in annex B for those cycle parts that are appropriate for the test vehicle, as follows:

5.3.2.1.	Gear lever in neutral and clutch disengaged

The gear lever shall be set to neutral and the clutch shall be disengaged, if the following conditions are met:

-	During stop phases,

-	During cruise or deceleration phases in second gear, if

the vehicle speed drops below 10 km/h, or

the engine speed drops below nidle + 0.03*(s – nidle).

5.3.2.2.	Gear choice for acceleration phases

Gear = 6, if v > v5→6,

Gear = 5, if v > v4→5,

Gear = 4, if v > v3→4,

Gear = 3, if v > v2→3,

Gear = 2, if v > v1→2,

Gear = 1, if v <= v1→2.

5.3.2.3.	Gear choice for deceleration or cruise phases

Gear = 6, if v > v4→5,

Gear = 5, if v > v3→4,

Gear = 4, if v > v2→3,

Gear = 3, if v > v1→2,

Gear = 2, if v <= v1→2.

5.3.3.	Step 3 – Corrections according to additional requirements

The gear choice has then to be modified according to the following requirements:

a)	No gearshift at a transitions from an acceleration phase to a deceleration phase: keep the gear that was used for the last second of the acceleration phase also for the following deceleration phase unless the speed drops below a downshift speed.

b)	No upshifts during deceleration phases.

c)	No gearshift in cycle phases, where "no gearshift" is indicated.

d)	No downshift to first gear at a transition from a deceleration or a cruise phase to an acceleration phase, if "no 1. gear" is indicated.

e)	If a gear is used for only one second, this gear shall also be assigned to the following second.  Since it could happen that the modifications according to this criterion create new phases where a gear is used for only one second, this modification step has to be applied several times.

RWTUEV developed Excel calculation sheets for 3 to 6 speed gearboxes and both cycle versions (normal and reduced speed) as an assistant for test engineers.  These sheets will be available on the UNECE/WP29 website: http://www.unece.org/trans/main/wp29/wp29wgs/wp29grpe/wmtc.html.

5.4.	Calculation example

An example of input data necessary for the calculation of shift speeds is shown in Table 21.  The upshift speeds for acceleration phases for the first gear and higher gears are calculated with equations 1 and 2.  The denormalisation of engine speeds can be executed by using the equation:

n = n_norm * (s – n_idle) + n_idle.

The downshift speeds for deceleration phases can be calculated with equation 4.  The n/v values in Table 22 can be used as gear ratios.  These values can also be used to calculate the affiliated vehicle speeds (vehicle shift speed in gear i = engine shift speed in gear i / n/v_i).  The corresponding results are shown in Table 22 and Table 23.

| Engine capacity in cm3 | 600 |
| --- | --- |
| Pn in kW | 72 |
| mk in kg | 199 |
| s in min-1 | 11800 |
| nidle in min-1 | 1150 |
| ndv1 *) | 133.66 |
| ndv2 | 94.91 |
| ndv3 | 76.16 |
| ndv4 | 65.69 |
| ndv5 | 58.85 |
| ndv6 | 54.04 |
| pmr **) in kW/t | 262.8 |
| *) ndv means the ratio between engine speed in min-1 and vehicle speed in km/h **) pmr means the power to mass ratio calculated by Pn / (mk+75) 1000, Pn in kW, mk in kg |  |

Table 21:	Input data for the calculation of engine and vehicle shift speeds

|  | EU/USA/Japan driving behaviour |  |
| --- | --- | --- |
|  | n_acc_max (1) | n_acc_max (i) |
| n_norm*) | 24.8 per cent | 34.8 per cent |
| n in min-1 | 3804 | 4869 |
| *) n_norm means the calculated value by equation 1 and equation 2. |  |  |

Table 22:	Shift speeds for acceleration phases for the 1. gear and for higher gears (according to Table 21)

Table 23:	Engine and vehicle shift speeds according to Table 22

In a further step the possibility of a simplification of the above-described gearshift algorithms was examined by additional analyses and calculations.  It should especially be checked whether engine shift speeds could be replaced by vehicle shift speeds.  The analysis showed that vehicle speeds could not be brought in line with the gearshift behaviour of the in-use data.

6.	DRIVEABILITY TESTS

Right after the WMTC cycle and the corresponding gearshift procedure were developed measurements were carried out on roller benches in order to validate the driveability of the cycles and the functionality of the gearshift procedure.

In order to run these tests a test protocol was developed on the basis of the test protocol of the USA certification procedure FTP.  Some modifications were necessary due to the different design of the WMTC cycle (higher vehicle speeds, 3 parts) and the gearshift procedure.  The gearshift protocol represented EU/USA gearshift behaviour because the Japanese gearshift behaviour database was not complete at that time.

In total the results (roller speed data) of 27 vehicles were delivered, 18 from Europe, 6 from Japan and 2 from the USA.

The driveability problems that were reported can be focussed on tyre slip, wheel lock and traceability problems due to poor power in case of low power vehicles.  In addition some malfunctions of the gearshift calculation sheet were detected.

The problem with tyre slip and traceability was biggest for part 1 and smallest for part 3.  It is obviously related to the individual roller-tyre combination.  An additional questionnaire was sent out to all participants in order to get further information about the roller benches as well as about the tyres used for the measurements.  The results of the questionnaire show big variations in technical specifications (diameter, maximum power, maximum speed) of the roller benches.  The tyre slip problem could neither be related to tyre type or size nor to vehicle specifications.  It is tyre related rather than roller bench related but the available tyre information is not suitable to show a clear picture about the influencing parameters.  No tyre slip problems were reported for roller benches with "textured" surfaces.  The results of the questionnaire do not suggest further reductions of the cycle dynamics during acceleration phases.  The risk of tyre slip can be reduced by textured roller surfaces.

The wheel lock problem is partly caused by the fact that only the rear wheel break can be used during deceleration on a roller test bench and partly related to the tyre characteristics like the tyre slip problem.  As there is no significant influence of the deceleration on the emissions the wheel lock problem can be reduced by a reduction of the deceleration values.

The poor power problem can only be avoided when the cycle dynamics are adjusted to the vehicles at the lower end of the power to mass ratio scale.  But this solution would not be in line with practical use.  Nevertheless countermeasures to the poor power problem should be further investigated.

7.	VEHICLE CLASSIFICATION

The vehicle classification is one of the important issues of the WMTC development process. For practical reasons the following first provisional classification of vehicles was made by TNO within the frame of the cycle development work:

C-class I:	vehicles with an engine capacity of 	< 150 cm³

C-class II:	vehicles with an engine capacity of 	150 cc – 450 cm³

C-class III:	vehicles with an engine capacity of 	> 450 cm³

During the analysis of the driveability validation test results some conflicts with practical use and technical possibilities revealed for this classification.  On one hand there is a series of vehicles on the European market that would be classified as C-class I or C-class II although their maximum speed exceeds the maximum speed of the corresponding cycle parts substantially (see Table 24).  On the other hand there are also vehicles of C-class 2 and C-class 3 whose maximum speed is below the maimum speed of the corresponding cycle parts.

Table 24:	Vehicle type distribution for engine capacity and maximum vehicle speed classes (data source: KBA statistics of type approval values)

For that reason some alternatives based on power to mass ratio and maximum vehicle speed as substitute or additional criteria were discussed but the FE-group could not find a compromise.  It was decided to postpone a final decision till the analysis of the emissions validation test results will have been finished.

For the emissions validation tests the following provisional classification was used in order to get as much information as possible about the "border" areas:

Pclass 1:	vehicles with a maximum speed of less than 80 km/h,

Pclass 2:	vehicles with a maximum speed of 80 km/h or higher but less than 120 km/h,

Pclass 3:	vehicles with a maximum speed of 120 km/h or higher

Maximum speed is the maximum vehicle speed as declared by the manufacturer.

Since the existing classifications for motorcycle type approval in all three regions use engine capacity classes, the FE group reached an agreement that the classification system should be based on engine capacity and maximum vehicle speed.

During interim discussion phases it was agreed that the lower limit for class I is formed by engine capacity of more than 50 cm³ and maximum speed of more than 50 km/h, that special class I is defined by engine capacity of up to 50 cm³ and maximum speed of more than 50 km/h but not more than 60 km/h.

But there was no agreement for maximum speed borders between the classes 1, 2 and 3 at that stage of the project.  The proposals for a maximum speed border between class 1 and class 2 on one hand and class 2 and class 3 on the other hand that were discussed were justified by two contrasting philosophies about the driveability capabilities.  Some members of the group argued that the classification must be defined in a way that the traceability is guaranteed for any cycle part, while some others would accept deviations from the cycle trace resulting in full throttle operation for about 10 per cent of the total cycle time in order to reproduce the practical use and in order to cover a wider area of the engine map.  These contrary philosophies resulted in different proposals for maximum speed borders between class 1 and 2 (80 km/h versus 120 km/h) and class 2 and class 3 (120 km/h versus 140 km/h).

As part of the United Kingdom's contribution to the validation work of the WMTC working group, 12 motorcycles were tested by Ricardo over a near final version of the cycle.  In the main, this work looked at machines that would be subject to all three parts of the new cycle.  It was observed that motorcycles operating at or near their maximum speed would be subject to excessive and unrepresentative operation on full throttle.  This would be necessary to achieve both the required speed and acceleration of the cycle.  This observation suggests that there should be a reasonable allowance between the vehicle's potential maximum speed and the speed at which it is tested.

The United Kingdom proposed that a factor of 0.85*vmax for the maximum cycle speed would ensure an appropriate allowance.  In order to smooth the transition between the vehicle classes, the United Kingdom recommended to develop cycles with reduced top speeds for all parts and use them for those vehicles whose speed factor (0.85*vmax) is below but close to the maximum speed of the original cycle part.

This approach was taken by the WMTC group and discussed.  As the outcome of the discussion the following compromise was found in connection with additional cycle versions with reduced speeds for all cycle parts (see chapter 4.5.):

Class 1:

Vehicles that fulfil the following specifications belong to class 1:

engine capacity <= 50 cm³ and 50 km/h < vmax < 60 km/h	subclass 1-1,

50 cm³ < engine capacity < 150 cm³ and vmax < 50 km/h 	subclass 1-2,

engine capacity < 150 cm³ and 50 km/h <= vmax < 100 km/h

but not including subclass 1-1 	subclass 1-3.

vmax is the maximum vehicle speed.

Class 2:

Vehicles that fulfil the following specifications belong to class 2:

engine capacity < 150 cm³ and 100 km/h <= vmax < 115 km/h or

engine capacity >= 150 cm³ and vmax < 115 km/h	subclass 2-1,

115 km/h <= vmax < 130 km/h	subclass 2-2.

vmax is the maximum vehicle speed.

Class 3:

Vehicles that fulfil the following specifications belong to class 3:

130 <= vmax < 140 km/h	subclass 3-1,

vmax >= 140 km/h	subclass 3-2.

vmax is the maximum vehicle speed.

Table 25 gives an overview of the vehicle classification in terms of engine capacity and maximum vehicle speed.

Table 25: vehicle classification

8.	WEIGHTING FACTORS FOR AN OVERALL WMTC EMISSIONS RESULT

Another open question besides the vehicle classification is the calculation of an overall emissions result for class 2 and class 3 vehicles.  As already mentioned in chapter 4.4., it is necessary to apply weightings to the emissions results of the different cycle parts in order to calculate an overall result that reflects the statistics of vehicle use, because otherwise the equal time length of 600 s for each cycle part leads to a coincidental weighting of 14 per cent for part 1, 31 per cent for part 2 and 55 per cent for part 3 that is not in line with practical use.

At the 8th WMTC FE meeting in Ann Arbor, JARI presented a calculation method that is based on the following in-use parameters:

- annual mileage for different road category,

- average one trip (engine start to stop) distance,

- equivalent cold start ratio

The equivalent cold start ratio is estimated by the soak time distribution and cold start factors.  The cold start factor is 100 per cent for soak times > 6 hours and is 0 per cent when the coolant temperature reaches the temperature at hot condition.  The equivalent cold start ratio is calculated by summarising the soak time frequency (derived from traffic surveys) multiplied with the cold start factor.

The weightings for the different cycle parts are derived from the annual mileage for different road categories.  The weighting for part 1 is then splitted into a weighting for part 1, cold and part 1, hot.  The weighting for part 1, cold is derived from the average one trip distance, the distance driven in part 1 and the equivalent cold start ratio.  The weighting for part 1, hot is the difference between the total weighting for part 1 (derived from the annual mileage for urban use) and the weighting for part 1, cold.

The FE group agreed on the use of the Japanese approach for the weighting calculation.  The Japanese delegates made also a proposal for a simplification in order to skip the measurement of part 1, hot for class 2 and class 3 vehicles, which was accepted by the majority of the FE group.

For Japan the statistical data needed for the weighting factor calculation were derived by national traffic monitoring and inventories.  IMMA investigated similar data for Europe and the US.  RWTUEV was asked to assess the statistical data delivered by different parties and work out a comparison of the results for weightings to be applied on the WMTC cycle parts based on averages of the 3 regions EU, USA, Japan.  This comparison is shown in Table 26.

Table 26: Comparison of different proposals for weighting factors

In a discussion between experts from the WMTC group, the EU Commission, Member States and NGO’s in late December 2002 it was agreed that the main problem of the weighting factor discussion is the lack of reliable statistical data for average trip length and mileage distribution for road categories.  Nevertheless, the previous table shows a bias towards part 2.  Therefore it was proposed to use simplified weighting values as shown in the following table:

| Vehicle class | Cycle | Weighting |  |
| --- | --- | --- | --- |
| Class 1 | Part 1, cold | w1 | 50 per cent |
|  | Part 1, hot | w1hot | 50 per cent |
| Class 2 | Part 1, cold | w1 | 30 per cent |
|  | Part 2, hot | w2 | 70 per cent |
| Class 3 | Part 1, cold | w1 | 25 per cent |
|  | Part 2, hot | w2 | 50 per cent |
|  | Part 3, hot | w3 | 25 per cent |

Table 27: Final weighting factors

The weightings have to be applied to the emission results and the fuel consumption as described below:

The emission results in g/km and the fuel consumption in litre/100 km are averaged for each cycle part.  The (averaged) result of part 1 or part 1, reduced speed is named R1, the (averaged) result of part 2 or part 2, reduced speed is named R2 and the (averaged) result of part 3 or part 3, reduced speed is named R3.  Using these emission results in g/km and the fuel consumption in litre/100 km; the final result R, depending on the vehicle class as defined in chapter 7., shall be calculated by means of the following formulas:

| Class 1 | R=R1×w1+R1 hot×w1 hot | Equation 8 |
| --- | --- | --- |
| Class 2 | R=R1×w1+R2×w2 |  |
| Class 3 | R=R1×w1+R2×w2+R3×w3 |  |

for each pollutant, the carbon dioxide emission and the fuel consumption using the weightings shown in Table 27.

9.	EMISSIONS VALIDATION TESTS

9.1.	General information

One important step for the whole project was the emissions validation programme.

The following decisions/recommendations were made with respect to the emissions validation programme by the WMTC group:

a)	Cycle version 7 (the latest version at that time) should be used for the bench tests, part 1 should be measured with cold start first and then repeated at the end in warm condition.

b)	In case of poor power the cycle trace should be followed as good as possible.

c)	The following provisional vehicle classification (indicated in tables and figures as "Pclass") should be used for the emissions validation programme:

-	Cycle part 1 is mandatory for all vehicles,

-	Part 2 should be driven if v_max >= 80 km/h,

-	Part 3 should be driven if v_max >= 120 km/h.

v_max is the maximum vehicle speed as declared by the manufacturer.

d)	The gearshift procedure as described in chapter 5. should be applied.  The use of higher engine speeds were allowed for driveability or practicability reasons.

To be able to start this programme in September 2000 updates of the test protocol, the gearshift procedure and the results delivery format were carried out and distributed amongst the participants of this programme.  21 scooters and 38 motorcycles were announced to be measured within this programme.

The following cycles were mandatory:

-	Draft WMTC cycle for emissions validation programme, latest version,

-	Appropriate regional certification cycle according to the corresponding measurement procedure.

For Europe, the test cycle as described in COM 2000 314 final EU's proposal for amendment of Directive 97/24/EC, section 5.3.1., appendix 1 should be used.  Additional cycles like the European passenger car test cycle (EU Directive 98/69/EC, NEDC) for Europe were recommended.  It was also recommended to measure additional conditions that can be used for off cycle emissions provisions.

The following guidelines were given for the road load settings:

It was recommended to perform coast down measurements on the road and use the results for the specification of load settings.  If coast down measurements were not possible the settings of the US-FTP specifications should be used.  If the maximum speed of a vehicle as declared by the manufacturer was below 130 km/h and this speed could not be reached on the roller bench with the US-FTP test bench settings, they had to be adjusted until the maximum speed was reached.

The following results should be delivered:

-	Technical data of the vehicle including maximum speed,

-	Bag results of the emissions for each part of the WMTC and other test cycles,

-	Roller speed with 1 Hz resolution, drive wheel speed, if possible,

-	Engine speed for vehicles with automatic gearbox (1 Hz resolution),

-	Emissions with 1 Hz resolution,

-	Temperatures at exhaust tailpipe and CVS metering device (1 Hz resolution),

-	Temperature, barometric pressure and humidity of test cell,

-	Humidity of dilution air.

The results of 54 vehicles were delivered:

-	only bag results: 3 vehicles,

-	bag results and roller speed (second by second): 19 vehicles,

-	bag results, roller speed and engine speed (second by second): 8 vehicles,

-	bag results, roller speed and emissions (second by second): 11 vehicles,

-	bag results, roller speed, engine speed and emissions (second by second): 9 vehicles,

-	bag results, roller speed, drive wheel speed and emissions (second by second): 2 vehicles,

-	bag results, roller speed, drive wheel speed, engine speed and emissions (second by second): 2 vehicles

The following problems occurred in some cases:

-	Some participants were not able to measure emissions with 1 Hz resolution,

-	The road load setting requirements were not fulfilled,

-	The provisional vehicle classification was not met,

-	The speed tolerances were not met,

-	Only bag results were delivered,

-	The wrong cycle version was used.  (This vehicle was excluded from the analysis).

But 90 per cent of the results could be used for further analysis.   shows an overview of the vehicle sample distribution for different regions and provisional vehicle classes.  83 per cent of the vehicles were measured in Europe.

Table 28: Vehicle sample for the emissions validation test programme

gives an overview of the distribution of engine type and reduction system within the vehicle sample, Table 30 shows the participating institutes/organisations and Table 31 contains the technical data of the vehicles.

It has to be pointed out that road load settings based on coast down measurements on road were recommended.  If such coast down results were not available the settings of the US-FTP specifications should be used.  But, these settings are no more up-to-date as the ISO/TC22/WG17 has shown on the basis of new measurements in Japan.  In its final report the ISO group presented a new list with updated settings.  The differences between US-FTP and ISO/TC22/WG17 settings are depending on vehicle mass and speed.  For 11 vehicles the road load settings of the US-FTP were not appropriate, most of them are trial and enduro vehicles.  It can be expected that for these vehicles the differences are significant.

Table 29: Distribution of engine type and reduction system within the vehicle sample

Table 30:	Institutes/organisations participating on the emissions validation test programme

Table 31: Technical data of the vehicles

9.2.	Results of the emissions validation programme

The results of the emissions validation tests are given in Table 32 to Table 35 in g/test and in Table 36 to Table 39 in g/km.  The CO2 values are measured values from the exhaust gas.  The gtr (see [4]) provides the calculation procedure for the total CO2 emission directly related to the fuel consumption.

Since the vehicle classification and the weightings were still under discussion, when the validation data analysis was performed, the results for the WMTC are shown for each cycle part separately, no overall WMTC result was calculated and no comparison with the results of the regional cycles was carried out.

NEDC denominates the European passenger car test cycle (EU Directive 98/69/EC), TRIAS denominates the Japanese type approval test cycle that is a modified version of the ECE Regulation No. 40.

A more detailed analysis of the validation test results can be summarised as follows:

The emissions variance is:

-	Dependent of individual engine control technique,

-	Dependent of emission level,

-	Independent of traceability,

-	Not different from others if the maximum roller speed is below the maximum set speed.

High variances in emissions results within each vehicle group (engine type/reduction system) due to individual vehicle design, high overlap in range between different groups

The high variances of the emissions results indicate that there is a substantial optimisation potential in some cases.  As a consequence substantially lower variances can be expected if the reduction systems are optimised in accordance to the WMTC.

Table 32: Results of the emissions validation tests for HC in g/test

Table 33: Results of the emissions validation tests for CO in g/test

Table 34: Results of the emissions validation tests for NOx in g/test

Table 35:	Results of the emissions validation tests for CO2 (measured values from the exhaust gas) in g/test

Table 36: Results of the emissions validation tests for HC in g/km

Table 37: Results of the emissions validation tests for CO in g/km

Table 38: Results of the emissions validation tests for NOx in g/km

Table 39:	Results of the emissions validation tests for CO2 (measured values from the exhaust gas) in g/km

10.	TEST PROTOCOL

An update of the test protocol was worked out on the basis of the state of the discussion in the WMTC group so far and on the basis of the ISO work on updating ISO 11486 (Motorcycles - Chassis dynamometer setting method) and ISO 6460 (concerning the gas sampling and cooling aspects).  This protocol was used for the preparation of the round robin test.  It can now be replaced by the draft global technical regulation.

11.	ROUND ROBIN TEST

The round robin test was started in April 2003.  Three vehicles (one per class) were tested in different laboratories in all three regions.  The aim is to get experience about the interpretation / application of the WMTC test procedure in different laboratories and to get reliable data for the calculation of the reproducibility of the WMTC test procedure.

Eight laboratories were participated in the round robin test, one from Japan, one from the USA and six from Europe.  The results of the WMTC tests are shown in Table 40.  Table 41 shows the results for the ECE Regulation No. 40 test.  Large differences were found between the laboratories for both cycles.

The test result deviations were examined in detail, but examination could not be applied to the ECE cycle test results due to the fact that measurement was done only once in the ECE cycle round robin test.

Table 40: Results of the round robin test for the WMTC cycle

Table 41: Results of the round robin test for the ECE Regulation No. 40 cycle

The following findings were obtained from the results of the round robin test and its analysis:

a)	The salient outcome of the test was large differences in test results for the WMTC as well as for the ECE Regulation No. 40 among the laboratories that had performed the test.

b)	The one-way layout variance analysis of WMTC test results showed the following :

-	There was no significant difference in repeatability (variance) among the laboratories.

-	There was difference in test results among the laboratories.

c)	The multiple regression analysis of WMTC test results and velocity traceability resulted in the following conclusions:

-	Some of the laboratories failed to keep velocity traceability errors within the tolerance established in the WMTC.

-	In many cases, there was a correlation between THC emission amount and velocity traceability.

-	In addition to velocity traceability, the emission measurement system including the setting of the chassis dynamometer may be considered as an influencing factor for emission test results.

d)	The analysis of emission concentration in time series of laboratories recording different velocity traceability performances resulted in:

-	With the WMTC, THC emission behaviour differed when velocity traceability performances differed.

-	There were differences in total carbon concentration among the laboratories, and these differences may be attributable to discrepancies in the calibration of the emission measurement system including the setting of the chassis dynamometer.

e)	The possible influence of vehicle condition on emission behaviour was suggested by NOx and other emission patterns, hut the available data were not sufficient to allow effective analysis on this possibility.

Data needed to evaluate correlations among laboratories

The discrepancies in the test results of laboratories may be corrected by the methods listed below.  These methods will need to be implemented by each laboratory in order to improve its measurement accuracy.

a)	Errors in the measurement system:

-	Chassis dynamometer:

Removal of systematic errors: Examination into running resistance (examination of dynamo load meter and coast down time);

Removal of accidental errors: Repetition testing (already in action).

-	Exhaust system.

Removal of systematic errors: Calibrate the emission measurement system by propane shooting prior to a round robin test.  Additionally, calibrate the emission measurement system by span gas shooting after each round of the test.

Removal of accidental errors:  Repetition testing (already in action).

b)	Change in vehicle condition:

After setting the running resistance, measure the emission amount (g/km) at idle and constant speeds (for example, at 40, 60, 80, 100 and 120 km/h), and find out changes in the vehicle condition and errors in the emission measurement system.

Removal of systematic errors: Respect the test protocol, and set uniform testing conditions such as temperature, humidity and cooling wind direction and other conditions.

Removal of accidental errors: Drive the test vehicle at least two cycles before starting the test.

c)	Running method:

Consider the methods listed below, assuming the driving of the test vehicle within the tolerance established in the test method.

Removal of systematic errors: Use the same rider throughout the test.

Removal of accidental errors: Repetition testing (already in action).

12.	OFF CYCLE EMISSIONS

The discussion has already been started, but this issue is subject of future work.  Generic aspects and definitions that are worked out in the parallel working HDV-Off Cycle emission group (GRPE informal group) will be considered for the discussions in the WMTC group.

13.	SUMMARY AND CONCLUSIONS

The developed test cycle and the accompanying gearshift procedure were tested in several laboratories in all three regions with respect to driveability.  They form a good balance between representativity of in-use driving and bench test requirements like reproducibility.  The dynamics of the WMTC cycle reflect the average driving behaviour for motorcycles in real live operation.  The final result of the cycle development is cycle version 9 with 3 parts and two versions per part (normal and reduced speed).

The road load settings and other test conditions like cooling fan specifications were updated according to the outcome of the ISO work.

Although the requirements of the emissions validation programme were not fully met, there are enough valid results left for future analysis.  All current reduction systems are represented in the vehicle sample; reduction systems were a bit underrepresented in provisional class 2, whereas the major part of the provisional class 3 vehicles was equipped with reductions systems.  The major part of the emissions validation test results can be used for a comparison with regional emissions test procedures.

The round robin test is finalised.  The salient outcome of the test was large differences in test results for the WMTC as well as for the ECE Regulation No. 40 among the laboratories that had performed the test.  These differences need to be reduced.  Some recommendations were given how to achieve this.

The compromises found for vehicle classification and weighting of the results are well balanced between the different requirements like representativity, practicability and simplicity.

With the developed test cycle, gearshift prescriptions and test protocol a world wide harmonized emissions test procedure for motorcycles can be established.  A draft version of a global technical regulation has already been worked out.  The only issue that needs to be clarified is the performance requirements.

14.	LITERATURE

[1]	R R.C. Rijkeboer:

WMTC – Final report, TNO Report 01.OR.VM.034.1/RR, by order of the Netherlands Ministry of the Environment (VROM), May 2001

[2]	F. Schröder:

"Betriebsweise, Emissionen und Kraftstoffverbrauch von Motorrädern". Thesis of the Technical University Darmstadt. Published as part of the series Fortschritt-Berichte VDI, Reihe 12, Nr. 435, May 2000

[3]	ISO TC22 SC22 WG17:

"Expert proposals on the four technical aspects to WMTC FE". April 2002

[4]	Draft global technical regulation (gtr) on uniform provisions concerning the measurement procedure for motorcycles equipped with a positive-ignition engine with regard to the emission of gaseous pollutants, CO2 emissions and fuel consumption by the engine (TRANS/WP.29/GRPE/2004/11)

[5]	JARI:

Presentation material for the WMTC group related to in-use data analysis with respect to cycle development work, dealing with statistical distributions of vehicle speed, idling time, trip length, acceleration, deceleration and cruising phases, 2000 to 2001

[6]	M. Akai:

"Analysis of the Candidate Cycles for the WMTC activities", Japan Automobile Research Institute, 02. April 2001

[7]	JARI:

"Analysis of WMTC Round Robin Test Results" (Draft), Japan Automobile Research Institute, 17. December 2003, WMTC-2004-143

15.	ANNEX A

DESCRIPTION OF THE MODIFICATION WORK ON THE WMTC CYCLE

The following modifications were carried out based on technical discussions in the WMTC group and preliminary tests by the industry.

TNO Automotive did first modifications on the WMTC cycle that are mainly related to the improvement of the driveability.  These modifications concern cycle parts below 20 km/h, the smoothing of ripples for the cruising parts caused by vehicle speed measurement uncertainties and the maximum speed of part 3 of the cycle.  A detailed description of these modifications is given in [1].

The resulting cycle was named "version 3" and is shown in Table 42 to Table 44.

Table 42: Cycle version 3, part 1

Table 43: Cycle version 3, part 2

Table 44: Cycle version 3, part 3

RWTUEV Fahrzeug carried out further modifications on the basis of discussions and decisions in the WMTC group.

The following modifications were done in a second step:

Part 1:

-	Modules 2, 3 and 5 of part 1 were replaced by more representative ones (length, average speed and dynamics were kept),

-	The rank order of the modules was changed (module 8 was shifted to the 2nd position),

-	The top speed of module 8 was limited to 60 km/h,

-	The idle time distribution was brought in line with the statistics.

Part 3:

-	The top speed was increased to 125 km/h.

These modifications resulted in version 4.

Furthermore an analysis of the acceleration pattern showed unrealistic "jumps" in some cycle phases.  Smoothing the acceleration pattern and recalculating the vehicle speed pattern from the smoothed acceleration pattern eliminated these "jumps".  This results in version 5.  Version 5 was used for the driveability validation tests and is shown in Table 45 to Table 47.

Table 45: Cycle version 5, part 1

Table 46: Cycle version 5, part 2

Table 47: Cycle version 5, part 3

Since vehicle speed pattern and gearshift procedure are closely linked, the elimination of malfunctions in the gearshift procedure resulted in cycle version 6.

As an outcome of the driveability validation tests the following modifications led to version 7 which is now the basis for the emissions validation:

In part 1 the modules 2 and 3 were exchanged to get a more realistic pattern for the cold start phase.

To reduce the risk of wheel lock cycle phases with excessive decelerations were modified so that the following limitations are fulfilled:

- vehicle acceleration	<= -2m/s²

- acceleration * vehicle speed 	<= -30 m²/s³

The second criterion (acceleration*vehicle speed) was necessary because there were still some sections in the cycles with acceleration values above -2 m/s² that caused wheel lock in some cases.  The modified cycles (version 7) are shown in Table 48 to Table 50.

Table 48: Cycle version 7, part 1

Table 49: Cycle version 7, part 2

Table 50: Cycle version 7, part 3

Although the results of a detailed analysis of the tyre slip phenomenon showed that the dynamics of the WMTC do not cause higher risks for tyre slip than existing certification cycles it was decided in the WMTC group that this risk should be minimised for the WMTC to improve driveability, independent of the situation for existing cycles.

A renewed analysis showed that the transitions from standstill to ride and vice versa become smooth enough if da/dt is between -0.8 m/s²/s and +0.8 m/s²/s.  Consequently the vehicle speed pattern were modified until da/dt falls into that range.

Since it is logical to apply the same criterion also for phases in ride condition where da/dt is outside this range (in cases where a deceleration is immediately followed by an acceleration and vice versa), these cycle phases were modified accordingly.

The results are shown in the following figures.

Table 51: Cycle version 8, part 1

Table 52: Cycle version 8, part 2

Table 53: Cycle version 8, part 3

During the WMTC FE meeting in Tokyo in April 2002, the Japanese delegation requested a special version of part 1 with a top speed of 50 km/h for low powered motorcycles whose technical specifications are close to mopeds.

Since there are only two modules in part 1 with vehicle speeds above 50 km/h (module 3 and 8), only these modules were modified. Module 3 was replaced by a module taken from in-use measurements of vehicle 6 (this module was closest to the existing one) and module 8 was lowered in speed to fulfil the limitation of 50 km/h.

The result is shown in the following figure.

Table 54:	Cycle version 8, part 1, reduced speed, with top speed limited to 50 km/h for low powered motorcycles with 50 cm³ engine capacity whose technical specifications are close to mopeds

The compromise found on vehicle classes made it necessary to create additional cycle versions with reduced speed also for part 2 and part 3.  In order to achieve that the dynamics of the cycle parts with reduced speed were only changed slightly compared to the normal versions, the speed pattern was not simply limited by the reduced maximum but the whole module containing the maximum speed was lowered at its first acceleration phase and its last deceleration phase (e.g. see Table 57).  For consistency reasons this approach was then also applied to part 1, so that version 9 of part 1, reduced speed is slightly different from version 8.  But there is no difference between the normal cycle parts of version 8 and 9.

The cycles version 9 are shown in the following figures.

Table 55:	Cycle part 1

Table 56: Cycle part 2 for vehicle classes 2 and 3

Table 57: Cycle part 3 for vehicle class 3

16.	ANNEX B – FINAL CYCLE VERSION

Table 58: Cycle part 1, version 9, 1 to 120 s

Table 59: Cycle part 1, version 9, 121 to 240 s

Table 60: Cycle part 1, version 9, 241 to 360 s

Table 61: Cycle part 1, version 9, 361 to 480 s

Table 62: Cycle part 1, version 9, 481 to 600 s

Table 63: Cycle part 2, version 9 for vehicle classes 2 and 3, 1 to 120 s

Table 64: Cycle part 2, version 9 for vehicle classes 2 and 3, 121 to 240 s

Table 65: Cycle part 2, version 9 for vehicle classes 2 and 3, 241 to 360 s

Table 66: Cycle part 2, version 9 for vehicle classes 2 and 3, 361 to 480 s

Table 67: Cycle part 2, version 9 for vehicle classes 2 and 3, 481 to 600 s

Table 68: Cycle part 3, version 9 for vehicle class 3, 1 to 120 s

Table 69: Cycle part 3, version 9 for vehicle class 3, 121 to 240 s

Table 70: Cycle part 3, version 9 for vehicle class 3, 241 to 360 s

Table 71: Cycle part 3, version 9 for vehicle class 3, 361 to 480 s

Table 72: Cycle part 3, version 9 for vehicle class 3, 481 to 600 s

___________