TRANSPORT CANADA REPORT -

“REVIEW OF MOTORCYCLE BRAKE STANDARDS” (Doc. 01-GTRBR-04)

IMMA QUESTIONS (04-03-25)

- Control Input Forces :

We cannot find any information regarding the level of force applied by the test rider to the brake control – hand / foot, front / rear.

Please confirm target forces and the actual forces recorded for each test carried out.

Also, for tests in which both controls are applied simultaneously, how was the front / rear (or hand / foot) apportion defined ? eg. FMVSS Wet test page B-6.

- ABS Testing :

- What was the friction coefficient of the 2 road surfaces used for straight line tests, braking in a turn and the changing surface used for the wheel lock up confirmation tests ?

- How were the maximum deceleration base line tests with the ABS disabled carried out ?   Eg. How was the maximum input force applied ?  Using pressure cut off valve or other ?

Also, how were the 2 Hondas with Dual CBS tested ?  Were both wheels being braked ?  Was the CBS modified ?  Were hand and foot controls used ?

The above applies to ECE, JSS, and the Test Procedure developed by Transport Canada.

- Wet Brake Tests :

On page B-6, we would like clarification of the “Minimum Allowable” column. Some configurations have a requirement and some do not.  Please explain.

4.  Fade and Recovery Tests :

On page B-8, we would like clarification of the “Minimum” column. Some configurations have a requirement and some do not.  Please explain

- Motorcycle Modifications :

For all tests, but particularly ABS tests, could you please confirm the modifications made to the motorcycles being tested. Eg. How was the ABS disabled ?

Were the CBS brake systems on the two Honda models modified ? If so, how was this done and for which tests ?

- General :

- In section 5.2, the report states that “No speeds were conducted above 160 km/h …..”  Why was this ?

- Could you please provide the weight of each motorcycle as tested, for each test ?

- It would be useful if raw test data, such as the recorded vehicle decelerations /stopping distances could be available for the next informal GTR meeting in April.