TRANSPORT CANADA REPORT -

“REVIEW OF MOTORCYCLE BRAKE STANDARDS” (Doc. 01-GTRBR-04)

Reply to IMMA QUESTIONS

- Control Input Forces:

- We cannot find any information regarding the level of force applied by the test rider to the brake control – hand / foot, front / rear.  Please confirm target forces and the actual forces recorded for each test carried out.   The target forces or min/max brake lever/pedal actuation forces required by the national regulations were included in the Regulation Comparison Table in Appendix A. The actual data for each individual test was recorded, a summary of which was provided in Appendix B, when required (e.g. see FMVSS water recovery and fade and recovery tests).

- Also, for tests in which both controls are applied simultaneously, how was the front / rear (or hand / foot) apportion defined? eg. FMVSS Wet test page B-6. No specific apportion is defined. The operator can modulate as necessary to meet the requirements of the test. Where the test procedure does not require a specific brake force, the test rider was instructed to be consistent in the application of the brakes during the test, in order to allow for the comparison of the results.

- ABS Testing:

- What was the friction coefficient of the 2 road surfaces used for straight line tests, braking in a turn and the changing surface used for the wheel lock up confirmation tests ? The straight-line tests were conducted on either asphalt or jenite. For asphalt, average skid number on dry = 87, wet = 48. For jenite, average skid number on dry = 60, wet = 18. The braking tests while in a turn were conducted on a newly paved section, having an average wet skid number = 85. For wheel lock-up confirmation tests, surface change was between dry asphalt and wet jenite. Also, Adhesion Utilization figures are available in a separate document, attached to e-mail.

- How were the maximum deceleration base line tests with the ABS disabled carried out ?   Eg. How was the maximum input force applied ?  Using pressure cut off valve or other ? Brakes were applied simultaneously, to impending lockup of the wheels. There was no limit on brake actuation forces in this case. No pressure cut off valve or other device was used to assist the operator.

- Also, how were the 2 Hondas with Dual CBS tested ?  Were both wheels being braked ? Yes  Was the CBS modified ? No Were hand and foot controls used ? Yes. The CBS was not altered in any way, and braking was performed as if without CBS.

The above applies to ECE, JSS, and the Test Procedure developed by Transport Canada.

- Wet Brake Tests:

On page B-6, we would like clarification of the “Minimum Allowable” column. Some configurations have a requirement and some do not.  Please explain. The minimum and maximum requirements are based on the baseline tests, where brake lever and pedal forces are measured to attain 10 to 11 fps². In the fifth wet recovery stop (i.e. the pass or fail stop), in order to achieve that same deceleration (10-11 fps²), the brake lever and pedal forces must be within +89N and –45N from the baseline tests, but not less than 0N. Therefore, the “Minimum Allowable” and “Maximum Allowable” forces will depend on the baseline results, and will vary with each motorcycle.

4.  Fade and Recovery Tests:

On page B-8, we would like clarification of the “Minimum” column. Some configurations have a requirement and some do not.  Please explain. Same answer for question #3.

- Motorcycle Modifications:

- For all tests, but particularly ABS tests, could you please confirm the modifications made to the motorcycles being tested. Eg. How was the ABS disabled ? Removing the fuse from the respective ABS controllers disabled the ABS systems.

- Were the CBS brake systems on the two Honda models modified ? If so, how was this done and for which tests ?  The CBS brake systems were not modified in any way.

- General:

- In section 5.2, the report states that “No speeds were conducted above 160 km/h …..”  Why was this ?  To limit the exposure to the testing riders.

- Could you please provide the weight of each motorcycle as tested, for each test ? See table below.  It would be useful if raw test data, such as the recorded vehicle decelerations /stopping distances could be available for the next informal GTR meeting in April.

|  | Make/Model |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Condition (Kg) | Honda VFR 800 | Honda ST 1100 | Suzuki GZ 250 | H-D FXD | BMW C1 125 cc |
| Unladen | 344.1 | 435.2 | 239.5 | 380.3 | 203.9 |
| Laden (ECE) | 432 | 508.2 | 361.8 | 501.5 | 360.8 |
| Laden (JSS) | 489.9 | 569 | 413.9 | 545.9 | 411.2 |