2nd Informal meeting: GTR motorcycle braking – 04/04/26-28

Participants:

Canada	: Messrs Davis, Brault

USA		: Mr Soodoo

IMMA		: Messrs  Cart, Chesnel, Daddi, Dutrieux, Honda, Rogers, Stocker, Evans

FEMA		: Mr Perlot

Italy		: Mr Lomonaco,

Japan		: Messrs Yamagishi, Hibara

JAMA		: Messrs Osuga, Toya

1.	Minutes

1.1 	Minutes of 1/GTRBR

Agreed	: The minutes of 1/MCGTR (01-GTRBR-03 of 03/07/30)

2.	Documents issued since the last agenda

[Note: the Informal Group's documents are on the UN website.

Look at the path : www.unece.org/trans/GRRF//]

| Date | Reference | Subjects |
| --- | --- | --- |
| 03/07/30 | 01-GTRBR-03 | Minutes of the 1st meeting |
| 04/02/10 | 01-GTRBR-04 | Biokinetics Report |
| 04/03/16 | 02-GTRBR-04 | Agenda |
| 04/03/25 | 03-GTRBR-04 | List of questions from IMMA |
| 04/04/16 | 04-GTRBR-04 | Revised GTR outline |
| 04/04/23 | 05-GTRBR-04 | Comments from India |

3.	NHTSA testing

3.1.	Technical Discussion of the Biokinetics report (Phase 1)

Document	: 01-GTRBR-04

Noted	: The presentation from Transport Canada of their philosophy for the test protocol used for

the Phase 1 testing.

: IMMA wanted to know the brake lever efforts used.

: Transport Canada replied that their rider had been instructed not to achieve the maximum	  	  possible stop but to be subjectively consistent when applying the brakes over the 6 runs per test.

: The test margin of compliance (MOC) used by IMMA had been on the basis of a known,

stabilized input (i.e. a lever force) or a stabilized output (i.e. a certain level of deceleration or

stopping distance).

: NHTSA thought that the ECE and JSS procedures might not reflect normal driving practice

because of testing each brake system independently.

: NHTSA’s view was that each change of each parameter would have an effect on the

final test result, making any direct comparison between the Brake Standards very difficult.

: Transport Canada had made its comparison on the basis of an averaging of all the results, which

were considered consistent.

: Except for the JSS Brake Standard, all the testing had been made with one single rider.

: For IMMA's tests with both brakes applied together, the measurements had been made once the

rider had achieved a stabilized percentage of brake force proportioned between the front brake

system and the rear brake system, normally by using a handlebar-mounted force display.

: The general acceptance for a brake distribution of 60% for front and of 40% for rear when both

brakes were applied together for most braking system.

: BMW pointed out that the initial performance of a combined braking system (CBS) might not

reach its maximum

best operability depending if the CBS design gave a fixed response or was including a

variable intelligent system which measured variables such as the temperature of pads.

: Differences in heating procedures of the heat fade test explained why the ECE Brake Standard

had heated the systems more than the FMVSS one.

: Transport Canada would possibly run extra testing with a fixed input forces during the coming

summer.

Agreed	: The replacement of the term “distance” by the term “effort” in Table 7 Page 14.

: The 2,5 value on Page B3 for the ST1100 & VFR800 was due to  ambiguous text within the

ECE 78 Brake Standard requirements for the second braking device. IMMA indicated this value

should be 5,1.

: The replacement of the value “238kg” by a value “350kg” in Table 4, Page 10.

: The selection of vehicles was not representative of the worldwide production.

: The final GTR text had to cover all types of vehicles.

3.2.	Update from Transport Canada on current anti-lock brake system (ABS)  testing (Phase 2)

Noted	      : Transport Canada confirmed the following list of tested motorcycles :

  - 2 Yamaha FJR1300 (ABS and non ABS versions)

  - 2 BMW 1150R (ABS and non ABS versions)

  - 1 Honda VFR 800

  - 1 BMW F 650

: Burnishing would be done before assessing the high speed braking performance based on the

JSS  Brake Standard.

: Tyres had not been changed after each test procedure of each Brake Standard as it had during

Phase 1.

: The reminder from Transport Canada that these new tests were to assess the overall 		        performance of ABS without any regulatory comparison intended.

: The number of maximum stops depending on the loading conditions would be recorded.

Agreed	      : The need to distribute the maximum load on the motorcycle to reflect typical use, eg passenger

on the pillion seat

and additional weight in luggage compartments at the rear of the motorcycle

3.3.	Answers to the IMMA list of questions

Noted 	: The following answers to the IMMA document (03-GTRBR-04)		(Annex3.doc)

: The level of force applied by the test rider to the brake control had not been stabilized.

: The test rider was instructed to "pass the test", while being consistent in the application of the

brakes throughout testing.

: There had been no pressure cut off valve or any other type of system.

: The ABS system had been disabled by means of a fuse.

: The CBS brake systems on the two Honda models had not been modified

: By Mid May Transport Canada would check which test had been made on asphalt and which

not.

: Transport Canada would provide more information in order to explain the discrepancies seen in

Appendix C / Item 5.5.2 and would provide the weight of each motorcycle as tested.

: For test rider safety, NHTSA considered 160km/h a maximum for the high speed test.

: NHTSA would be the contact point for any request of information about the Transport Canada

report.

: Transport Canada would check the wrong deceleration value of 5.12m/s2 for the Suzuki 250cc.

: IMMA pointed out that the baseline test values on Page B7 for the ECE wet brake test were

often wrong and inconsistent

: The BMW C1 was categorized somewhat similar to a Passenger vehicle in Canada.

: Regarding some “Minimum Allowable” being shown as zero in the results tables, this was due to

the fact that the regulation gives a plus and minus tolerance. For the specified tests, if the minus

tolerance had been applied, it would have resulted in a negative value which was not possible,

hence zero was written.

3.4.      Clarification of the IMMA Severity Test Work

Noted	: For the ECE tests, the forces had been applied at a position, which included any ball-end on the

lever.

: During a dry stop test procedure (Table 1.2.2), Transport Canada highlighted that, for the 1500 cc

model, the rear brake deceleration "achieved decel" results were lower than the ECE requirement,

whereas the result should have been higher given a greater brake control force was used, based on

the values in Table 1.2.1.

: The reason provided for this was the fact that the ECE test is fully laden whereas the FMVSS test is

lightly loaded

: It was pointed out those variations in the apportioning of the front/rear application force in the

testing of the FMVSS Brake Standard could give rise to a different result. This is a general

comment and could apply to both IMMA and Transport Canada.

: IMMA reminded that the lever effort was stabilized to the minimum limit value obtained before a

lock.

: IMMA confirmed that stable and repeatable results were achieved.

4.	Discussion of new braking systems

Noted	: Transport Canada asked how the GTR might be affected by new technological improvements.

: IMMA's response was that the only development foreseen was the introduction of "brake by wire"

for the transmission between the brake and the control lever . In itself this would not need special

provisions

: IMMA was not currently aware of any prototype of an hybrid (hydraulic-electronic) system.

5.	Discussion of the updated GTR outline

Documents : 05-GTRBR-04, the India comments

Noted	: IMMA reminded the group that :

a) the type approval system was not similar to self-certification since the Industry had to pay for

each test, hence each test had to be technically justified

b) The 1998 Agreement required the GTR to be cost effective

c) GRRF would expect a full rationale for the final proposal

: The IMMA position which was that the GTR regulation should be written in a way so that

requirements were testable.

: BMW's view that the practicality of testing was greater with MFDD than with stopping distances

: BMW pointed out that an immediate calculation of the deceleration was possible by only

monitoring the speed and the pressure input based on the general principle of a 0.5s delay for

recording

: The general view in the USA that a stopping distance was a more obvious basis for a test.

: The absence of a MFDD requirement in the FMVSS 135 Brake Standard whereas there was

a MFDD requirement and a stopping distance requirement in the Regulation 13H.

: IMMA thought that a car driver input had less influence on the test result than that of a rider,

because the rider had to control more variables .

: The specific situation in Japan where manufacturers could choose either MFDD or the stopping

distance.

: Italy pointed out that the stopping distance was equivalent to the mean deceleration and not the

MFDD and therefore would create discrepancies.

: The USA's proposal for the use of a Peak friction coefficient PFC instead of the skid number for

measuring the track surface

: The general acceptance that a parking brake should be tested statically

: The general US statement that if requirements have been included, then they should in principle be

testable and therefore enforceable.

: The absence of labelling requirement for fluid reservoirs in EU.

: The general philosophy for certification and conformity testing in North America. It was expected

that the manufacturer would test on the more severe side of the nominal requirement (e.g. the PCF

of a test track) whereas the administration would test on the lenient side. This was completely

different form the ECE philosophy of testing within specified tolerances.

: IMMA presented a burnish procedure which was the result of a survey of all motorcycle

manufacturers 									     (Annex 4.doc)

: For the partial effectiveness test, IMMA recommended that the remaining brake system should have

to meet the relevant dry stop requirement for that system. The partial failure test was only needed

for a split-service brake system because the proposed basic GTR dry stop test was for each

individual system.

: BMW emphasized the difficulty of keeping the temperature of the pads in CBS systems between

the FMVSS upper and lower limits. For example, BMW stated that for their models without any

braking, the temperature of the disk and pad appeared to be around 40-50°C and repeated tests

increased the difference between the front and rear brakes

: There were no exposed drum brakes in the current motorcycle production.

: The reported comment #20 from India that the current wet testing from ECE 78 would be suitable,

even for a country with heavy monsoon rain. They did not think that tan immersion test was

necessary.

: FEMA asked what would be the legal implication in case of an accident for not testing

above160km/h .

: Transport Canada answered that any testing above 160km/h would not be representative of a

normal riding on open roads.

: The major occurrence of motorcycles accidents was below 60mph in the USA.

: Japan pointed out that durability could not be covered by the test procedure.

: BMW confirmed that a ECE & FMVSS type approval process cost an average of 35000€ for 3 days

of work

: The ECE/EU mandated two separate brake controls and so no split service brake systems had been

developed

Agreed	: The revised GTR outline							           (Annex 1.doc)

: IMMA would add the legal definitions of brake system layouts as an annex to the GTR text.

: IMMA would produce the text for the durability of components.

: IMMA would review the brake application force values.

: IMMA would check the proposed values for the foot and hand control of the parking brake.

: IMMA would look for the rationale established by Germany in the early 1980s when they proposed

the current ECE test methods

: The difficulty of defining a constant relationship or equivalence between MFDD and stopping

distance.

: The parking brake was only required for 3 wheelers.

: NHTSA would consider whether the word "friction" was needed for the parking brake definition

: The open question of whether or not a Contracting Party would be allowed to add new parts to a

GTR text at national level.

: The heat fade test would be the last one in the test sequence.

: The request for a test sequence in order to satisfy USA certification purposes and consistency.

: The ECE text for the point of application of the brake actuation force would be added to the GTR

text

: With respect to the parking brake test, the material used for the road surface would not be specified.

: NHTSA would provide the group with the range of skid numbers typically used for their tests,

based on ASTM E-274-97.

: The burnishing procedure of CBS systems would depend on the CBS technology.

: Manufacturers would present their motorcycles for testing already burnished.

: The GTR would specify the burnishing procedure, so that there was consistency in preparing the

motorcycle for manufacturer and administration testing (see Annex 1, the revised GTR outline for

the technical basis of the procedure)

: NHTSA would check the rationale for specifying a low end brake temperature and whether or not it

should be at the start of each test or only for the very first brake test in a sequence.

: The wet test would be applied to all types of brakes, including disc, enclosed disc and drum brakes.

: IMMA would provide a definition of a baseline stop

: India would be asked to clarify what was the rationale for offering a disconnected scenario (item16

of their report) vis à vis the current ECE 78 requirement.

: The low speed L5 vehicles should not be required to pass the high speed test

: IMMA would provide a definition for a locked brake

: The need to specify the idle condition because of a possible impact of engine speed on braking with

the engine connected

: The group would consider the addition of a rider-alone, both brakes applied together test from 	  100 km/h

: The need for a technical rationale to be presented to GRRF in order to convince the European

Countries for the justification of extra new tests.

: The coming advantage for not dealing with border line problems during the dry stop test.

6.	The new test procedure

Agreed	: IMMA, U.S. and Canada would collaborate toward a draft proposal.		(Annex 2.doc)

7.	Interaction GRRF - GRSG

Noted	: All the GRs had been asked to agree RE5 in order to see if it could be used.

: Once agreed, RE5 would be turned into a consolidated resolution.

: Transport Canada would deal nationally with the requirements for 3 wheelers.

: Canada has a class for limited-speed motorcycles (LSM), for those motorcycles with a top speed of

70 km/h or less. The LSM’s must meet the same brake system regulations as full-size motorcycles.

There is no separate class for mopeds or motorcycles not being capable of going more than 30 mph.

8.         Process & timetable

Agreed: The following sequence of steps :

- IMMA would provide inputs by 04/07/15

- Transport Canada would provide a draft report on “Phase 2” to NHTSA by 04/07/31

- NHTSA would review and finalize report by 04/09/30

- A preliminary report to AC3 from the group would be prepared by Transport Canada before 04/11/15

- The next GRRF would be given a presentation of the technical basis for the GTR

- The GRRF in February 2005 would be able to start discussion of the draft text

- In the absence of any major disagreement, the final draft GTR text could be approved by GRRF in September 2005

- Cost effectiveness study finished by the 05/12/31

- Final draft GTR text by the 06/02/15 for approval by AC3 – Canadian Consultation

- NHTSA-EU would publish the text

- Completion of the GTR for voting by the 06/06/15

: The drafting team would consist of :

Mr Soodoo

Mr Cart

Mr Brault

9.	Recommendations to 56/GRRF

Agreed	: The technical content agreed so far, which would be presented at the next session to

obtain comments on the principles

: As a basis for discussion, the group would further consider a new test :

- Dry stop under light loaded conditions

- Both brakes together

- Speed of 100 km/h or 0.9 of real Vmax for medium limit speed motorcycles

10.	Special Thanks

Noted       : The Chairman thanked India for the excellence of their report.

11.	Next Session

Agreed     : The next meeting would be held in Montreal tentatively on the 04/11/8-10.

: What needed to be included into the Cost Effectiveness Analysis would be discussed.

Philippe C. CHESNEL