GLOBAL HARMONIZATION OF PTW, AND 3W BRAKING

DEVELOPMENT OF DRAFT GTR PROPOSAL

PROPOSED GTR OUTLINE FOR DISCUSSION AT 2ND INFORMAL MEETING

Document revision date : 04/04/28 as prepared by John Cart

Transport Canada yellow comments date : 04/05/27

| LAYOUT |  | PROPOSED CONTENT IN SUMMARISED FORM – TEXT LATER | NOTES/ACTIONS FROM PREVIOUS MEETING Jobs at 2/Informal Brakes meeting: 04/04/28 Changes at meeting |
| --- | --- | --- | --- |
| # 1. SCOPE |  | Intended to apply to 2 and 3 wheeled motorcycles including mopeds and electric power. For reference, extracts from ECE REG 78 :- Applies to the braking of 2 and 3 wheeled vehicles. Excludes those with V max < 25 km/h and fitted for invalid drivers. Summary of vehicle categories : L1 = 2 wheels, engine  50cc and max speed  50 km/h L2 = 3 wheels, engine  50cc and max speed  50 km/h L3 = 2 wheels, engine  50 cc or max speed  50 km/h L4 = 3 wheels – asymmetric, engine  50 cc or max speed  50 km/h (motorcycle + sidecar) L5 = 3 wheels – symmetrical, max weight 1000kg., engine  50 cc or max speed 50 km/h . | Wait for the outcome of GRSG vehicle classification. . |
| # 2. DEFINITIONS |  | Review after tests have been agreed. Include: brake, transmission, control, CBS, split service, ABS, laden | Add during GTR development |
| # 3. REQUIREMENTS |  |  |  |
| 3.1 Performance |  | - Based on MFDD or the corrected stopping distance - Test brakes separately unless otherwise specified. - Specify laden or unladen for each test. - Clarify requirements for split brake systems where necessary. | Use the existing R78 formulas as a general formula for converting Saturated Deceleration (SD) into MFDD and then say that Conformity of Production (CoP) must be done in the same way as the original type approval. |
| 3.2 General - Equipment - Function | Durability | Agreed: - During tests and on their completion, there shall be no lining detachment and no leakage of fluid. |  |
|  | Parking brake characteristics | Agreed: For 3 wheelers only. 5.1.2.3 – must hold the vehicle stationary on a slope in the absence of the driver. Working parts locked in position by a mechanical device, actuated from the driving seat. | USA to check if "friction" brake should be specified |
| 3.3 Design and Operational Characteristics |  |  |  |
| 3.3.1 Design | Type of service brake system | In principle :- - 2 separate braking systems - 1 or 2 brake controls | Include in “Definitions”. . the location of controls should be kept separate from these performance requirements |
|  | # Mechanical service brake system | For reference ECE R78, states :- 5.2.1.1 – Parts such as the brake, cylinder, pistons, etc., shall not be regarded as liable to breakage if they are amply dimensioned, readily accessible for maintenance and exhibit sufficient safety features. Agreed The 2 service braking devices may have a common brake so long as a failure in 1 does not affect the performance of the other. | delete the ECE REG 78 5.2.1.1 items because they are not testable. |
|  | Hydraulic service brake system |  |  |
|  | Master cylinder reservoirs | FMVSS 122 :- S5.1.2.1. – Each m/cylinder shall have a separate reservoir for each brake circuit and openings having their own cover etc Reword the capacity requirements to say that the amount of fluid must make the pads touch the disc when they are worn right down ? Each reservoir shall have a min. capacity based on 1.5 times volume required to cover difference between new and fully worn linings – brakes applied. | Must be worded to be objective, measurable and enforceable |
|  | Reservoir labelling | FMVSS 122 :- S5.1.2.2. – Brake fluid warning statement that specifies : - Text and size of letters - Method of application - Location | ### Delete from the GTR and leave to consumer legislation |
| 3.3.2 Operation | Failure indicator lamp | FMVSS 122 :- S5.1.3 – Additional requirement for vehicles with split service brake systems S5.1.3.1 – Details of the lamp function : - Position - When it functions eg. pressure failure, low reservoir level. - Ignition switch activation etc. Colour and marking of lens | Agreed to use FMVSS text The telltale will be red. IMMA to check for a symbol for brake failure in R60. |
|  | Inspection of pad/lining | FMVSS 122 :- S5.1.5 – Lining thickness of drum brakes shall be visually inspected without removing drums and pad thickness visible without removal. | Agreed to use FMVSS text. Do the inspection at the end of the certification/procedure USA to check how this is done/interpretation |
| 4. TESTS |  |  |  |
| 4.1 General | Test sequence | Sequence of testing necessary and to be determined. The heat fade test should be done last, in order to eliminate reburnishing from the test sequence. | . Sequence of testing necessary |
|  | Transmission | Unless otherwise specified, all stops to be made with the clutch disengaged. (Not applicable to auto gearbox) | ← Proposed Content agreed at 1st GTR meeting. |
|  | Ambient temperature | 4 - 38 C | To avoid frozen surface. |
|  | Wind velocity | JAPAN SS 12 – 61 :- Not more than 5 m/s |  |
|  | Vehicle position and wheel lock | All stops to be made without deviation from the test lane and without wheel lockup (not applicable to ABS equipped vehicles < 10 km/h). Vehicles shall start in the middle of the lane. | ← Proposed Content agreed at 1st GTR meeting. |
|  | Brake actuation force | For reference, ECE REG 78 states :- Annex 3 – 1.2.4.2.4 Hand control:  [200N] Foot control:  [350 N] (L1,L2,L3,L4)  [500 N] (L5) Point of application will be the ECE text. | ### T.Canada does not think minimum values are necessary. USA to review need for minimum values and clarify current requirements. Put the ECE definition of the application point in the definitions list. |
|  | Temperatures | the temperature measured inside the brake linings or on the braking path of the disc or drum , is between 55 and 100 C, [at the beginning of each stop] | Agreed that brake temperatures are measured on the disc or shoe with thermocouples. Need to agree whether this requirement is necessary at the beginning of each stop, or on the fist stop of a series of stops. USA to research which test has to meet this minimum temperature, the first of a series or all the stops. |
|  | Engine condition | Engine idle speed to be at manufacturers specification |  |
| 4.2 Preparation |  |  |  |
| 4.2.1 Track | Road surface | Test area to be clean, dry and level road surface. Test surface shall have a nominal [maximum] skid number of 81 ( ) or a peak friction coefficient of 0.9. using an ASTM E1136 standard reference tyre in accordance with ASTM Method E 1337-90 Maximum lane width of 2.5 m for 2 wheeled motorcycles; vehicle width plus 2.5 m for 2 wheeled motorcycles with sidecar or 3 wheeled motorcycles. [For ABS, see ECE 78 Annex 4] For parking brake test specify "a clean, dry, solid surface." | USA to provide full reference to ASTM test procedures for the skid number and the PFC GRRF Test houses to be asked what their skid numbers are USA to provide information on the range of skid values actually used in testing |
| 4.2.2 Vehicle | Vehicle mass | For reference, ECE REG 78 :- In general, vehicle is fully laden except : - High speed test unladen. - CBS tests laden and unladen - ABS tests unladen Note: Fully laden = manufacturers max mass. Unladen = rider and test equipment Final decision will depend on agreed test procedures | Wait for the definition of mass from GRSG (if time allows) USA to consider any consequences from Phase 2 testing. |
|  | Tyre pressure | Manufacturer recommendation | ← Proposed Content agreed at 1st GTR meeting. |
|  | Brake burnishing | Include a requirement that the manufacturer will carry out the burnishing and show records to the test house on request. Procedure to be adopted by a government when doing conformity testing: rider alone initial speed min 50 km/h reacceleration speed 5-10 km/h decel CBS 3.5-4.0 m/s2 + general statement that all the brakes must be burnished decel front separately 3.0-3.5 m/s2 decel rear separately 1.5-2.0 m/s2 100 stops per brake engine disconnected the initial brake temperature before each stop <100°C | USA to provide the technical reasons for why a minimum temperature should be included. |
| 4.3 Dry Stop Tests |  | Agreed the ECE tests - 6 stops maximum - Laden vehicle only (but CBS where fitted also tested unladen) - Initial speed = 40 km/h for L1, L2 and 60 km/h for L3, L4, L5 vehicles - All stops with engine disconnected - Separate test for front brake, rear brake, and CBS (if fitted) Requirements : Minimum deceleration : Front : L1 = 3.4 m/s2 L2 = 2.7 m/s2 L3 = 4.4 m/s2 L4 = 3.6 m/s2 L5 = 2.9 m/s2 Rear : L1 = 2.7 m/s2 L2 = 2.7 m/s2 L3 = 2.9 m/s2 L4 = 3.6 m/s2 L5 = 2.9 m/s2 CBS : L1,L2 = 4.4 m/s2 L3 = 5.1 m/s2 L4 = 5.4 m/s2 L5 = 5 m/s2 CBS secondary brake = 2.5 m/s2 for all vehicle types. | The group will consider a100 km/h stop to the requirements of the FMVSS, as an additional test to be added to the GTR. The draft specification will be circulated with the minutes. The wording of the CBS tests would be reviewed by IMMA in the light of modern designs. |
| - Wet Brake - Base line - Wet brake |  | Agreed ECE REG 78 wording:- Summary - For vehicle categories L1,L2,L3,L4 (not L5) - Laden vehicle only (CBS where fitted also tested unladen) - Initial speed = 40 km/h for L1, L2 and 60 km/h for L3, L4 vehicles - Stops with engine disconnected - Separate tests for front brake, rear brake, and CBS (where fitted) - Perform a Dry Stop test (as 4.3) but measure the control force to achieve 2.5 m/s2 - Repeat Baseline test after the brake has been continuously wetted at a flow rate of 15 l/h for > 500 m. Requirement: - Deceleration recorded between 0.5 and 1 sec after brake application = > 60% to 120% of Base line test using same control force. | ## All brake types must be tested |
| - Heat Fade - Base line - Fade - Recovery |  | Agreed ECE REG 78 :- - L3,L4, and L5 vehicles only - All with laden vehicle - Separate test for each brake system - If CBS fitted, test only CBS - Perform a single Dry stop test as in 4.3 above - Perform 10 repeated stops as quickly as possible - Speeds – Front + CBS = 100 km/h or 70% v max whichever is lower - Rear = 80 km/h or 70 % v max whichever is lower - Braking interval = 1000 m - Engine connected with suitable gear for 50% stop, disconnected for remainder. - For the first stop, deceleration = 3 m/s2 with constant control force and the same force for remainder - Repeat Baseline test ASAP or at least within 1 minute after completion of Fade test. Requirement : Recovery test performance =  60% of Baseline test performance | Mopeds (L1 and L2) would be excluded. Group to consider engine disconnected for the heating stops, in time for the next meeting. IMMA review the USA proposed application forces. |
| 4.6 High Speed |  | JAPAN SS 12 – 61 (with higher speed) :- - L3, L4, & L5 vehicles with a Vmax of ≥ 100 km/h - Unladen vehicle only - Stops with engine connected - Test front brake + rear brake together OR CBS+2ndary brake - Initial speed = 160 km/h or 0.8 v max whichever is less Requirement: - Deceleration > 5.8 m/s2 or the stopping distance S≤ 0.1V + V2/149 - the vehicle to remain inside the specified lane - Maximum of 4 stops to achieve requirements | ← Use test procedure from Japanese standard. USA to review the need for an initial brake temperature. Group to review the USA's proposed application forces |
| OPTIONAL / IF FITTED |  |  |  |
| 4.7 Parking Brake |  | Agreed: ECE REG 78 summary :- - Static test - Laden vehicle - 18% slope - up and down - control forces: hand < 400N; foot < 500 N Requirement : - Vehicle remains stationary on slope during a period of 5 minutes. | IMMA to review 245N hand and 400N foot control forces. and the inclusion of the 5 minute test time. . Suggested alternate text, in the event the slope is too slippery. |
| 4.8 ABS |  | For reference, ECE REG 78 summary :- - L1 + L3 vehicles only - Tests on 2 road surfaces : > 0.8, and < 0.45 - Unladen vehicle only - Initial speed = 60 km/h - Separate test for front brake and rear brake. - Stability tests for low-high surface, high-low surface and both brakes together Requirements: - = > 0.7 for L3 vehicles only. No requirement for L1. - Electrical supply failure shall be signalled to the rider - If ABS failure, Dry Stop test performance (4.3) shall be maintained - Wheel must not lock on test surface (but acceptable if low speed and vehicle stable) - For low-high surface test, vehicle decel “must rise to the appropriate high value within a reasonable time” Note : - Additional requirements may be required for vehicles fitted with CBS | ← ECE is the only procedure available. May be altered following future tests with ABS systems. Need to wait for the results of the USA testing. Group to review terms like "sudden", "full force" etc. and either find alternatives or provide a testable definition. |
| 4.9 Partial Failure |  | Requirement only applicable to a “Split service brake system” – see FMVSS 122 S4. The remaining sub- system must meet the relevant Dry Stop requirement |  |
| ITEMS PROVISIONALLY DELETED |  |  |  |
| Pre burnish |  | Provisionally not necessary | Delete |
| Final effectiveness test |  | Duplicate test, therefore not necessary. | Delete. |
| Pretest instrumentation check |  | Test house responsibility | Delete |
| Final inspection |  | Inspection referenced in Section 3.2. | ### See 3.2 |