June 30, 2000

BURNISHING AND INSTRUMENTATION-CHECK

As agreed at the first BHTF meeting held in the IMMA offices on 00/05/02-02, this is the report of the data collection regarding “burnishing and instrumentation-check” in view to answer the question:

what should the burnishing and instrument-check procedures be?

1.	DATA COLLECTION

To collect information on the subject matter it had been prepared a questionnaire and sent to 10 motorcycle manufacturers; (see ANNEX 1).

The answers had been summarised on “Summary table” of this report (note that not all the questionnaires had been returned at the moment).

On the table the columns contain:

-	“Q”		The number of the question;

-	“Row”		The number of the row of any question as defined in ANNEX 2;

-	“M1 ÷ M10”	The answers of the single manufacturer;

-	“BR”		The parameters of the BREMBO procedure for burnishing;

-	“USA”		The parameters of the FMVSS 122 burnishing procedure.

2.	PRELIMINARY CONSIDERATIONS

  - On the burnishing

It is well known that the best efficiency of a brake arise when the contact between rotor and lining is uniformly on all the contact surface.

It is also well known that when a new set of rotor and lining is installed on a vehicle the above mentioned contact is not possible for reasons well known to a technical person.

The only manner to obtain said uniform and intimate contact is the burnishing.

2.2.	On the answers

Also if not all questionnaire had been returned it is possible to make some considerations:

- All the manufactures consider necessary to perform a burnishing of the braking system in view of the homologation of the vehicle.

- Except for some manufacturer answers the burnishing procedures performed are very similar, and more particularly:

- the number of stops: 100 seem to be considered adequate to perform the burnishing;

- the initial speed: 50 ÷ 60 Km/h seem to be considered appropriate;

- the final speed: 0 Km/h seem to be considered appropriate;

- the braking interval: 1000 m, except for some manufacturer, seem to be considered appropriate;

- deceleration: 3.5 m/s2 is practically considered appropriate for all manufacturers.

- All manufactures equip their vehicles with appropriate instrumentation to measure and to record various braking parameters.

- All manufacturers, except one, make, also if with different procedures, an instrumentation-check, and repeat the check procedure if a reparation or adjustment occur.

- Except for the number of stops the FMVSS 122 burnishing procedure and the burnishing procedure performed from the manufacturers asked seem not so distant.

3.	CONSIDERATIONS IN VIEW OF A BURNISCHING PROCEDURE

For the considerations at point 2.2. seem evident that burnishing and instrumentation-check are necessary before of the homologation test.

The philosophy of burnishing seem different, because:

1	FMVSS 122 burnishing procedure imposes 200 stops in any case; this fact implies that vehicles may to fail the tests following;

2	For homologation under R 78 regulation the vehicles may be prepared at the best mode; making the burnishing considered the best from the manufacturer.

From the answers received it is possible to see at least two different modes to make the burnishing:

- making a quantity of stops similarly to the FMVSS 122 except in the number;

- driving the vehicle for a quantity of Kilometers during a pre-test session before the homologation.

It seems too easy to tell “view that all manufacturers make burnishing then way do not procedurize it?”; but this sentence implies at least other two considerations as:

- it does not be never again possible to make a burnishing in the manner considered the best; but only the procedurized burnishing;

- a GTR with a burnishing procedure would be more expensive in respect of R 78 regulation;

4	PROPOSAL OF BURNISHING AND INSTRUMENTATION-CHECK PROCEDURE

  - Burnishing procedure

A satisfactory burnishing should be obtained with a series of braking as follow:

number of braking	50			at least

initial speed		50 ÷ 60 Km/h

final speed		5 ÷ 10 Km/h

deceleration		3.5 ÷ 4.5 m/s2		for combined brake

deceleration		3 ÷ 3.5 m/s2		for front brake

deceleration		1.5 ÷ 2 m/s2		for rear brake

interval		1000 ÷ 1200 m

notes:	final speed is not “0” because thus the driver does not put a foot to earth;

it is foreseen to make burnishing for front and rear brake because thus it is easy to obtain the same deceleration at any braking without to balance the braking action between front and rear.

4.2	Instrumentation check procedure

An instrumentation check should be obtained as follow:

3 stops from 40 ÷ 50 Km/h at a deceleration of 3 ÷ 4 m/s2;

3 stops from 80 ÷ 100 Km/h at a deceleration of 3 ÷ 4 m/s2;

3 stops from 40 ÷ 50 Km/h at a deceleration of 6 ÷ 7 m/s2;

3 stops from 80 ÷ 100 Km/h at a deceleration of 6 ÷ 7 m/s2;

notes:	this proposal permits to have a check on the functionality of the instrumentation further a general check;

the instrumentation check may be substituted with a check and calibration when the vehicle is prepared;

a general check of the instrumentation may be done during the burnishing and only if a repair, replacement, or adjustment is necessary must be done.

4.3	Burnishing and instrumentation check sequence

The sequence of this procedure should be as follow:

- prepare the vehicle with a new set of brakes (rotors and linings) and equip them with the necessary instrumentation;

- make the necessary check and calibration of the instrumentation;

- make the burnishing with contemporary general instrumentation check;

- make, if necessary, repair, replacement, or adjustment of the instrumentation

- make the instrumentation check procedure if , repair, replacement, or adjustment had been necessary.

Must be clear that the above proposal is the burnishing and instrumentation check procedure considered least indispensable by BREMBO.

L. CAVESTRO

BURNISHING AND INSTRUMENTATION-CHECK

Summary table

| Q | Row | M1 | M2 | M3 | M4 | M5 | M6 | M7 | M8 | M9 | M10 | BR (11) | USA (13) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Y | Y | Y | Y | Y | Y | Y | Y |  |  | Y | Y |
|  | 2 | Y | Y | Y | Y | Y | Y | Y | Y |  |  | Y | Y |
|  | 3 | 100 | 100 | (03) | 20÷30 | 175 | 100 (10) | 20 | ≤100 |  |  | 200 | 200 |
|  | 4 | 50 | 60 | (03) | 60÷70 (05) | 120÷ 150 | 60 | 50 | ≤50 |  |  | 80 | 48.3 |
|  | 5 | 5 | 0 | (03) | 0 | 30 | 0 | 0 | 0 |  |  | 30 | 0 |
|  | 6 | various | 1000 | (03) | 500 | 1200 | 1000 | (15) | (18) |  |  | 1000 | (14) |
|  | 7 | 3 | 3.5 | (03) | 3÷4 | 3÷4 | ≤3.5 | 5 | 3 |  |  | 3 | 3.6576 |
|  | 8 |  |  |  |  | (09) |  | (16) |  |  |  |  |  |
|  | 9 | Y |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 1 | Y | Y | Y | Y | Y | Y | Y | Y |  |  | Y |  |
|  | 2 | Y | Y | Y | Y | Y | Y | Y | Y |  |  | (12) |  |
|  | 3 | Y | Y | Y | (06) | Y | Y | Y | Y |  |  | Y |  |
|  | 4 | Y | N | Y | Y | N | N | Y | N |  |  | (12) |  |
|  | 5 | Y | N | Y | N | N | N | N | N |  |  | (12) |  |
|  | 6 | Y | Y (01) | Y | (07) | N | Y | Y | Y |  |  | Y |  |
|  | 7 |  | (02) | speed |  |  |  | (17) |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 1 | Y | Y | Y | Y | N | Y | Y | Y |  |  | Y |  |
|  | 2 | Y | Y | N | Y |  | Y | Y | N |  |  |  |  |
|  | 3 |  |  | (04) |  |  |  |  |  |  |  |  |  |
|  | 4 | Y |  |  |  | Y |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 1 | Y | Y | Y | Y | N | Y | Y | Y |  |  |  |  |
|  | 2 | Y | Y | Y |  |  | Y | Y | Y |  |  |  |  |
|  | 3 |  |  |  | (08) |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |

Cross-reference:

- on the questionnaire is specified “ambient” temperature;

- “wind velocity”;

- for these parameters on the questionnaire is specified:

“We  make some stops but don’t always use same pattern of braking for burnishing”

“We usually use same system which is already burnished well for pre-test of braking”;

- “speed and distance and force check”;

- this value is given for MCs; for CMs is given 40, not reported on this summary;

- “calculated by Correvit”;

- “Surface thermometer not on MC”;

- for repaired instruments on the questionnaire is specified:

“Strain gauge calibrated by dynamometer”

“Correvit calibrated before each test”;

- “The above is one example of a certain model”;

- “about 100 (CASE BY CASE)”;

- this is the procedure used by Brembo for burnish;

- the instrumentation corresponding at these parameters change with the test;

- speed and deceleration values are expressed in IS unit;

- as specified on FMVSS 122 standard

- “Necessary space to reach the speed”

- “(Usually this procedure is not necessary because we get the complete bike used driving at least 200 km, before the homologation)”

- “Temperature only in case of FMVSS 122”

(18)	“Between 500 and 1000”

BURNISHING AND INSTRUMENTATION-CHECK

Dear Sirs,

at the meeting held on 02th-03th may 2000 in IMMA offices, I was given the responsibility to provide a draft research regarding “burnishing and instrumentation-check” of a PTW, in view of developing a GTR for PTW braking.

ECE R.78 does not require “pre-burnish”, “burnish” and “instrumentation-check”; now I would ask You to fill in the following questionnaire, which I have prepared, for data collecting.

1.	Do You prepare the PTW with a burnish?				  YES	  NO

if YES:	do you make some stops?					  YES	  NO

-	if YES:	Number of stops	_________

Initial speed		_________	Km/h

# Final speed		_________	Km/h

Interval		_________	m   or   _______  sec

Deceleration		_________	m/s2

-	if NO:	please specify			_____________________________

if NO:	Do You start the omologation tests with the braking system new? (particularly disc/drum and pad/lining)					  YES	  NO

-	if NO:	please specify			_____________________________

2.	Do You equipe the PTW with instrumentations?			  YES	  NO

if YES:	instrument for:	control forces				  YES	  NO

deceleration				  YES	  NO

stopping distance			  YES	  NO

pressures				  YES	  NO

temperatures				  YES	  NO

other			_____________________________

if NO:	please specify					_____________________________

3.	Do You make an instrumentations check?				  YES	  NO

if YES:	how:	you some check stops					  YES	  NO

other						_____________________________

if NO:	you start directly with omologation tests			  YES	  NO

other						_____________________________

4.	If the test instruments are repaired or adjusted:

-			You make an other instrumentations check			  YES	  NO

if YES:	same at point 3						  YES	  NO

other						_____________________________

if NO:	please specify					_____________________________

I kindly ask You to return to me the questionnaire as soon as possible, before June 22th, 2000.

Thanks in advance and Best regards.

Luigi CAVESTRO	(Brembo SpA)

Tel.:	+39 035 605 566

Fax.:	+39 035 605 579

E-mail: luigi_cavestro@brembo.it							ANNEX 1

BURNISHING AND INSTRUMENTATION-CHECK

Dear Sirs,

at the meeting held on 02th-03th may 2000 in IMMA offices, I was given the responsibility to provide a draft research regarding “burnishing and instrumentation-check” of a PTW, in view of developing a GTR for PTW braking.

ECE R.78 does not require “pre-burnish”, “burnish” and “instrumentation-check”; now I would ask You to fill in the following questionnaire, which I have prepared, for data collecting.

1.	Do You prepare the PTW with a burnish?				  YES	  NO	  (1)

if YES:	do you make some stops?					  YES	  NO	  (2)

-	if YES:	Number of stops	_________				  (3)

Initial speed		_________	Km/h			  (4)

# Final speed		_________	Km/h			  (5)

Interval		_________	m   or   _______  sec	  (6)

Deceleration		_________	m/s2			  (7)

-	if NO:	please specify			_____________________________	  (8)

if NO:	Do You start the homologation tests with the braking system new? (particularly disc/drum and pad/lining)					  YES	  NO	  (9)

-	if NO:	please specify			_____________________________	  (10)

2.	Do You equipe the PTW with instrumentation ?			  YES	  NO	  (1)

if YES:	instrument for:	control forces				  YES	  NO	  (2)

deceleration				  YES	  NO	  (3)

stopping distance			  YES	  NO	  (4)

pressures				  YES	  NO	  (5)

temperatures				  YES	  NO	  (6)

other			_____________________________	  (7)

if NO:	please specify					_____________________________	  (8)

3.	Do You make an instrumentation check?				  YES	  NO	  (1)

if YES:	how:	you some check stops					  YES	  NO	  (2)

other						_____________________________	  (3)

if NO:	you start directly with homologation tests			  YES	  NO	  (4)

other						_____________________________	  (5)

4.	If the test instruments are repaired or adjusted:

-			You make an other instrumentation check			  YES	  NO	  (1)

if YES:	same at point 3						  YES	  NO	  (2)

other						_____________________________	  (3)

if NO:	please specify					_____________________________	  (4)

I kindly ask You to return to me the questionnaire as soon as possible, before June 22th, 2000.

Thanks in advance and Best regards.

Luigi CAVESTRO	(Brembo SpA)

Tel.:	+39 035 605 566

Fax.:	+39 035 605 579

E-mail: luigi_cavestro@brembo.it								ANNEX 2