Proposal from the 2/GTRBR meeting, Brussels 04/04/26-28 (Prepared by Dr Rogers)

Specification for the additional test to be added to the GTR

Basis for the test

The test procedure is based on the current FMVSS dry stop test at 60 mph. It represents the case of a rider on an unladen motorcycle at a medium speed, applying both brakes together with the engine disconnected.

Where CBS is installed, test the CBS applied at the same time as the secondary brake system.

Actuation forces: 		Hand control:  [250N]

Foot control:  [400 N].

[Question from IMMA : is it necessary to specify the distribution of the braking effort for this test, to make it easier to repeat, and if so, how should it be done?

Reply from Transport Canada : No. The maximum possible braking effort will vary with the motorcycle type and weight distribution, therefore preferred to leave this open.]

Initial brake temperature:	between 55 and 100 C

[Note: USA is clarifying whether the lower temperature is only relevant for the first stop of a series of stops or for all stops.]

Comment from Transport Canada :  The brakes should be within this temperature range at the beginning of each stop.

Vehicle loading:		the vehicle shall be unladen, i.e. the rider alone

Test Procedure:

6 stops from 100 km/h or 0.9 Vmax, whichever is the lower. In any case the lowest initial test speed shall be 45 km/h [0.9 x 50 km/h, the definition of a moped, which are all tested at 40 km/h.]

[For comparison, note that ECE high speed and ABS tests use 80% Vmax]

Draft Performance requirements based on the existing FMVS test  (Prepared by IMMA)

| test speed (V) |  |  |  | stopping distance (S) |  |  | V2 /2xS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mph | km/h | m/s | V2 | Feet | Metres | 2S | decel |
| 25 | 40 | 11 | 124.85 | 30 | 9 | 18.3 | 6.82 |
| 30 | 48 | 13 | 179.78 | 43 | 13 | 26.23 | 6.85 |
| 35 | 56 | 16 | 244.71 | 58 | 18 | 35.38 | 6.92 |
| 40 | 64 | 18 | 319.61 | 75 | 23 | 45.75 | 6.99 |
| 45 | 72 | 20 | 404.51 | 95 | 29 | 57.95 | 6.98 |
| 50 | 80 | 22 | 499.40 | 128 | 39 | 78.08 | 6.40 |
| 55 | 88 | 25 | 604.27 | 155 | 47 | 94.55 | 6.39 |
| 60 | 97 | 27 | 719.13 | 185 | 56 | 112.85 | 6.37 |
|  |  |  |  |  |  | Average | 6.71 |

Comment from Transport Canada : This draft should be based on the same average deceleration, or the corresponding stopping distance. An equation is suggested to correlate the equivalent stopping distance, as it will vary with the vehicle’s top speed (a table cannot provide stopping distances for every possible speed).