# Informal document No. 86-GRSG-30

# (86th GRSG, 19 - 23 April 2004,

# agenda item 3.)

# ECONOMIC COMMISSION FOR EUROPE

## INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

- Working Party on General Safety Provisions (GRSG)

- PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 107

- Transmitted by the Expert from Germany

TRANS/WP 29/GRSG/2003/21/Rev. 1

Paragraph 5.7.9.5.2, amend to read:

"5.7.9.5.2.	For a seat behind a seat and/or a seat facing the gangway, a minimum clear foot space of at least 300 mm depth and a width according to paragraph 7.7.8.1.1. shall be provided as shown in annex 4, figure 11b. The local presence in this space of seat legs, passenger footrests and of intrusions as provided by paragraph 5.7.9.6.2.3 shall be permitted provided that adequate space remains for the passengers' feet. This foot space may partly be situated in and/or above the gangway but shall not create any obstruction when measuring the minimum gangway-width in accordance with paragraph 5.7.5. "

Annex 4

Figure 11b

"Space for seated passengers behind a seat and/or a seat facing the gangway"

(see annex 3, paragraph 7.7.8.5.2)

60 cm max

15 cm min (12 cm in the case of a seat over a wheelarch

30 cm min

30 ° max 
-8 ° min

70 cm min

Justification:

The intention of the proposal is to clarify that the foot space in front of a passenger seat may be restricted by passenger footrests and wheelarches.