| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/GRSP/2004/1 20 February 2004 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Passive Safety (GRSP)

(Thirty-fifth session, 3-7 May 2004,

agenda item B.1.1.)

## PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 14

(Safety-belt anchorages)

# Transmitted by the expert from Spain

Note:  The text reproduced below was prepared by the expert from Spain in order to clarify the definition proposed for an effective anchorage.  It is based on informal document GRSP-34-19 distributed at the thirty-fourth GRSP session (TRANS/WP.29/GRSP/34, para. 15).

____________

_______________

Note: This document is distributed to the Experts on Passive Safety only.

A.	PROPOSAL

Paragraph 2.4.1.1., amend to read:

"2.4.1.1.	when a safety-belt incorporates a rigid part which is attached to a lower belt anchorage, forming a rigid system with only one degree of freedom, the effective belt anchorage for all positions of seat adjustment is the point where the strap is attached to that rigid part;"

Insert a new paragraph 2.4.1.1.1., to read:

"2.4.1.1.1.	If that rigid part supporting the band may rotate around one point, the effective anchorage may be the pivoting point, considering that if the rigid part is substituted by a strap attached to the theoretically effective anchorage, the contact areas between belt and occupant does not change. If contact areas between belt and occupant changes, then paragraph 2.4.1.1. applies."

*       *       *

B.	JUSTIFICATION:

The effective anchorage may not be the same when the buckle is supported by a completely rigid system or a rigid system with one degree of freedom (rotation around one axis) and when the buckle is supported by a semi-rigid cable or by a pivoting rigid free-part.

_______________