| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/GRSP/2004/4 20 February 2004 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on General Safety Provisions (GRSP)

(Thirty-fifth session, 3-7 May 2004,

agenda item B.1.4.)

## PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 44

(Child restraints)

Transmitted by the secretariat

Note:  The text reproduced below was prepared by the secretariat, according to the request of GRSP expressed at its thirty-fourth session (TRANS/WP.29/GRSP/34, para. 27). It is based on  documents TRANS/WP.29/GRSP/2003/5 and TRANS/WP.29/GRSP/2003/14.

________________

Note: This document is distributed to the Experts on Passive Safety only.

Paragraphs 8.4. to 8.4.1.2., amend to read:

"8.4.	Registration of dynamic behaviour

8.4.1.	In order to determine the behaviour of the manikin and its displacements, all dynamic tests shall be registered according to the following conditions:

      - Filming and recording conditions:

-	the frequency shall be at least [500] frames per second;

-	the test shall be recorded on cine film, video or digital data carrier;

8.4.1.2.	Determination of uncertainty:

The determination of uncertainty of the displacement evaluation shall be  achieved by utilizing document EA-4/02 (December 1999) of the European Accreditation Organization.  The uncertainty shall be within [+/- 25] mm."

___________________