| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/GRSP/2004/15 17 September 2004 ENGLISH Original: ENGLISH ENGLISH AND FRENCH ONLY |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Passive Safety (GRSP)

(Thirty-sixth session, 7-10 December 2004,

agenda item A.4.)

HEAD RESTRAINTS

COMPARISON BETWEEN

NORTH AMERICAN REGULATIONS (FMVSS 202 and FMVSS 202, NPRM)

AND UNECE REGULATIONS Nos. 17 and 25

# Transmitted by the expert from the United States of America

Note: This document is distributed according to the request of GRSP (TRANS/WP.29/GRSP/35, para. 9).  It is based on the text of informal document No. GRSP-34-17.

___________________

_______________

Note: This document is distributed to the Experts on Passive Safety only.

HEAD RESTRAINTS

COMPARISON BETWEEN NORTH AMERICA REGULATIONS (FMVSS 202 and FMVSS 202, NPRM)

AND UNECE REGULATIONS Nos. 17 and 25

| Head Restraint Component |  | U.S. – FMVSS 202 |  | US FMVSS 202 NPRM | UNECE Regulations Nos. 17 and 25 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| # A. Application |  |  |  |  |  |  |
| # 1. Vehicles |  |  |  |  |  |  |
| Passenger cars, MPVs and trucks with a GVWR  4536 kg |  | Front outboard seating positions |  | All outboard seating positions | UNECE Regulation No. 17: Required in front outboard positions, optional in other positions, in vehicles with  9 seating positions (M1) | US required in all outboard vs UNECE that requires only front outboard, but if HR present in rear seat, UNECE regulates. |
| 2. Requirements |  |  |  |  |  |  |
| a. Height |  |  |  |  |  |  |
| 1. Front outboard |  |  |  |  |  |  |
| A. Fixed |  | At least 700 mm above H-point as measured parallel to the torso reference line |  | Increased to 800mm | Same as NPRM |  |
| B. Adjustable |  | Same as 202-fixed |  | Must achieve a height of 800 mm and cannot be adjusted below 750 mm. | Same as NPRM |  |
| 2. Rear outboard |  |  |  |  |  |  |
| A. Fixed |  | Not specified |  | Minimum height of 750 mm above H-point | If HR available, same as NPRM |  |
| B. Adjustable |  | Not specified |  | No adjustment below 750 mm | If HR available, same as NPRM |  |
| 3. Rear Centre |  |  |  |  |  |  |
|  |  | Not specified |  | Not specified | If HR available, minimum height of 700 mm |  |
| b. Backset |  |  |  |  |  |  |
| 1. All outboard positions | Not specified |  | Backset limited to a maximum 50 mm in any adjustment position as measured with HRMD. |  | Not specified. |  |
| c. Width |  |  |  |  |  |  |
| 1. Front outboard | Minimum of 171 mm on single seats and 254 mm on bench seats |  | Same as 202 |  | Minimum of 170 mm for all seat types | -171/170 difference due to rounding -US requires wider HRs on bench seats. |
| 2. Rear outboard | Not specified |  | Same as above |  | If HR available, minimum of 170 mm. | -171/170 difference due to rounding |
| d. Height of adjustable head restraint front surface |  |  |  |  |  |  |
|  | Not specified |  | Not specified |  | Minimum height of 100 mm |  |
| e. Gaps |  |  |  |  |  |  |
| 1. All outboard positions | Not specified |  | -In lowest position, gap between HR and seat back is  25mm, for a single backset adjustment. -In other positions, gap cannot exceed 60 mm -Both measured with a 165 mm diameter sphere placed in front of the HR. |  | -In lowest position, gap is  25, with no reference to backset adjustment. Measured along straight line between HR and seat back. -In other positions the gap  60 mm as measured with 165 mm dia. sphere, or must pass a load test. |  |

| Head Restraint Component | U.S. – FMVSS 202 | US FMVSS 202 NPRM | UNECE Regulations Nos. 17 and 25 | Comments |
| --- | --- | --- | --- | --- |
| f. HR Adjustment Retention Devices (locks) |  |  |  |  |
| 1. Height | Not specified | Must maintain height in several positions, including maximum and minimum, while a downward force is applied. | If adjustable, requires automatic locking system (UNECE Regulation No. 17, para. 5.1.1.). No downward test required. | UNECE has no downward testing requirement. |
| 2. Backset | Not specified | Under applied rearward moment, while adjusted to 800mm for front and 750mm for rear, HR must maintain any position of backset adjustment. | Not specified. |  |
| g. Removability |  |  |  |  |
| 1. Front | Not specified | Cannot be removed solely by hand (tools required) | Can be removed with deliberate action distinct from any act necessary for adjustment. | US proposed rule more specific (stringent). |
| 2. Rear | Not specified | Can be removed by hand. | Same as above |  |
| h. Clearance |  |  |  |  |
|  | Not specified | 25 mm clear space allowed where rear HRs, when seat is occupied, interfere with roofline or rear window. Minimum seat height could be less than 700 mm. | If HR available, 25 mm clear space allowed where interference with vehicle structure. Seat does not need to be occupied. Minimum height of 700mm must be maintained. | Requiring a minimum height would force some vehicle manuf. to alter roofline. Not an issue in UNECE. If no HR, minimum height does not need to be met. |

| Head Restraint Component | U.S. – FMVSS 202 | US FMVSS 202 NPRM | UNECE Regulations Nos. 17 and 25 | Comments |
| --- | --- | --- | --- | --- |
| i. Non-use positions |  |  |  |  |
| 1. Front | Not specified | Not allowed | Allowed, provided HR automatically returns to proper position when seat is occupied. |  |
| 2. Rear | Not specified | Allowed, provided HR automatically returns to proper position when seat is occupied or non-use position changes torso angle by 10 as measured with H-point device. | Allowed as long as non-use position is “clearly recognizable to the occupant”. | US proposed rule more stringent. Only allows HRs that can fold forward or are automatic. |
| j. Radius of curvature |  |  |  |  |
|  | Not specified | Any part of the HR outside of the impact zone for the energy absorption requirement must not have a radius smaller than 5mm. | Same as NPRM. Refers to the back of the head restraint. | 202 does not apply to the back of the head restraint. |
| k. Energy absorption |  |  |  |  |
|  | Not specified | Front of HR impacted with head form at v=24.1 km/h. 3ms deceleration of head form must not exceed 80gs . Impactor is free motion head form with mass of 6.8 kg. | Similar to NPRM: Uses pendulum impactor with same weight and velocity as linear impactor. | Results should be the same. |

| Head Restraint Component | U.S. – FMVSS 202 | US FMVSS 202 NPRM | UNECE Regulations Nos. 17 and 25 | Comments |
| --- | --- | --- | --- | --- |
| l. Displacement test procedures |  |  |  |  |
|  | Load is applied to back pan of seat, load is applied to head restraint after seat load is removed. 102 mm of displacement allowed with 373 Nm moment. Load is increased until 890N or seat back fails. Use spherical or cylindrical form to apply load. | Test procedure modified from 202. Seat back and HR loaded together. Moments and displacements same. Maximum load the same, seat back cannot fail. Use spherical form to apply load | Same as NPRM |  |
| m. Dynamic sled test |  |  |  |  |
|  | Seat accelerated so the pulse falls in a corridor defined by 2-½ sine waves with amplitudes of 78 m/s2 and 86 m/s2. Corridor cannot be met. 95th male dummy used, max rotation 45. | New corridor based on scaled version 208 sled test. Target pulse the same as 202. Tested with 50th and 95th male dummy. 50th max rotation 12 w/HR in min position. 95th max rotation 20 w/HR in min position. | Not specified |  |

__________________________