Minimum Standard for Type Approval Testing of Active Deployable Systems of the Bonnet / Windscreen Area

Note:

Part II, Chapter I, Paragraph 1.1.2 and 2.1.3 of the ANNEX of the Technical Prescription (2004/90/EC) dated on 04. February 2004 to which is referred in Article 3 of the EU Directive for Pedestrian Protection (2003/102/EC) dated on 06. December 2003.

Scope:

- Only type approval conditions considered according to the draft GTR or the EU Directive (assuming that EU Phase 2 is identical with the GTR), i.e.:

Passing the LEGFORM TEST

Passing the alternative UPPER LEGFORM TEST to bumper

Passing the HEADFORM TESTS

- Based on contact sensor techniques

- Marking up the vehicle in deployed or undeployed position may be made on the choice of the manufacturers.

Minimum Standard for Type Approval Testing of Active Deployable Systems of the Bonnet / Windscreen Area

Testing:

| 1 | Calculate the head impact time (HIT) for 6yo-child, 5%-female and 50%-male (50% is only needed when tests with the adult headform impactor are to be performed) in deployed position of the system, at a vehicle speed of 40km/h, at the centre of the vehicle, in walking posture, leg facing the vehicle is backwards, using an appropriate simulation tool. HIT = time from first contact on bumper to time of head to bonnet contact. The manufacturer and the Technical Services should agree the appropriate choice of the HIT (6yo or 5% or 50%) for each impact point. |
| --- | --- |
| 2 | The manufacturer provides the sensor time of the system In principle: total response time (TRT) = sensor time (ST) + deployment time (DT) |
| 3.1 | Testing of systems that remain in a permanent deployed position: Demonstrate: TRT < HIT (6yo or 5% or 50%) The system reaches and remains in the intended position before head impact. The system can be supported in a representative way (e.g. spring system) Perform LEGFORM TEST or UPPER LEGFORM TEST to bumper to measure TRT at the lifting device. Perform HEADFORM TESTS in deployed position of the bonnet. |
| 3.2 | Testing of systems that do not remain in a permanent deployed position: Perform “dynamic” HEADFORM TESTS ”dynamic” means: Triggered ignition time of the deployable system and the headform propulsion device is needed. This shall be agreed between the manufacturer and the Technical Service, based on the HIT & ST. The HIT may be calculated as in item 1, but for the undeployed condition. |
| 4 | System without contact sensors: For systems without contact sensors the TRT has to be considered separately. |