UN/ECE/WP29/GRSP Pedestrian Safety gtr Informal Group

2nd Interim Report

1. Introduction

- At the 33rd GRSP (2-6 June 2003) meeting, the informal group on pedestrian safety presented its first preliminary report as informal document 2. The document was adopted by GRSP and by WP29.

- The first preliminary report included amongst other:

  - Accident analysis looking at:

    - Overall pedestrian fatalities / injuries and their evolution over time

    - Distribution of injuries

    - Crash Speeds

  - The conclusions from this analysis were:

    - The majority of fatalities (numbers) and serious injuries occurred on:

- Child head vs. top surface of bonnet/ wing.

- Adult head vs. top surface of bonnet/wing + windscreen area and A-pillars,

- Adult leg vs. front bumper of vehicles,

    - A crash speed (between a car and a pedestrian) of 40 km/h can cover more than 75% of total injuries including fatalities. Injuries caused by higher speed crashes will also be influenced positively by a reduction in injury severity.

  - The group also asked advice on the approach it took for the scope and is still awaiting clear directions from WP29.

- Based on this first preliminary report and on request of GRSP, the informal group continued its work and started with the development of a gtr.

2. Draft gtr proposal: working paper.

The working paper for the draft gtr can be found attached. The working paper is a mix of decided items and items that are still under study. An action plan is in place (see item 3) to provide answers for all the questions that are left unanswered at this stage.

The preamble will be included in the first official draft gtr proposal to GRSP in December 2004. The preamble will include cost effectiveness/benefits conclusions, conflicts with other existing requirements, other measures that can be potentially reduce pedestrian injuries, etc.

- Legform test to bumper

Either the lower or the upper legform test are required to be performed:

      - Lower legform to bumper

        - Impact zone: On the bumper

        - Impact speed of the impactor: 40 km/h

        - Test tool:

          - The existing lower legform impactor used in the EU

Or

          - The FlexPLI legform developed in Japan.

The group is undertaking further studies to enable a decision on the selection of the impactor.

        - Criteria:

          - dynamic knee bending angle < [21]°, maximum dynamic knee shearing displacement < [6]mm; acceleration < [200]g for the EU impactor

Or

          - dynamic knee bending angle < [20]°, [maximum dynamic knee shearing displacement < [6]mm, acceleration < [200]g], tibia bending moment < [350] Nm for the FlexPLI impactor

      - [Upper legform to bumper (at the manufacturer request if the lower bumper height at the test position > 500 mm)

        - Impact zone: On the bumper

        - Impact speed of the impactor: 40 km/h

        - Test tool:

          - The existing lower legform impactor used in the EU

        - Criteria:

          - sum of impact forces < [7,5] kN; bending moment < [510] Nm]

- Child Head protection requirements

      - Impact zone

- 1000 < WAD (Wrap Around Distance) < 1700

- Includes A-pillar/windscreen area

      - Test tool

- 165 mm diameter

- 3,5 kg mass

### Impact speed of impactor: [32] km/h

### Impact angles

Depends on vehicle shape (bonnet angle (BA), bonnet leading edge height (BLEH)):

| Child | BLEH < 835 mm (Sedan) | BLEH > 835 mm (SUV) | BA > 30° (1 box) |
| --- | --- | --- | --- |
| Bonnet | 65° | 60° | 25° |
| Windscreen | 40° | 40° | 25° |

  - Criteria

  - HIC < [1000] for the child headform test area.

  - HIC < 1000 for the windscreen test area

  - The group agreed that the area required to meet HIC < 1000 may need revision and some consideration given to allow some exemption zones.

- Adult Head protection requirements

### Impact zone

- 1700 < WAD < 2100

- Includes A-pillar/windscreen area

### Test tool

- 165 mm diameter

- [4,5] kg mass

### Impact speed of impactor

[32] km/h

### Impact angles

Depends on vehicle shape (bonnet angle (BA), bonnet leading edge height (BLEH)):

| Adult | BLEH < 835 mm | BLEH > 835 mm | BA > 30° |
| --- | --- | --- | --- |
| Bonnet | 65° | 90° | 50° |
| Windscreen | 40° | 40° | 50° |

### Criteria

- HIC < [1000] for the child headform test area.

- HIC < 1000 for the windscreen test area

- The group agreed that the area required to meet HIC < 1000 may need revision and some consideration given to allow some exemption zones.

- Adult Upper Leg protection requirements:

The group agreed that inclusion of the upper leg test to the bonnet leading edge needed further review. In particular it must be verified whether this test is actually needed on the basis of accident data, technical feasibility, test procedure repeatability, reproducibility, etc.

3. Action plan

- General time schedule:

- draft gtr (based on updated PS/69) to May 04 GRSP. To serve as basis for the feasibility / effectiveness / repeatability / reproducibility assessment and resulting in

- first official draft gtr to Dec 04 GRSP. To serve as basis for comments from all CPs and industry and resulting in

- official draft gtr to May 05 GRSP for final adoption at AC3 in November 05.

- The action plan as agreed during the 6th informal group meeting (see also INF / GR / PS / 83)

For the head test area proposal including the windscreen and A-pillars: Need for more test data (all) Japan to provide J-NCAP data + proposal

Governments to make a recommendation on the selection of the impact points based on their own national / regional approach (recommended impact points versus free selection (worst case) by test house)

Head impact speed: EU and US study reservation: NHTSA to check if more information is available + EU to study results of PS/72.

EU study reservation for specifications of the 3,5 kg headform and for the weight of the adult headform.

Headforms: Accelerometer position accuracy under discussion in J and US. Any info from other source is welcome.

Head impactor skin ageing under study (J)

Head impactors: bring together certification data and check the response uniformity (J, IDIADA).

Include the certification test established by IHRA (PS/49 and PS/50) in the draft gtr.

OICA to give comments on a proposal for an active bonnet test method.

In order to decide which legform to be used we need by Sept 04: - IHRA to finalise its work asap (corridors, injury risk curves, …). (Summer 04) - info on repeatability and reproducibility of EU legform (EEVC) / FlexPLI (J) - biofidelity information of FlexPLI (J) / EU impactor (EEVC) - comparison result after test on same car of FlexPLI versus EU impactor (J to provide info on 2 cars) - information on general availability of FlexPLI (J) - information on sensitivity, durability, … - information on the certification procedures and specifications for each impactor (J, EEVC)

Results of ACEA study on feasibility (EU Phase 2) will be available next meeting. JAMA to check if J mfr can provide feasibility test data. EU feasibility study result will be available next meeting. Korea will check if KAMA has information to share.

4. Attachment

* Working paper for the draft gtr

* All documents used by the informal group can be found on the UN/ECE/WP29/GRSP website. For reference the complete list of working documents is attached:

| Number | Title |
| --- | --- |
| INF GR/PS/1* | Agenda 1st meeting |
| INF GR/PS/2 | Terms of reference |
| INF GR/PS/3 | IHRA accident study presentation |
| INF GR/PS/4* | JMLIT proposed legislation |
| INF GR/PS/5 | IHRA feasibility study |
| INF GR/PS/6 | J information on possible scope |
| INF GR/PS/7 | Attendance list 1st meeting |
| INF GR/PS/8* | Draft Meeting Minutes 1st meeting |
| INF GR/PS/9* | Report to GRSP 32 inf doc |
| INF GR/PS/10 | Draft action plan |
| INF GR/PS/11 | Agenda 2nd meeting |
| INF GR/PS/12 | GIDAS accident data |
| INF GR/PS/13 | GIDAS accident data graphs |
| INF GR/PS/14 | Italian accident data |
| INF GR/PS/15 | UN accident data |
| INF GR/PS/16 | Spanish accident data |
| INF GR/PS/17 | ACEA accident data |
| INF GR/PS/18 | Draft Meeting Minutes 2nd meeting |
| INF GR/PS/19 | Agenda 3rd meeting |
| INF GR/PS/20 | Canadian accident data |
| INF GR/PS/21 | Netherlands accident data |
| INF GR/PS/22 | Scope overview |
| INF GR/PS/23 | Draft content table preliminary report |
| INF GR/PS/24 | Attendance list 3rd meeting |
| INF GR/PS/25 | GIDAS presentation |
| INF GR/PS/26 | Leg injuries ITARDA |
| INF GR/PS/27* | Draft Meeting Minutes 3rd meeting |
| INF GR/PS/28 | Technical feasibility general |
| INF GR/PS/29 | Infrastructure effectiveness |
| INF GR/PS/30 | Pelvis / Femur fracture |
| INF GR/PS/31 | IHRA/PS-WG Pedestrian accident data |
| INF GR/PS/32 | ESV summary paper on IHRA/PS-WG report |
| INF GR/PS/33 | Introduction of the regulation od pedestrian head protection in Japan; Nishimoto, Toshiyuki |
| INF GR/PS/34 | Proposal for a directive of the European Parliament and the Council relating to the protection of pedestrians and other vulnerable road users in the event of a collision with a motor vehicle and amending Directive 70/156/EEC; Commission of the European Communities, Brussels, February 2003 |
| INF GR/PS/35 | List of conflicts with existing legislation / requirements |
| INF GR/PS/36 | Draft preliminary report |
| INF GR/PS/37 | Agenda 4th meeting |
| INF GR/PS/38 | Technical prescriptions concerning test provisions for pedestrian safety |
| INF GR/PS/39* | Vehicle safety standards report 1 |
| INF GR/PS/40 | US Cumulative 2002 Fleet GVMR |
| INF GR/PS/41 | Swedish accident data |
| INF GR/PS/42 | TRANS/WP.29/GRSG/2003/10 proposal for common definitions |
| INF GR/PS/43 | Category 1-1 GVM |
| INF GR/PS/44 | Light duty truck |
| INF GR/PS/45 | EURO-NCAP results and what they mean in relation to EU Phase 1 |
| INF GR/PS/46 | JAMA / JARI child and adult head impactors |
| INF GR/PS/47* | Preliminary report to GRSP 33 |
| INF GR/PS/48* | Draft meeting minures 4th meeting |
| INF GR/PS/49 | IHRA child head test method |
| INF GR/PS/50 | IHRA adult head test method |
| INF GR/PS/51 | Attendance list 4th meeting |
| INF GR/PS/52 | Provisional agenda for the 5th meeting |
| INF GR/PS/53 | Draft gtr format |
| INF GR/PS/54 | gtr proposal to WP29 |
| INF GR/PS/55 | Draft gtr |
| INF GR/PS/56* | Comparison table |
| INF GR/PS/57 | Proposed schedule of the group |
| INF GR/PS/58 | Presentation on veh shape, bound line, ... |
| INF GR/PS/59 | A-pillar IHRA OICA presentation |
| INF GR/PS/60 | ISO/TC22/SC10/WG2 N613 |
| INF GR/PS/61 | IHRA PS 237 |
| INF GR/PS/62 | Action plan from 5 meeting |
| INF GR/PS/63 | Attendance list 5th meeting |
| INF GR/PS/64* | Draft meeting minutes 5th meeting |
| INF GR/PS/65* | Provisional agenda for the 6th meeting |
| INF GR/PS/66 | AUS-NCAP pedestrian data |
| INF GR/PS/67 | Test-method - active hood / bonnet systems |
| INF GR/PS/68 | Target population head injuries - US |
| INF GR/PS/69 | Working paper draft gtr |
| INF GR/PS/70 | Korean information |
| INF GR/PS/71 | Head test area windscreen + A-pillar |
| INF GR/PS/72 | Head test data on windscreen |
| INF GR/PS/73 | Head impact angle / speed re-assessment based on vehicle geometry |
| INF GR/PS/74 | IHRA/PS/270 headform impactor specification |
| INF GR/PS/75 | Powerpoint explanation of PS/67 |
| INF GR/PS/76 | IHRA legform discussions |
| INF GR/PS/77 | Corridors proposed by UVA (lower legform) |
| INF GR/PS/78 | Bio rating method: Maltese |
| INF GR/PS/79 | IHRA antropometric proposal |
| INF GR/PS/80 | IHRA/PS/278 |
| INF GR/PS/81 | Schedule for legform impactor for gtr |
| INF GR/PS/82 | Injury threshold for ped legform test |
| INF GR/PS/83 | Decided items and action items of the 6th meeting |
| INF GR/PS/84 | Draft meeting minutes of the 6th meeting |
| INF GR/PS/85 | Attendance list of the 6th meeting |
| INF GR/PS/86 | Draft gtr EU working document |
| INF GR/PS/87 | IHRA PS 273 Development of FlexPLI2003 |
| INF GR/PS/88 | Second interim report to GRSP 35 |