# UN/SCEGHS/9/INF.7

COMMITTEE OF EXPERTS ON THE TRANSPORT OF

DANGEROUS GOODS AND ON THE GLOBALLY

HARMONIZED SYSTEM OF CLASSIFICATION

AND LABELLING OF CHEMICALS

##### Sub-Committee of Experts on the Globally

##### Harmonized System of Classification

##### and Labelling of Chemicals

Ninth session, 11-13 July 2005

Item 3 (c) of the provisional agenda

Hazard communication issues

Codification of Precautionary Statements (PS)

Transmitted by the expert from Australia

Introduction

- This INF paper describes work done to codify the GHS precautionary statements.  During the course of this work a number of precautionary statements were identified which may require editorial changes.

Background

- While the GHS provides an extensive classification framework for determining a collection of pictograms, signal words, hazard statements and precautionary statements that can be applied to a substance or mixture, less work has been done to identify how this potentially complex collection of hazard information should be best displayed on a product label.

- Some of the questions that have arisen in developing hazard-based labelling guidance for Australia are as follows:

- Should a label always include all of the pictograms, hazard statements and precautionary statements that are indicated as a result of the chemical’s GHS classification?  It is noted that the GHS provides some guidance on allocation of symbols and signal words at 1.4.10.5.3.1 and 1.4.10.5.3.2.

- Should the communication of some “more serious” hazard statements be given precedence over other “less serious” hazard statements when (a) there is insufficient label space to include all the information indicated by the full classification and/or (b) the inclusion of multiple hazard statements may divert attention from the critical and immediate hazard?

- Are there situations where a chemical that meets the criteria for multiple hazard classes may be assigned a series of precautionary statements that are potentially in conflict or overlapping and how should such situations be dealt with on a label?

A further question that has arisen in discussions with Australian regulators and industry groups that operate within regulatory frameworks that take a risk-based approach to labelling is:  To what extent should the authorised use of a chemical, its packaging or other factors that may moderate the intrinsic hazard, be taken into account when determining the hazard communication elements included on the label?

- If it is accepted that there may be situations where it is reasonable to use some discretion in managing the application of hazard classification information onto the label, then it is preferable that this process be done in a rigorous and consistent way.  The development of a hierarchy of GHS classification information could be used as a tool to manage the information conveyed on a label, whilst reflecting the principles and the intent of the GHS.

- Recent work in Australia has centred on the codification of the precautionary statements.  This is considered to be an important first step in assessing how the different statements may complement or conflict with each other, and whether any measure of priority can be assigned to them.

Discussion

- The list of codified precautionary statements is shown at Annex 1.  The basis for the codification is set out below.

- Each type of precautionary statement was allocated a two letter code (ie. PP for prevention statements, PR for response statements, PS for storage statements and PD for disposal statement).

- The statements within each type (PP, PR, PS or PD) were allocated to categories based on the similarity of the information conveyed in the statements.  The categorisation information is included in the attached spreadsheet and shows how the groupings were determined.  Within each grouping, statements were ordered, based on either seriousness or generality, with the more serious or broader statements listed first within each grouping, depending on the type of statement.

- Each statement in the ordered list was then assigned a two digit numeral which in combination with the two letter code forms a unique identifier (eg PR04, PP22 etc).  The numbering for each grouping was commenced at 1, 10, 20, 30… etc.  If related statements are added to the system in the future then they can be inserted into the list at the appropriate place using the unassigned numbers.  This is an important consideration as the precautionary statements may be refined as implementation experience is accumulated.

- Where two or more statements are only relevant to one classification category and are always used in combination, they were grouped under a single code number.  For example, the response statements for cryogenics were grouped together and assigned the code of PR50, as shown below:

PR50		Thaw frosted parts with lukewarm water.

Do not rub affected area.

Seek immediate medical attention/advice.

- The numbering system described above differs from the proposal presented in ST/SG/AC.10/C.4/2005/1 - Numbering of GHS Hazard Statements, where the numbering system is continuous.  The disadvantage of a continuous numbering system is that it is difficult to place new statements in the list in such a way that they are grouped with related statements.

- Problems could also arise where hazard statements are grouped (for example, Fatal if swallowed, fatal in contact with skin and fatal if inhaled) and given the same number.  It is suggested that these statements be separated and given unique numbers.

- During the codification of the precautionary statements the following points were noted where we believe some clarification or editorial revision may be required:

- The following two "response" statements are very close in meaning and differ by the variation of a single word.  It is recommended that these statements be replaced by a single statement.

PR37	If skin irritation occurs, get medical advice/attention (Skin corrosion/irritation chapter 3.2 hazard category 3)

PR38	If skin irritation occurs, seek medical advice/attention (Skin corrosion/irritation chapter 3.2 hazard category 2)

- The following two "response" statements are very close in meaning and differ only by the inclusion of a single word.  It is recommended that these statements be replaced by a single statement.

PR01	IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. (Acute toxicity- Inhalation chapter 3.1 hazard category-1 and 2)

PR02	IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. (Acute toxicity- Inhalation chapter 3.1 hazard category 3 and 4 and skin corrosion/irritation chapter 3.2 hazard category 1A to 1C)

- The response statement PR34 "Brush off loose particles from skin and immerse in cool water/wrap in wet bandages” (Pyrophoric solids - chapter 2.10 hazard category 1) should be written as “If on skin brush off loose particles from skin and immerse in cool water/wrap with wet bandages” to be consistent with other statements (eg PR33 – “If on skin immerse in cool water/wrap with wet bandages").

Conclusions

- Consideration should be given to editing the specific precautionary statements highlighted in this paper.

- Further guidance on the development of a hierarchy of GHS label elements is needed from SCEGHS.  If members are supportive of developing a hierarchy, Australia would consider putting a formal paper to a subsequent meeting. A number of other members have expressed interest in collaborating on this project.

Annex 1

Codified Precautionary Statements

(a) 	Prevention statements (Coded PP)

| Code | Statement (Prevention) |
| --- | --- |
| PP01 | Obtain special instructions before use. |
| PP02 | Do not handle until all safety precautions have been read and understood. |
| PP10 | Keep away from ignition sources such as heat/sparks/open flame. - No smoking. |
| PP11 | Keep away from flames and hot surfaces. |
| PP12 | Keep away from heat. |
| PP13 | Do not spray on an open flame or any white-hot material. |
| PP20 | Do not allow contact with air. |
| PP21 | Keep from any possible contact with water, because of violent reaction and possible flash fire. |
| PP22 | Handle under inert gas, protect from moisture. |
| PP23 | Keep away from ... incompatible materials specified by the manufacturer/supplier or the competent authority. |
| PP24 | Keep away from combustible materials. Keep reduction valves free from grease and oil. |
| PP25 | Keep away from clothing and other combustible materials. |
| PP26 | Take any precaution to avoid mixing with combustibles/… other incompatible materials specified by the manufacturer/supplier or the competent authority. |
| PP30 | Keep wetted with ... appropriate material specified by the manufacturer/supplier or the competent authority.- if drying out increases explosion hazard, except as needed for manufacturing or operating processes. Example: nitrocellulose |
| PP31 | Keep cool and protect from sunlight. |
| PP32 | Keep container tightly closed. |
| PP33 | Keep only in original container. |
| PP40 | Ground/bond container and receiving equipment - if the explosive is electrostatically sensitive. |
| PP41 | Ground/Bond container and receiving equipment. - if electrostatically sensitive material is for reloading. |
| PP42 | Ground/Bond container and receiving equipment - if electrostatically sensitive material is for reloading. .- if product is as volatile as to generate hazardous atmosphere. |
| PP43 | Take precautionary measures against static discharge. |
| PP44 | Use explosion-proof electrical/ventilating/ lighting/…/equipment … other specified by the manufacturer/supplier or the competent authority. |
| PP45 | Use explosion-proof electrical/ventilating/lighting/.../equipment. ... other specified by the manufacturer/supplier or the competent authority. - if dust clouds can occur. |
| PP46 | Use only non-sparking tools. |
| PP50 | Do not subject to rough handling such as grinding/shock/friction. |
| PP51 | Pressurized container: Do not pierce or burn, even after use. |
| PP60 | Do not breathe dust/fume/gas/ mist/vapours/spray. |
| PP61 | Do not breathe dust or mist - if inhalable particles may occur during use. |
| PP62 | Do not get in eyes, on skin, or on clothing. |
| PP63 | Avoid contact during pregnancy/while nursing. |
| PP64 | Avoid breathing dust/fume/gas/ mist/vapours/spray. |
| PP65 | Use only outdoors or in a well-ventilated area. |
| PP66 | Avoid release to the environment. - if this is not the intended use. |
| PP67 | Do not eat, drink or smoke when using this product. |
| PP68 | Contaminated work clothing should not be allowed out of the workplace. |
| PP69 | Wash thoroughly after handling. |
| PP70 | Wash hands thoroughly after handling. |
| PP80 | Wear respiratory protection as specified by the manufacturer/supplier or the competent authority. |
| PP81 | In case of inadequate ventilation wear respiratory protection as specified by the manufacturer or the competent authority. |
| PP82 | Wear protective gloves and eye/face protection as specified by the manufacturer /supplier or the competent authority. |
| PP83 | Wear eye/face protection as specified by the manufacturer/supplier or the competent authority. |
| PP84 | Wear cold insulating gloves/face shield/eye protection. |
| PP85 | Wear face protection as specified by the manufacturer /supplier or the competent authority. |
| PP86 | Wear protective gloves/clothing as specified by the manufacturer/supplier or the competent authority. |
| PP87 | Wear fire/flame resistant/retardant clothing. |
| PP88 | Wear protective gloves as specified by the manufacturer/supplier or the competent authority. |
| PP89 | Use personal protective equipment as required. |
| * Personal Protective Equipment |  |

(b)	Response statements (Coded PR)

| Code | Statement (Response) |
| --- | --- |
| PR01 | IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. Immediately call a POISON CENTRE or doctor/physician. Specific treatment is urgent (see ... on this label) ... reference to Supplemental First Aid Instruction - if immediate administration of antidote is required. |
| PR02 | IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTRE or doctor/physician. Specific treatment (see ... on this label) ... reference to Supplemental First Aid Instruction - if immediate specific measures are required. |
| PR03 | IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTRE or doctor/physician if you feel unwell. |
| PR04 | IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. |
| PR05 | IF INHALED: If breathing is difficult, remove to fresh air and keep at rest in a position comfortable for breathing. |
| PR06 | IF INHALED: Call a POISON CENTRE or doctor/physician if you feel unwell |
| PR10 | IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician. Rinse mouth. |
| PR11 | IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician. Do NOT induce vomiting. |
| PR12 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. |
| PR13 | IF SWALLOWED: Call a POISON CENTRE or doctor/physician if you feel unwell. Rinse mouth. |
| PR20 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| PR21 | If eye irritation persists, get medical advice/attention. |
| PR30 | IF ON SKIN (or hair): Remove/take off immediately all contaminated clothing. Rinse skin with water/shower. |
| PR31 | IF ON SKIN: Wash with plenty of soap and water. |
| PR32 | IF ON SKIN: Gently wash with plenty of soap and water. |
| PR33 | IF ON SKIN: Immerse in cool water/wrap with wet bandages. |
| PR34 | Brush off loose particles from skin and immerse in cool water/wrap in wet bandages. |
| PR35 | Wash hands after handling. |
| PR36 | If skin irritation or rash occurs, seek medical advice/attention. |
| PR37 | If skin irritation occurs, get medical advice/attention. |
| PR38 | If skin irritation occurs, seek medical advice/attention. |
| PR40 | IF ON CLOTHING: Rinse immediately contaminated clothing and skin with plenty of water before removing clothes. |
| PR41 | Remove/Take off immediately all contaminated clothing. |
| PR42 | Take off contaminated clothing and wash before re-use. |
| PR43 | Wash/Decontaminate removed clothing before reuse. |
| PR44 | Wash contaminated clothing before reuse. |
| PR50 | Thaw frosted parts with lukewarm water. Do not rub affected area. Seek immediate medical attention/advice. |
| PR60 | Immediately call a POISON CENTRE or doctor/physician. |
| PR61 | Call a POISON CENTRE or doctor/physician if exposed or if you feel unwell. |
| PR63 | If exposed or concerned: Get medical attention/advice. |
| PR62 | If exposed: Call a POISON CENTRE or doctor/physician. |
| PR64 | If experiencing respiratory symptoms call a POISON CENTRE or doctor/physician. |
| PR65 | Call a POISON CENTRE or doctor/physician if you feel unwell. |
| PR66 | Call a POISON CENTRE/doctor/physician if you feel unwell. |
| PR67 | Get medical attention/advice if you feel unwell. |
| PR70 | Specific treatment (see ... on this label) ... reference to Supplemental First Aid Instruction. - if immediate administration of antidote is required. |
| PR71 | Specific measures (see ... on this label) ... reference to Supplemental First Aid Instruction. - if immediate measures such as specific cleansing agent is advised. |
| PR72 | Specific treatment (see ... on this label) ... reference to Supplemental First Aid Instruction. may include a cleansing agent specified by the manufacturer/supplier or the competent authority. |
| PR73 | Specific treatment (see ... on this label) ... reference to Supplemental First Aid Instruction. - if immediate measures are required. |
| PR80 | DO NOT fight fire when fire reaches explosives. Explosion risk In case of fire. In case of fire, evacuate area. |
| PR81 | DO NOT fight fire when fire reaches explosives. In case of fire, evacuate area. Fight fire with normal precautions from a reasonable distance - if explosives are 1.4S AMMUNITION AND COMPONENTS THEREOF. |
| PR82 | In case of fire: evacuate area and fight fire remotely due to the risk of explosion. |
| PR83 | In case of major fire and large quantities: evacuate area and fight fire remotely due to the risk of explosion. |
| PR84 | In case of fire, use ... for extinction ... appropriate media specified by the manufacturer/supplier or the competent authority. - if water increases risk. |
| PR85 | Leaking gas fire: Do not extinguish, unless leak can be stopped safely. Eliminate all ignition sources if safe to do so. |
| PR86 | In case of fire, stop leak if safe to do so. |
| PR90 | Absorb spillage to prevent material damage. |
| PR91 | Collect spillage. |

(c)	Storage statements (Coded PS)

| Code | Statement (Storage) |
| --- | --- |
| PS01 | Store away from other materials. |
| PS02 | Store away from combustibles/... ... other incompatible materials specified by the manufacturer/supplier or the competent authority. |
| PS03 | Store in a dry place and/or in closed container. |
| PS04 | Store contents under ... ... appropriate liquid or inert gas specified by the manufacturer/supplier or the competent authority. |
| PS05 | Store in corrosive resistant/... container with a resistant inliner. ... other compatible materials specified by the manufacturer/supplier or the competent authority. |
| PS10 | Store in cool place at temperatures not exceeding …°C/…°F. ... specified by the manufacturer/supplier or the competent authority. |
| PS11 | Store at temperatures not exceeding …°C/…°F. ... specified by the manufacturer/supplier or the competent authority |
| PS12 | Store bulk masses greater than … kg/…lbs at temperatures not exceeding …°C/…°F. ... specified by the manufacturer/supplier or the competent authority. |
| PS13 | Protect from sunlight and do not expose to temperatures exceeding 50 ºC/122 ºF. |
| PS14 | Protect from sunlight. |
| PS15 | Protect from sunlight and store in well-ventilated place . |
| PS16 | Store in cool/well-ventilated place. |
| PS17 | Store container tightly closed in well-ventilated place. - if product is as volatile as to generate hazardous atmosphere. |
| PS18 | Store in well-ventilated place. |
| PS19 | Maintain air gap between stacks/pallets. |
| PS30 | Store … (in accordance with local/regional/national/ international regulation). |
| PS31 | Store locked up. |

(d)	Disposal statements (Coded PD)

| Code | Statement (Disposal) |
| --- | --- |
| PD01 | Dispose of contents/container to … (in accordance with local/regional/national/international regulation). |

_____________