# UN/SCEGHS/10/INF.7

COMMITTEE OF EXPERTS ON THE TRANSPORT OF

DANGEROUS GOODS AND ON THE GLOBALLY

HARMONIZED SYSTEM OF CLASSIFICATION

AND LABELLING OF CHEMICALS

Sub-Committee of Experts on the Globally Harmonized

System of Classification and Labelling of Chemicals

Transport of Dangerous Goods

Tenth session

Geneva, 7-9 December 2005

Item 3(a) of the provisional agenda

HAZARD COMMUNICATION ISSUES

Hierarchy of labelling elements

Codification of hazard and precautionary statements

Transmitted by the European Chemical Industry Council (CEFIC)

Introduction

CEFIC endorses and supports the principles underlying paper 2005/8 submitted by the Chairperson and the Expert from Australia, but is concerned at the codification system being proposed.  We consider that from a human cognitive perspective that a one alpha, three numeric system is much easier to assimilate, memorise and recognise than a two alpha, two numeric system and is more user friendly.

Discussion

CEFIC has reviewed paper 2005/8 as per the attached Annexes which are:

-	Annex I – Hazard Statement Codification

-	Annex II – Precautionary Statement Codification

Annex I - We have revised the hazard statement codification in paper 2005/8 by changing them to a one alpha, three numeric code.

Annex II – We have revised the precautionary statement codification in paper 2005/8 by changing them to a one alpha, three numeric code.

CEFIC appreciates and supports all the work that has been put into the proposal.  However in the EU the phrases have to be translated into about 20 different languages, thus language is an important consideration, for example the translation of “Health” into French is “Santé”, therefore in French the coding for Health Hazard Statements could change from “PHXX” to “PSXX”.  We have also examined the possibility of building on the work that has already been done by trying to make the precautionary statement phrases simpler/easier to translate, update and computerise, i.e. to see if the proposal can be made even more user friendly.

For example, we noted that phrases PP40, PP41 and PP42 have the same wording but their criteria for applicability differ.  We also noted that in phrases PR01 to PR13 there is some inbuilt duplication of some of the internal phraseology.  We have revised the content of 2005/8 to try to avoid this.  In doing so we have:

-	Used bold text for the wording of the phrases as they will appear on the label:

-	Used normal text to add "Modifiers" (modifying conditions) for the phrase wording as described in A3.5 of GHS Annex 3;

-	Used Italic text to add "Qualifiers" as described in A3.5 of GHS Annex 3;

We have broken down some of the longer phrases into their individual components, e.g. PR01 comprised of 4 individual phrases which are repeated in other phrases.  Where we have added new phrases to facilitate the changes we have made, we have used underline text and highlighted these in cyan.  The phrases that consequently become redundant are highlighted in yellow and are in strikethrough text, but we have included an indication of which new or existing phrases constitute the original ones (some of these are also referenced by text in square brackets and highlighted in grey).  In some cases where we ran out of numbers, we have put the "P" after the numbers, e.g. 234P for the sake of this paper, but once the phrases in yellow are deleted, the numbers that become free can be reallocated to revert to the normal numbering, i.e. P234 or whatever.  It may be that there are opportunities for further refinement, and if considered appropriate CEFIC is willing to work with the Chairperson and the Expert from Australia to develop the proposal further.

Annex 1

Codified hazard statements

Physical hazards (Coded H1)

| Code | Hazard statement |
| --- | --- |
| H101 | Unstable Explosive |
| H102 | Explosive; mass explosion hazard |
| H103 | Explosive; severe projection hazard |
| H104 | Explosive; fire, blast or projection hazard |
| H105 | Fire or projection hazard |
| H106 | May mass explode in fire |
| H107 | Heating may cause an explosion |
| H108 | Heating may cause a fire or explosion |
| H109 | May cause fire or explosion; strong oxidizer |
| H120 | Catches fire spontaneously if exposed to air |
| H121 | May cause or intensify fire; oxidizer |
| H122 | May intensify fire; oxidizer |
| H123 | In contact with water releases flammable gases which may ignite spontaneously |
| H124 | In contact with water releases flammable gas |
| H125 | Self-heating; may catch fire |
| H126 | Self-heating in large quantities; may catch fire |
| H127 | Heating may cause a fire |
| H140 | Extremely flammable gas |
| H141 | Flammable gas |
| H142 | Extremely flammable aerosol |
| H143 | Flammable aerosol |
| H144 | Extremely flammable liquid and vapour |
| H145 | Highly flammable liquid and vapour |
| H146 | Flammable liquid and vapour |
| H147 | Combustible liquid |
| H148 | Flammable solid |
| H160 | Contains gas under pressure; may explode if heated |
| H161 | Contains refrigerated gas; may cause cryogenic burns or injury |
| H162 | May be corrosive to metals |

Health hazards (Coded H2)

| Code | Hazard statement |
| --- | --- |
| H201 | Fatal if swallowed |
| H202 | Toxic if swallowed |
| H203 | Harmful if swallowed |
| H204 | May be harmful if swallowed |
| H210 | Fatal in contact with skin |
| H211 | Toxic in contact with skin |
| H212 | Harmful in contact with skin |
| H213 | May be harmful in contact with skin |
| H214 | Causes severe skin burns and eye damage |
| H215 | Causes skin irritation |
| H216 | Causes mild skin irritation |
| H217 | Causes serious eye damage |
| H218 | Causes serious eye irritation |
| H219 | Causes eye irritation |
| H220 | May cause an allergic skin reaction |
| H230 | Fatal if inhaled |
| H231 | Toxic if inhaled |
| H232 | Harmful if inhaled |
| H233 | May be harmful if inhaled |
| H234 | May be fatal if swallowed and enters airways |
| H235 | May be harmful if swallowed and enters airways |
| H236 | May cause respiratory irritation |
| H237 | May cause drowsiness or dizziness |
| H238 | May cause allergy or asthma symptoms or breathing difficulties if inhaled |
| H250 | May cause genetic defects <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H251 | Suspected of causing genetic defects <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H252 | May cause cancer <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H253 | Suspected of causing cancer <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H254 | May damage fertility or the unborn child <state specific effect if known> <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H255 | Suspected of damaging fertility or the unborn child <state specific effect if known> <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| Code | Hazard statement |
| H256 | May cause harm to breast-fed children |
| H260 | Causes damage to organs <or state all organs affected, if known> <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H261 | May cause damage to organs <or state all organs affected, if known> <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H262 | Causes damage to organs <or state all organs affected, if known> through prolonged or repeated exposure <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |
| H263 | May cause damage to organs <or state all organs affected, if known> through prolonged or repeated exposure <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard> |

Environmental hazards (Coded H3)

| Code | Hazard statement |
| --- | --- |
| H301 | Very toxic to aquatic life |
| H302 | Toxic to aquatic life |
| H303 | Harmful to aquatic life |
| H304 | Very toxic to aquatic life with long lasting effects |
| H305 | Toxic to aquatic life with long lasting effects |
| H306 | Harmful to aquatic life with long lasting effects |
| H307 | May cause long lasting harmful effects to aquatic life |

Annex 2

Codified precautionary statements

General statements (Coded P0) (see Annex III A3.4.4)

| Code | Statement (Disposal) |
| --- | --- |
| P001 | Keep out of reach of children. |
| P002 | Read label before use. |
| P003 | If medical advice is needed: Have product container or label at hand. |
| P010 | IF INHALED: |
| P011 | IF SWALLOWED: |
| P012 | IF ON SKIN: |
| P013 | IF exposed: |

Prevention statements (Coded P1)

| Code | Prevention |
| --- | --- |
| P101 | Obtain special instructions before use. |
| P102 | Do not handle until all safety precautions have been read and understood. |
| P110 | Keep away from heat/sparks/open flame/hot surfaces – No smoking. - Manufacturer/supplier or the competent authority to specify relevant ignition sources |
| P111 | Do not spray on an open flame or other ignition sources. |
| P120 | Do not allow contact with air. |
| P121 | Keep from any possible contact with water, because of violent reaction and possible flash fire. |
| P122 | Handle under inert gas, protect from moisture. |
| P123 | Keep away from ... - Manufacturer/supplier or the competent authority to specify incompatible materials |
| P124 | Keep away from combustible materials. Keep reduction valves free from grease and oil. |
| P125 | Keep away from clothing and other combustible materials. |
| P126 | Take any precaution to avoid mixing with combustibles/… - Manufacturer/supplier or the competent authority to specify other incompatible materials |
| P130 | Keep wetted with ... - Manufacturer/supplier or the competent authority to specify appropriate material - if drying out increases explosion hazard, except as needed for manufacturing or operating processes. Example: nitrocellulose |
| P131 | Keep cool and protect from sunlight. |
| P132 | Keep container tightly closed. |
| P133 | Keep only in original container. |
| P140 | Ground/bond container and receiving equipment. - if the explosive is electrostatically sensitive. - if electrostatically sensitive material is for reloading. - if product is as volatile as to generate hazardous atmosphere. |
| P141 | Ground/Bond container and receiving equipment. [see P140] if electrostatically sensitive material is for reloading. |
| P142 | Ground/Bond container and receiving equipment . [see P140] if electrostatically sensitive material is for reloading. .- if product is as volatile as to generate hazardous atmosphere. |
| P143 | Take precautionary measures against static discharge. |
| P144 | Use explosion-proof electrical/ventilating/lighting/… equipment - Manufacturer/supplier or the competent authority to specify other equipment. - general conditions - if dust clouds can occur. |
| P145 | Use explosion-proof electrical/ventilating/lighting/… equipment … [see P144] other specified by the manufacturer or the competent authority. if dust clouds can occur. |
| P146 | Use only non-sparking tools. |
| P150 | Do not subject to grinding/shock/friction. - Manufacturer/supplier or the competent authority to specify conditions - Such as rough handling |
| P151 | Pressurized container: Do not pierce or burn, even after use. |
| P160 | Do not breathe dust/fume/gas/mist/vapours/spray. - Manufacturer/supplier or the competent authority to specify conditions - general conditions - if inhalable particles of dusts or mists may occur during use. |
| P161 | Do not breathe dust or mist [see P160] if inhalable particles may occur during use. |
| P162 | Do not get in eyes, on skin, or on clothing. |
| P163 | Avoid contact during pregnancy/while nursing. |
| P164 | Avoid breathing dust/fume/gas/ mist/vapours/spray. [see P160] |
| P165 | Use only outdoors or in a well-ventilated area. |
| P166 | Avoid release to the environment. - if this is not the intended use. |
| P167 | Do not eat, drink or smoke when using this product. |
| P168 | Contaminated work clothing should not be allowed out of the workplace. |
| P169 | Wash thoroughly after handling. |
| P170 | Wash hands thoroughly after handling. |
| P180 | Wear respiratory protection - Manufacturer/supplier or the competent authority to specify relevant protection |
| P181 | In case of inadequate ventilation wear respiratory protection - Manufacturer/supplier or the competent authority to specify relevant protection |
| P182 | Wear protective clothing/gloves and eye/face protection - Manufacturer/supplier or the competent authority to specify relevant protection |
| P183 | Wear protective gloves and eye/face protection - Manufacturer/supplier or the competent authority to specify relevant protection |
| P184 | Wear eye/face protection - Manufacturer/supplier or the competent authority to specify relevant protection |
| P185 | Wear cold insulating gloves/face shield/eye protection. - Manufacturer/supplier or the competent authority to specify relevant protection |
| P186 | Wear face protection. [see P184] - Manufacturer/supplier or the competent authority to specify relevant protection |
| P187 | Wear protective gloves/clothing - Manufacturer/supplier or the competent authority to specify relevant protection |
| P188 | Wear fire/flame resistant/retardant clothing. |
| P189 | Wear protective gloves - Manufacturer/supplier or the competent authority to specify relevant protection |
| P190 | Use personal protective equipment as required. |

Response statements (Coded P2)

| Code | Response |
| --- | --- |
| P201 | IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. Immediately call a POISON CENTRE or doctor/physician. Specific treatment is urgent (see ... on this label) ... reference to Supplemental First Aid Instruction - if immediate administration of antidote is required. |
| P201 | P010 + P267 + P260 + P274 |
| P202 | IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTRE or doctor/physician. Specific treatment (see ... on this label) ... reference to Supplemental First Aid Instruction - if immediate specific measures are required |
| P202 | P010 + P267 + P269 + P274 |
| P203 | IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor/physician if you feel unwell. |
| P203 | P010 + P267 + P265 |
| P204 | IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing. |
| P204 | P010 + P267 |
| P205 | IF INHALED: If breathing is difficult, remove to fresh air and keep at rest in a position comfortable for breathing. |
| P205 | P010 + P268 |
| P206 | IF INHALED: Call a POISON CENTER or doctor/physician if you feel unwell |
| P206 | P010 + P265 |
| P210 | IF SWALLOWED: Immediately call a POISON CENTER or doctor/physician. Rinse mouth |
| P210 | P011 + P260 + P275 |
| P211 | IF SWALLOWED: Immediately call a POISON CENTER or doctor/physician. Do NOT induce vomiting. |
| P211 | P011 + P260 + P276 |
| P212 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. |
| P212 | P011 + P275 + P276 |
| P213 | IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell. Rinse mouth. |
| P213 | P011 + P265 + P275 |
| P220 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P221 | If eye irritation persists, get medical advice/attention. |
| P230 | IF ON SKIN (or hair): Remove/take off immediately all contaminated clothing. Rinse skin with water/shower. |
| P230 | P012 + P241 + P238 |
| P231 | IF ON SKIN: Wash with plenty of soap and water. |
| P231 | P012 + P239 |
| P232 | IF ON SKIN: Gently wash with plenty of soap and water. |
| P232 | P012 + 230P |
| P233 | IF ON SKIN: Immerse in cool water/wrap in wet bandages. |
| P233 | P012 + 231P |
| P234 | Brush off loose particles from skin and immerse in cool water/wrap in wet bandages. |
| P234 | 234P/231P |
| P235 | Wash hands after handling |
| P236 | If skin irritation or rash occurs, get medical advice/attention. |
| P237 | If skin irritation occurs, get medical advice/attention. |
| P238 | Rinse skin with water/shower. |
| P239 | Wash with plenty of soap and water. |
| 230P | Gently wash with plenty of soap and water. |
| 231P | Immerse in cool water/wrap in wet bandages. |
| 234P | Brush off loose particles from skin. |
| P240 | IF ON CLOTHING: Rinse immediately contaminated clothing and skin with plenty of water before removing clothes. |
| P241 | Remove/Take off immediately all contaminated clothing. |
| P242 | Take off contaminated clothing and wash before re-use. |
| P243 | Wash contaminated clothing before reuse. |
| P250 | Thaw frosted parts with lukewarm water. Do not rub affected area. Get immediate medical attention/advice. |
| P260 | Immediately call a POISON CENTRE or doctor/physician. |
| P261 | Call a POISON CENTRE or doctor/physician if exposed or if you feel unwell. |
| P261 | P013 + P265 |
| P262 | IF exposed: Call a POISON CENTRE or doctor/physician. |
| P262 | P013 + P269 |
| P263 | If exposed or concerned: Get medical attention/advice. |
| P264 | If experiencing respiratory symptoms call a POISON CENTRE or doctor/physician. |
| P265 | Call a POISON CENTRE or doctor/physician if you feel unwell. |
| P266 | Get medical attention/advice if you feel unwell. |
| P267 | Remove to fresh air and keep at rest in a position comfortable for breathing. |
| P268 | If breathing is difficult, remove to fresh air and keep at rest in a position comfortable for breathing. |
| P269 | Call a POISON CENTRE or doctor/physician. |
| P270 | Specific treatment (see ... on this label) - ... reference to Supplemental First Aid Instruction. - if immediate administration of antidote is required. - if immediate measures are required. - if immediate measures such as specific cleansing agent is advised. - manufacturer/supplier or the competent authority may specify inclusion of a cleansing agent. |
| P271 | Specific measures (see ... on this label) ... reference to Supplemental First Aid Instruction. - if immediate measures such as specific cleansing agent is advised. |
| P272 | Specific treatment (see ... on this label) ... reference to Supplemental First Aid Instruction. may include a cleansing agent specified by the manufacturer/supplier or the competent authority. |
| P273 | Specific treatment (see ... on this label) ... reference to Supplemental First Aid Instruction. - if immediate measures are required. |
| P274 | Specific treatment is urgent (see ... on this label) - ... reference to Supplemental First Aid Instruction - if immediate administration of antidote is required. |
| P275 | Rinse mouth |
| P276 | Do not induce vomiting. |
| P280 | DO NOT fight fire when fire reaches explosives. Explosion risk in case of fire. Evacuate area. |
| P281 | DO NOT fight fire when fire reaches explosives. Explosion risk in case of fire. In case of fire, evacuate area. |
| P282 | DO NOT fight fire when fire reaches explosives. In case of fire, evacuate area. Fight fire with normal precautions from a reasonable distance - if explosives are 1.4S AMMUNITION AND COMPONENTS THEREOF. |
| P283 | In case of fire: evacuate area and fight fire remotely due to the risk of explosion. |
| P284 | In case of major fire and large quantities: evacuate area and fight fire remotely due to the risk of explosion. |
| P285 | In case of fire, use ... for extinction - Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |
| P286 | Leaking gas fire: Do not extinguish, unless leak can be stopped safely. Eliminate all ignition sources if safe to do so. |
| P287 | In case of fire, stop leak if safe to do so. |
| P290 | Absorb spillage to prevent material damage. |
| P291 | Collect spillage. |

Storage statements (Coded P3)

| Code | Statement (Storage) |
| --- | --- |
| P301 | Store away from other materials. |
| P302 | Store away from combustibles/... - Manufacturer/supplier or the competent authority to specify other incompatible materials. |
| P303 | Store in a dry place and/or in closed container. |
| P304 | Store contents under ... - Manufacturer/supplier or the competent authority to specify appropriate liquid or inert gas. |
| P305 | Store in corrosive resistant/... container with a resistant inliner. - Manufacturer/supplier or the competent authority to specify other compatible materials. |
| P310 | Store in cool place at temperatures not exceeding …°C/…°F. - Manufacturer/supplier or the competent authority to specify temperature. |
| P311 | Store at temperatures not exceeding …°C/…°F. - Manufacturer/supplier or the competent authority to specify temperature. |
| P312 | Store bulk masses greater than … kg/…lbs at temperatures not exceeding …°C/…°F. - Manufacturer/supplier or the competent authority to specify temperature. |
| P313 | Protect from sunlight and do not expose to temperatures exceeding 50 ºC/122 ºF. |
| P314 | Protect from sunlight. |
| P315 | Protect from sunlight and store in well-ventilated place . |
| P316 | Store in cool/well-ventilated place. |
| P317 | Store container tightly closed in well-ventilated place. - if product is as volatile as to generate hazardous atmosphere. |
| P318 | Store in well-ventilated place. |
| P319 | Maintain air gap between stacks/pallets. |
| P330 | Store … - in accordance with local/regional/national/ international regulation. |
| P331 | Store locked up. |

Disposal statements (Coded P4)

| Code | Statement (Disposal) |
| --- | --- |
| P401 | Dispose of contents/container to - in accordance with local/regional/national/ international regulation. |

_______________